TriGem Micro-ATX Motherboard (COGNAC)

Hardware Document

July, 27 1999

Revision 1.0

Prepared by JS Kim

Approved by SE Hong

Document Revision History

| Released date | Revision | Description |
| --- | --- | --- |
| June, 10 1999 | Rev. 0.9 | First released version for this document. This document describes the major specification of the Cognac motherboard and the functional feature to be extended by the customer. The motherboard revision number is EVT3(PR1) |
| July, 27 1999 | Rev. 1.0 | 1. PCB Rev Up (AC) 2. It is modified in the connector option of M/B |

# Marketing Information

Address		45-2 Yoidodong, Youngdeungpoku, Seoul, Korea

Telephone		82-2-3774-4251 (International Business Team)

Fax			82-2-786-9478

Email			jhpark@trigem.co.kr (Jin Hyun Park)

##### Technical Support

Address		1055 Shingil-Dong, Ansan, Kyunggi-Do, Korea

Telephone		82-345-491-9528 (Ext. 608)

Fax			82-345-593-6721

Email			sehong@trigem.co.kr (Seung Eung Hong)

ycpark@trigem.co.kr (Young chul Park)

jskim2@trigem.co.kr (Jin Soo Kim)

# User’s Notice

No part of this product, including the schematics and BIOS may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language in any form by any means without the express 	written permission of TriGem Computer Inc. except the document kept by the purchaser for backup purposes.

© Copyright 1999 TriGem Computer Inc.		All rights reserved

Contents of Table

I.  Introduction

1. Generation Description	------------------------------------------------------------	4

2. Function Block Diagram	------------------------------------------------------------	6

II. System Overview

1. Major Units   -----------------------------------------------------------------------------	7

2. Upgrade ability 	-------------------------------------------------------------------------	8

2-1. Processor   --------------------------------------------------------------------------	8

2-2. Main chipset Configuration   ----------------------------------------------------	9

2-3. Memory   -----------------------------------------------------------------------------	8

2-4. BIOS   ---------------------------------------------------------------------------------	9

2-5. Expansion Slot   --------------------------------------------------------------------	9

2-6. Advanced Configuration and Power Interface (ACPI)   ------------------- 	10

2-7. Manufacturing Options   ---------------------------------------------------------- 	10

III. Jumper & Connector Description

1. Motherboard Jumper Setting   ------------------------------------------------------	11

1-1. Selection for Processor CPU Clock  ------------------------------------------ 	11

1-2. Other functionality   --------------------------------------------------------------- 	11

1-3. PCI graphics device function  ------------------------------------------------- 	12

1-4. OEM/ODM selector  -------------------------------------------------------------- 	12

2. Motherboard Connector Description   --------------------------------------------- 	14

2-1.Motherboard Internal Connector   --------------------------------------------- 	14

2-2.Motherboard External I/O Port   ------------------------------------------------ 	16

I.	Introduction

The COGNAC Micro ATX motherboard offers a time to market consumer and corporate desktop solution featuring the Intel Celeron PPGA processor with 66/100MHz front side bus and the Whitney chipsets in a Micro ATX low profile motherboards. In addition, the integrated onchip graphics controller supports 4MB of SDRAM purpose of Display cache memory.

The Cognac motherboard was designed to be highly minimized system cost. In this effort a smaller form factor, Micro-ATX, gives the greater space economy and more affordable systems. Integrating onchip graphics controller and SDRAM, as well as onchip AC’97 audio solution with AC97 Codec onto the motherboard eliminates the need for more expensive graphic and audio add-in cards. The end result is a system platform with a primary component level of integration with translates into affordable solution for entry level users.

## 1. General description

	Motherboard

  Small PCB size in the Micro ATX form factor (ATX V1.2 form factor)

  238mm * 238mm * 1.6t (4 Layers)

	Processor

  Intel Socket-370 (370pin PPGA Socket)

-  Intel Celeron 400/433/466/500MHz processor @66MHZ Host clock

-  Intel TBD                                            @100MHZ Host clock

  Main Chipset

  Graphics and Memory Controller Hub (GMCH)	: Intel FW82810-DC100 / FW82810

  I/O Controller Hub (ICH)     		: Intel FW82801AA / FW82801AB

  AC 97’ Controller 	 			: Intel FW82801AA/AB included

  Audio Codec	 			: Cirrus CS4299-JQ

  Super I/O				: ITE IT8702F-A

  DC-DC Converter 			: HARRIS HIP6021CB

  Clock			 		: ICS 9250BF-10 (66/100MHz host clock,

100Mhz SDRAM Clock support)

  Memory Configuration

  System Memory

- Two banks of 3.3V SDRAM (168pin unbuffered DIMM) with max 256MB

- SDRAM operation in 100MHz front side bus

  Display Cache Memory

- Two 1M*16 3.3V SDRAM (4MB)

- SDRAM operation in 100MHz

  Flash Memory

- Intel N82802AB, 4Mb PLCC Type

- FirmWare Hub(FWH) Interface for platform operation

- Address/Address Multiplexed Interface for programming during manufacturing

	I/O Feature

  Integrated standard I/O functions in the rear side

- One multi-mode parallel port

- One FIFO serial ports and optional port by header type

- PS/2 styles keyboard and mouse port

- Two USB port

- Three audio jack for Speaker output ,Line input and MIC input

- One Joystick port

  I/O Controller (IT8702F-A)

- Low Pin Count (LPC) interface with Host controller(ICH)

- FDC, Parallel, Serial, keyboard/Mouse supported

- Joystick port and MPU-401 compatible MIDI interface

	Audio Subsystem

  Intel 82801AA/AB AC’97 Audio/Modem controller

  Independent channels for PCM in and PCM out, Microphone in

  Left and Right audio channels

  16 bit sample resolution

  multiple sample rates

  AC’97 2.1 compliant AC-link interface

- GMCH Subsystem

  Host/Memory(DRAM) Controller features

- 66/100Mhz System Bus Frequency

- 100Mhz system memory bus frequency

- Refresh mechanism: CBR only supported

- Support for Asymmetrical DRAM addressing only

- Support for Asymmetrical DRAM addressing only

- Support for 64-bit data interface

- Suspend to RAM support

  Graphics Controller features

- 3D Hyper Pipelined Architecture ( PDP, PPI)

- full 2D H/W Acceleration

- Motion Video Acceleration

- H/W motion Compensation Assistance for S/W MPEG2 Decode

- Software DVD at 30fps

  3D Graphics features

- Flat & Gouraud Shading

- Mip Maps with Bilinear and Anisotropic Filtering

- Fogging Atmospheric Effects

- Z Buffering

- 3D Pipe 2D Clipping

- Backface Culling

- Per Pixel Perspective Correction Texture Mapping

- Texture Compositing

- Texture Color Keying/Croma Keying

  Display features

- Integrated 24-bit 230Mhz RAMDAC

- Gamma Corrected Video

- DDC2B Compliant

  Graphics memory controller features

- Intel D.V.M Technology (Dynamic Video Memory)

  Display Cache Interface (FW82810-DC100 only)

- ICH Subsystem

  PCI Bus Interface

  Integrated IDE controller

  USB

  AC’97 Link for Audio and Modem

  Interrupt Controller

  Low Pin Count Interface

  SM Bus

  Firmware Hub Interface

- GMCH,ICH vs GMCH0,ICH0 Configuration Difference

| GMCH, ICH | GMCH0, ICH0 |
| --- | --- |
| Display Cache (4MB) Support | No Display Cache (4MB) Support |
| Up to Ultra 66 ATA Spec. Support | Up to Ultra 33 ATA Spec. Support |
| 6 PCI Slots Support | 4 PCI Slots Support |
| Alert on LAN Support | N/A |

Note : Pavilion 6530/6535 uses the GMCH0, ICH0

2. Function Block Diagram

Intel Celeron

PGA 370

DIMM Modules

MD

MA

HA

HD

Speaker

MIC

Line in

2M

2M

SDRAM

DCMD

DCMA

RGB

Video

DCMD

DCMA

## GMCH / GMCH0

Hub

HDD

PCI Slot

HD Data

HA Address

Keyboard

Mouse

Serial

Parallel

USB

CDROM

## ICH / ICH0

Code

c

CS4299

FWH

I/O

IT8702F

Joystick

Midi

II.	System Overview

1.	Major Units

FW82810

Display cache

(Not used)

AC97 codec

Audio Amp

PCI Slots

AMR Slots

FW82801AB

82802AB(ROM)

CDR CONNECTOR

FDD CONNECTOR

IT8702

POWER CONN

RTC Battery

DIMM

HIP6021CB

9250BF-10

CPU

Serial driver 2

Serial driver 1

HDD CONNECTOR

2.	Upgradeability

2-1.	Processor

COGNAC motherboard provides the 370pin PGA370 socket that is not backward compatible with ZIF socket-7 processors The Processor’s VID pins automatically program the voltage regulator on the motherboard to the required processor voltage. The motherboard supports processors that run internally at 300/333/366/400 /433/466/500MHz.

	Supported Intel Celeron Processors (PPGA Socket Type)

	Host Clock 66MHz		: Celeron 400MHz

: Celeron 433MHz

: Celeron 466MHz

: Celeron 500MHz

: tbd

	Host Clock 100MHz		: tbd

	Supported Cyrix Goby Processors (PPGA Socket Type)

	Host Clock 66MHz		: tbd

2-2.	Main Chipset Config

Cognac Motherboard has two main chipset configurations.

	810DC100 Configuration

 GMCH + ICH 	: FW82810DC100 + FW82801AA

	810L Configuration

 GMCH0 + ICH0	: FW82810 + FW82801AB

 It’s applied to Cognac, Cognac+

2-3.	Memory

The motherboard has two, dual inline memory module (DIMM), minimum 16MB to maximum 256MB memory size. The BIOS can automatically detect the memory type, size, and speed through SMBUS interface between the core chipset and DIMM module.

The motherboard supports the following memory features

	3.3V and unbuffered168-pin DIMM

Voltage detection

3.3V Version		 5V Version

Unbuffered detection

Unbuffered	          Buffered

	100MHz unbuffered SDRAM (PC100)

	Non-ECC memory only

	Single or double-sided DIMM with the following types (per each side of each DIMM)

| DIMM size | Non-ECC memory | DIMM size | Non-ECC memory |
| --- | --- | --- | --- |
| 8MB | 4*(1M * 16 bit) | 64MB | 8*(8M * 8 bit) |
| 16MB | 8*(2M * 8 bit) | 64MB | 4*(8M * 16 bit) |
| 32MB | 4*(4M * 16 bit) | 128MB | 8*(16M * 8 bit) |
| 32MB | 2*(4M * 32 bit) |  |  |

2-4 BIOS

The motherboard uses a TriGem-Phoenix BIOS, which is stored in flash memory and can be upgraded using a disk-based program. A new version of the BIOS can be upgraded from a diskette using the Flash Memory Update utility.

 	Flash memory organization

| Address (Hex) | Size | Functional description |
| --- | --- | --- |
| FFFF0000 - FFFFFFFF | 64KB | Boot block |
| FFF82000 - FFFEFFFF | 440KB | Main BIOS block |
| FFF80000 - FFF81FFF | 8KB | ESCD block |

	On-board device management

The BIOS can manage the devices on the motherboard over the CMOS setup menu.

| Device | Description | CMOS setup menu | Default value |
| --- | --- | --- | --- |
| PS/2 Mouse | Intel 82801AB (ICH) | Enabled / Disabled / Auto Detect | Auto Detect |
| Legacy USB | Intel 82801AB (ICH) | Enabled/ Disabled | Disabled |
| AC’97 Audio | ICH with CS4299 Codec | Enabled / Disabled | Enabledd |
| On board FDC | Super I/O (ITE8702) | Enabled / Disabled | Enabled |
| On board serial A | Super I/O (ITE8702) | Enabled / Disabled | Enabled |
| On board serial B | Super I/O (ITE8702) | Enabled / Disabled | Disabled |
| On board parallel | Super I/O (ITE8702) | Enabled / Disabled | Enabled |
| Midi port | Super I/O (ITE8702) | Enabled / Disabled / Auto | Auto |
| Game port | Super I/O (ITE8702) | Enabled / Disabled / Auto | Auto |

2-5.	Expansion Slot

The motherboard support PCI and GMCH function. PCI functions are extended to the additional slot with 3 PCI, and GMCH function is designed in the motherboard with Integrated System/Graphics controller.

	PCI configuration space map

| Bus number | Device number | Function number | Device |
| --- | --- | --- | --- |
| 00 | 30 | 00 | Intel 82801AB(ICH0) PCI Bridge |
| 00 | 31 | 00 | Intel 82801AB(ICH0) LPC Bridge |
| 00 | 31 | 01 | Intel 82801AB(ICH0) Bus master IDE |
| 00 | 31 | 02 | Intel 82801AB(ICH0) USB Host Controller |
| 00 | 31 | 03 | Intel 82801AB(ICH0) SM Bus Controller |
| 00 | 31 | 04 | Reserved |
| 00 | 31 | 05 | Intel 82801AB(ICH0) AC’97 Audio Controller |
| 00 | 31 | 06 | Intel 82801AB(ICH0) AC’97 Modem Controller |
| 00 | 31 | 07 | Reserved |
| 00 | 00 | 00 | Intel 82810(GMCH0) System/Graphics Controller |
| 00 | 01 | 00 | Intel 82810(GMCH0) Internal Graphics Device |
| 01 | 0E | 00 | PCI slot1 |
| 01 | 0D | 00 | PCI slot2 |
| 01 | 0C | 00 | PCI slot3 |

	PCI interrupts & Master Number Routing Maps

Intel 82801AB (ICH0) PCI bridge has four programmable interrupt request input signals. Any PCI interrupt source connects to one of these interrupt signals and It’s assigned to the free proper interrupt number by PnP BIOS.

| ICH INT signals | First PCI slot | Second PCI slot | Third PCI slot | ICH0 USB device | ICH0 SM Bus |
| --- | --- | --- | --- | --- | --- |
| PIRQA | INTA | INTB | INTC |  |  |
| PIRQB | INTB | INTC | INTD |  |  |
| PIRQC | INTC | INTD | INTA |  |  |
| PIRQD | INTD | INTA | INTB | INTC | INTB |
| Master | REQ0 | REQ1 | REQ2 |  |  |
| IDSEL | AD30 | AD29 | AD27 |  |  |

| ICH INT signals | ICH0 AC’97 Audio | ICH0 AC’97 Modem | GMCH Internal Graphics Device |
| --- | --- | --- | --- |
| PIRQA |  |  |  |
| PIRQB |  |  |  |
| PIRQC |  |  |  |
| PIRQD | INTB | INTB | INTA |
| Master |  |  |  |
| IDSEL |  |  |  |

2-6. Advanced Configuration and Power Interface (ACPI)

The motherboard and system BIOS supports the ACPI that requires an ACPI-aware operating system such as Windows-NT 5.0 or Windows 98 SE. ACPI feature include

	Plug and play and APM functionality normally contained in the BIOS

	Power management control of individual devices : add-in cards, hard disk drives, USB devices, and                Video

	A soft-off feature that enables operating system to power off the computer

	Support for multiple wakeup events

	Indication LED for normal mode (Green), standby mode (Blinking Green) function is dependent on the

LED logic.

	Wakeup devices and events

| Wakeup device | Wakeup events and functionality |
| --- | --- |
| Power switch | Wakeup from Power-off status and S1Status |
| LAN | Wakeup from S1 status |
| Modem | Wakeup from S1 status |

2-7.  Manufacturing Options

The motherboard has several manufacturing options according to OEM/ODM requirement. Make sure that these options can be applied in the assembly stage, and it’s impossible to upgrade or change in the customer field.

| Option items | Selectable functionality | Feature changes |
| --- | --- | --- |
| Joystick port | Front side / Rear side | Use additional board or not |
| USB port | Front side / Rear side | Use additional board or not |
| Super I/O | IT8712 / IT8702 | Include H/W monitoring or not |
| Graphics controller | FW82810 / FW82810-DC100 | Display Cache or not |
| Display Cache memory | None / 4MB | Display Cache or not |

III.	Jumper & Connector Description

1.	Motherboard Jumper Setting

J1 – J8

1-1.  Selection for Processor CPU Clock

  Intel Celeron PPGA Processor is auto set the core to bus frequency ratio.

1-2.  System functionality (J1,2,3,4,5,8 is TG Option)

| DIP Switch | Function | 1-2(Default) | 2-3 |
| --- | --- | --- | --- |
| J6 | Clear CMOS | Disable | Enable |
| J7 | Audio Function(AMR) Primary Down | Enable | Disable |

* Note : Pavilion 6530 uses J6,7

Pavilion 6535 uses J6

1-3.  PCI graphics device function

No jumper does set the functionality of the GMCH graphics controller

PCI Graphics device operates automatically if PCI Graphics card is inserted .

1-4.  OEM/ODM selector (TG Option)

These jumpers (J1, J2 & J3) will be optional parts for the OEM/ODM logo message selector of Trigem.

| BIOS Factory Setting (J1,2,3 | J2(2-3), J3(2-3),J4(2-3) | Reserved | J2(2-3), J3(2-3),J4(1-2) | Reserved |
| --- | --- | --- | --- | --- |
|  | J2(2-3), J3(1-2), J4(2-3) | Reserved | J2(2-3), J3(1-2), J4(1-2) | Reserved |
|  | J2(1-2), J3(2-3), J4(2-3) | Reserved | J2(1-2), J3(2-3), J4(1-2) | Reserved |
|  | J2(1-2), J3(1-2), J4(2-3) | Reserved | J2(1-2), J3(1-2), J4(1-2) | Reserved |

* Factory Default Setting  - bold type text

2. I/O Header Connector Description

2-1. Motherboard Internal Connector

MODEM SOUND(CN46)

MODEM SOUND(CN6)

VIDEO SOUND(CN45)

CD SOUND(CN43)

CD SOUND(CN44)

JOY_USB (CN72)

HPD INDICATOR(CN73)

TG INDICATOR(CN48)

FAN CONNECTOR

(CN53)

FAN CONNECTOR

(CN54)

LAN CONNECTOR

(CN50)

RING CONNECTOR(CN51)

	CPU FAN connector (CN53)

| 1 2 3 CN53 Pin number | Signal description |
| --- | --- |
| 1 | GND |
| 2 | FAN power |
| 3 | Tachometer (speed) |

	System Chassis FAN connector (CN54)

| 1 2 3 Pin number | Signal description |
| --- | --- |
| 1 | GND |
| 2 | FAN control |
| 3 | Tachometer (Speed)(GND) |

 	PC/PCI connector (CN23) (TG Option)

| 2 4 6 1 3 5 Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | /PCGNTA | 4 | /PCREQA |
| 2 | GND | 5 | N.C |
| 3 | Key | 6 | SER_IRQ |

  Joystick & USB connector (CN72) (TG Option)

| Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | VCC | 9 | VCC |
| 3 1 8 7 6 5 4 3 2 1 4 2 16 15 14 13 12 11 10 9 2 | GD(4) | 10 | GD(6) |
| 3 | GD(0) | 11 | GD(2) |
| 4 | GND | 12 | MIDI OUT |
| 5 | GND | 13 | GD(3) |
| 6 | GD(1) | 14 | GD(7) |
| 7 | GD(5) | 15 | MIDI IN |
| 8 | VCC | 16 | Key |
| 1 | GND | 3 | Positive DATA |
| 2 | Negative DATA | 4 | VCC |

	Video Sound (CN45) (TG Option)

| 1 2 3 4 Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | Left Sound | 3 | GND |
| 2 | GND | 4 | Right Sound |

	Modem Sound (CN46, CN6) (TG Option)

| 1 2 3 4 5 CN46 Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | MIC | 4 | GND |
| 2 | GND | 5 | MONO IN |
| 3 | MONO OUT |  |  |

| 1 2 3 4 CN6 Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | MONO IN | 3 | GND |
| 2 | GND | 4 | MIC |

	CD Sound (CN43(TG Option), CN44)

| 1 2 3 4 ATAPI CD (CN43) Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | Left Sound | 3 | GND |
| 2 | GND | 4 | Right Sound |

| 1 2 3 4 Mitsumi CD (CN44) Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | GND | 3 | GND |
| 2 | Left Sound | 4 | Right Sound |

	LAN Wakeup (CN50) (TG Option)

| 1 2 3 4 Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | +5VSB | 3 | LANWK |
| 2 | GND |  |  |

	Modem Wakeup (CN51) (TG Option)

| 1 2 3 4 Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | Modem Ring | 3 | +5VSB |
| 2 | GND |  |  |

	HPD Indicator Header (CN73)

PWR  HDD  DC/SW

8

9

| Pin | Signal description |
| --- | --- |
| 1 | NC |
| 2 | GND |
| 3 | LED POWER |
| 4 | NC |
| 5 | LED POWER |
| 6 | HDD access signal |
| 7 | GND |
| 8 | Power-ON switch signal |

	Aux. Indicator Header (CN49) (TG Option)

| Pin | Signal description | ### Pin | Signal description |
| --- | --- | --- | --- |
| 1 | Key lock Signal to Super I/O | 2 | GND |
| 3 | Key | 4 | Reset signal |
| 5 | GND | 6 | Key |
| 7 | Sleep Function signal | 8 | GND |

2-2. Motherboard External I/O Port

Parallel port  Serial port

Monitor                USB  Keyboard/Mouse

Joystick port

Line-in   Speaker

Mic

Mic in

Trademarks

Microsoft, Windows and Windows NT are registered trademarks of Microsoft Corporation.