# VS440FX Desktop

Pentium® Pro Processor, 82440FX

PCIset-based ATX Platform

Technology of the Future for

Professional Desktops

| The VS440FX delivers the excellent performance of the PentiumÒ Pro processor at an exceptional price for professional desktops. Features | Benefits |
| --- | --- |
| · Pentium Pro processor | · Optimized performance for 32-bit applications |
| · New ATX form factor chassis | · Maximum expandability, ease of integration and upgradeability |
| · Six peripheral bays | · Side panel allows easy access to all system features |
| · Full length slots | · Maximum flexibility for a variety of system configurations |
| · EDO and Fast Page memory support | · Performance choice of EDO; reliability of parity and ECC |
| · Integrated audio on platforms | · Ensured platform compatibility and lower total system cost |

Featuring the 82440FX PCIset, the VS440FX optimizes performance and reduces cost with the new integrated chip set. Based on the Pentium Pro processor, VS440FX is an ideal solution for 32-bit applications and operating systems, where the ultimate in processing speed is critical.

The system brings to market a complete, integrated, expandable ATX solution for both the professional desktop an workstation market segments. The platform delivers the robust feature set that is demanded by today’s corporate computing environment. It includes integrated Crystal* audio for quality sound, National’s I/O controller for Plug and Play compliance, system memory support up to 256 MB for expandability, and PCI 2.1 compliant performance.

World Class Performance -

at an Affordable Cost

VS440FX is designed to combine the power of the Pentium Pro processor with the integration of the 82440FX PCIset. The result is optimized performance and exceptional value with 32-bit applications and operating systems - such as Windows* NT, OS/2* and UNIX* - which are now commonplace on professional desktops and power workstations.

VS440FX supplies enough processing power for the most demanding engineering and software development programs. Pentium Pro processor support begins at clock speeds of 150 MHz and scales up to an uncompromised 200 MHz. VS440FX also supports future Pentium Pro OverDriveÒ processors.

Intel’s 82440FX PCIset channels the processor’s raw power into superior total system throughput. The 82440FX chip set supports either Fast Page or EDO system memory providing the latest in memory technology and performance. Users can upgrade to Error Checking and Correcting (ECC) functionality simply by installing parity SIMMs.

The 82440FX chip set with PIIX3 supports mixed PIO Mode drives ensuring the maximum performance possible from PIO Mode 4 devices. The chip set offers PIO Mode 4 transfer rates up to 16 MB per second on the two IDE interfaces, which support a total of four devices. The PIIX3 chip also maintains backward compatibility with current Pentium processor-based platforms.

Next Generation ATX System -

Continuing Industry Leadership

The VS440FX system solution uses the new Desktop ATX MiniTower chassis. The chassis is designed to be ATX 1.1 compliant and very user friendly. Chassis accessibility can be accommodated without the use of tools for easy end-user upgrades and serviceability. System memory and processors are readily accessible to the user as well as the full length add-in card slots. Reduced cabling enabled by the ATX form factor improves the emissions profile as well as simplifying integration.

The chassis has a security lock location and plastic panels that have a softer look and feel. The integrated back panel include two serial ports, one parallel port, audio jacks, PS/2*-style keyboard and mouse connectors.

The VS440FX MiniTower chassis offers four external and two internal peripheral bays. A 200 watt (peak)  power supply supports all of the voltage needs of a fully-loaded system.

PROCESSOR/CACHE

Processors Supported	Pentium Pro processors with

256 KB integrated L2 cache

150 MHz, 180 MHz and 200 MHz

Performance Upgrade	Type 8 socket supports a future OverDriveÒ 			processor upgrade

SYSTEM MEMORY

Memory Capacity	472-pin sockets for tin-lead DRAM SIMMs

2 banks (SIMMs filled in pairs for 64 bit bus)

Memory Size	8 MB minimum (two 4 MB SIMMs)

16 MB or 32 MB standard

256 MB* maximum (four 64 MB* SIMMs)

SIMM Sizes	4 MB, 8 MB, 16 MB, 32 MB, 64 MB*

Access Speed	60ns Fast Page Mode (FPM) and

Extended Data Out (EDO) memory

ECC and Parity memory support

Memory Voltage	5 volts

*64 MB support using 64 Mbit technology SIMMs

INTEGRATED AUDIO	Crystal Plug and Play audio

Yamaha OPL3 FM synthesis

INTEGRATED PCI IDE

Number of Devices	2 independent channels, 4 IDE devices

Device Types	Mode 0, PIO Mode 3, Mode 4 IDE

Hard Drives and CD-ROM supported

INTEGRATED SUPER I/O

RTC, Keyboard, I/O Cont.	National Plug and Play

Two Serial Ports	Async, RS-232C, PC16C550 compatible

Parallel Port	IEEE 1284 compatible, ECP, EPP

Floppy Controller	1.44 MB, 2.88 MB, 3-Mode support

Keyboard/Mouse Controller	8042A compatible

Real Time Clock	Century Reg. ± 13 minutes/year accuracy

APM/APC	Power On, Off Sleep, Resume features

Infrared Interface	IrDA and Consumer IR compatible

SYSTEM BIOS

BIOS Type	2 Mb Flash EEPROM with Intel BIOS

Special Features	Plug and Play; IDE drive autoconfigure;

APM, DMI, ECC/Parity support;

Multilingual support

PCI CHIP SET 	Intel 82440FX PCIset

Intel 82441FX PMC

Intel 82442FX DBX

Intel 82371SB PIIX3

JUMPERS AND FRONT PANEL CONNECTORS

Connectors	Speaker, Reset, LEDs (power, HDD),

infrared, power on/off, sleep/resume

Jumpers	Password clear, CMOS clear, setup disable,

BIOS recovery, CPU/Bus clock speed

EXPANSION SLOTS (all full length)

Description	Three dedicated PCI slots

Three dedicated ISA slots

One shared slot for either ISA or PCI

SYSTEM	ATX MiniTower chassis

Height	14.76" (37.5 cm)

Width	8.94" (22.7 cm)

Depth	17" (43.2 cm)

Weight	17.7 lbs (8 kg) without peripherals

Drive Bays

External	3 @ 5.25"; 1/2 height (1.6”)

1 @ 3.5"; 1/3 height (1.0")

Internal	2 @ 3.5"; 1/3 height (1.0")

Board Style	ATX standard mounting holes

Integrated baseboard I/O panel includes:

two serial ports, one parallel port, PS/2

style keyboard and mouse connectors

Board Size	9.6” x 12.0” (24.4 cm x 30.5 cm)

ATX information available at http://www.teleport.com/~atx

BASEBOARD POWER REQUIREMENTS

+3.3V	Tolerance ±5%, typical current 0.5A

+5V	Tolerance ±5%, typical current 6.0A

+12V	Tolerance ±5%, typical current 0.4A

-5V	Tolerance ±5%, typical current 0.01A

-12V	Tolerance ±5%, typical current 0.1A

SYSTEM ENVIRONMENTALS

Operating Temperature	+10°C to +35°C

Storage Temperature	-40°C to +70°C

Relative Humidity	90% RH @ 36°C

Operating Shock	Half sine, 2G, 11 msec

Unpackaged Vibration	2.2g RMS random 5-500Hz

Electrostatic Discharge	0 to 20.0 KV, limited errors, no hard failures

REGULATIONS

Safety

U.S.	UL 1950, 2nd edition

Canada	CSA C 22.2 No. 950M-93

Europe	TUV to EN 60950 with ZHI/618 ergonomics

Nordic	NEMKO IEC950

EMI/RFI

U.S.	FCC CFR 47 Part 15, Class B

Canada	DOC (IC) CRC ICES-003 Class B

Europe	EN55022, Class B & EN50082-1

International	CISPR22, Class B

CE

This product complies with European Directive 89/336/EEC when

tested in a representative chassis

ORDERING INFORMATION

For configurations available in your area, please contact your Intel field sales representative. For the number of your nearest Intel sales office,

call 800-628-8686 (U.S. and Canada) or +44 1793 403000 (Europe).

Information in this document is provided in connection with Intel products. No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in Intel’s Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel products are not intended for use in medical, life saving, or life sustaining applications. Intel retains the right to make changes to specifications and product descriptions at any time, without notice. This product may contain design defects or errors known as errata. Current characterized errata are available on request.

*Third-party brands and names are the property of their respective owners.

ãINTEL CORPORATION 1996		ORDER NUMBER 281819-001