# Advanced/MA

PentiumÒ processor-based Low Profile Desktop

Maximum Value from a

Fully Featured and Flexible Design

The Advanced/MA Low Profile is the ideal platform for applications taking advantage of today’s multimedia capabilities and looking toward the future. The highly-integrated baseboard provides an ATI mach64* graphics controller and 16-bit stereo audio to meet the most demanding needs. The result is reduced total system costs for the OEM and enhanced reliability, compatibility and ease-of-use.

This desktop offers the maximum range of scalability using PentiumÒ processors from 75 MHz to 133 MHz and beyond, along with the flexibility of a cache socket for either standard asynchronous SRAM or the higher-performing pipeline burst modules. Intel’s Triton chip set also supports advanced features like 100 MB/second PCI Data Streaming, support for EDO DRAM and Mode 4 drives, Bus Mastering IDE capability, plus compatibility with Windows*95.

Three ISA slots, two PCI slots, and four peripheral bays make the most of the expandability in a slimline chassis.

|  | Features |  |  | Benefits |  |
| --- | --- | --- | --- | --- | --- |
| · Support for Pentium processors from 75 MHz to 133 MHz and beyond |  |  | · High performance computing with the widest range of Pentium processors |  |  |
| · Cache module supports up to 512 KB of asynchronous, pipeline burst or synchronous burst cache |  |  | · Multiple cache options for scaleable price/performance and flexible inventory |  |  |
| · Integrated 64-bit ATI graphics and Crystal audio subsystems |  |  | · Lowers total system cost while enhancing reliability, compatibility, and ease-of-use |  |  |
| · Bus Mastering IDE |  |  | · Disk performance scales with Windows 95 |  |  |

| · Completely Plug and Play | · Easy to configure and setup; ready for Windows 95 |
| --- | --- |

PROCESSOR/CACHE

Advanced/MA Low Profile Desktop Specifications

Processor	Pentium processor at iCOMPÒ index:

1100 \ 133 MHz, 1000 \ 120, 815 \ 100,

735 \ 90, 610 \ 75 MHz and

future Pentium processors

Performance Upgrades	Supports a faster OverDriveÒ

processor upgrade

Cache Configuration	CELP connector accepts pipelined burst,

asynchronous or synch burst modules

SYSTEM MEMORY

Memory Capacity	4 72-pin sockets for tin-lead DRAM SIMMs

2 banks (SIMMs filled in pairs for 64 bit bus)

Memory Size	8 MB minimum (two 4 MB SIMMs)

128 MB maximum (four 32 MB SIMMs)

SIMM Sizes	4 MB, 8 MB, 16 MB, 32 MB

SIMM Types	70ns Fast Page Mode or EDO memory

(60ns EDO required @ 100 MHz/133 MHz)

Standard 32 bit SIMM module. SIMMs

can be mixed by bank; system will auto-				detect and optimize for EDO if present.

Memory Voltage	5 volts

INTEGRATED GRAPHICS

Controller	ATI mach64 CT

Memory	1 MB DRAM, upgradeable to 2 MB

Monitor Auto-configure	DDC-1 Compliant

Software enhancements	DCI and 3DR Software Driver Enabled

INTEGRATED AUDIO	CS 4232 and Yamaha OPL3LS

16 bit, Soundblaster*-class audio

Full Duplex enabled. 3.5 mm connectors

for line-in, line-out, mic-in, joystick/MIDI

ports. Can share PC speaker .

INTEGRATED PCI IDE

Number of Devices	2 independent channels, 4 IDE devices

Device Types	PIO Mode 3, Mode 4 IDE hard drives

ATAPI CD-ROM supported

Extended Modes	Master Mode IDE for Windows 95

INTEGRATED SUPER I/O

Controller	National 82C306

Two Serial Ports	Async, RS-232C, 9-pin 16C550 compatible

Parallel Port	Centronics compatible, 25-pin, ECP, EPP

Floppy Controller	2.88 MB, 765A compatible

Infrared Port	IrDA compliant interface to IR TX/RX

SYSTEM BIOS

BIOS Type	1 Mb Flash with Intel BIOS

Special Features	PC ‘95 compliant: Windows 95 ready Plug

and Play; APM 1.1; IDE drive

autoconfigure; Multilingual support.

PCI CHIP SET 	Intel Triton PCISet

82437FX, 82438FX, 82371FB

JUMPERS AND FRONT PANEL CONNECTORS

Connectors	Reset switch, keylock, external speaker,

LEDs (power, HDD, turbo), 12V CPU fan, 		Infrared, Sleep

Jumpers	Password  enable, CMOS clear and setup

disable, CPU bus speed and multiplier,

cache size

SYSTEM

Height	4.3" (11.7 cm)

Width	17.2" (43.7 cm)

Depth	16.2" (41.1 cm)

Weight	20 lbs (9.1 kg) without peripherals

Drive Bays

External	2 @ 5.25"; 1/2 hieght

1 @ 3.5"; 1/3 height (1.0")

Internal	1 @ 3.5"; 1/3 height (1.6")

ELECTRICAL

AC Voltage/Frequency	115v/60Hz; 230v/50Hz (switchable)

DC Power Supply	200w

+5V	18.6A maximum continuous

+12V	8.0A maximum continuous

-5V	0.5A maximum continuous

-12V	0.5A maximum continuous

ENVIRONMENT

Operating Temperature	+10°C to +35°C

Storage Temperature	-40°C to +70°C

Relative Humidity	92% RH @ 36°C

REGULATIONS

Safety

U.S.	UL 1950, 1st edition

Canada	CSA C 22.2 No. 950M-89

Europe	TUV to EN 60950 with ZHI/618

Ergonomics IEC950 with IEC 380

EMI/RFI

U.S.	FCC CFR 47 Part 15, Level B

Canada	DOC CRC c, 1374 Class B

Europe	VDE 0871 Class B, CISPR-B

ORDERING INFORMATION

Consult your Intel sales representative for exact configuration details before ordering.

For more information or the number of your nearest Intel sales office,

call 800-628-8686 (U.S. and Canada) or +44 1793 403 000 (Europe).

Intel Corporation disclaims all warranties and liabilities for the use of this document and the information contained herein, and assumes no responsibility for any errors that may appear in this document. Intel makes no commitment to update the information contained here, and may make changes at any time without notice. There are no express or implied licenses granted hereunder to any intellectual property rights of Intel Corporation or others to design or fabricate Intel integrated circuits or integrated circuits based on the information in this document. *Other product and corporate names may be trademarks or registered trademarks of other companies, and are used only for explanation and to the owners’ benefit, without intent to infringe.

ãINTEL CORPORATION 1995	ORDER NUMBER 281790-001