Server Platforms

Quick Guide

June, 1994

Available Monthly

Intel Server Business Unit

OEM Products Division

Confidential

CITR / NDA Required

Note:

Our goal is to be the server building block supplier of choice for OEM’s by delivering innovative, robust architectures to enable our customers to effectively and easily meet end user server needs.  This document is intended to provide our customers and Field Sales team the most up-to-date information on the OEM products in developement.  The document will be updated monthly, and you may request copies from your account manager or fax your request to receive monthly updates. If you have any comments or suggestions, please send them to my attention.

Major Changes from last publication:

- EISA & PCI NIC cards

- Adjusted Schedules

- Change on Entry PentiumÔ Processor Modules for Xtended Xpress

- Neptune based Pentium processor entry server for Q4-94

Thomas Urban

FAX (503) 640-6411

Product Manager - Server Business Unit

Andrew Greenhalgh

FAX (44) 793-480-452

European  Product Manager - Server Business Unit

i486, Intel486, Pentium, i960, LANDesk, IntelDX4, iCOMP, TokenExpress, EtherExpress, and OverDrive are trademarks of Intel corporation.

Other trademarks are the property of their respective owners.  Specifications subject to change.

Intel Confidential  Material, CITR / NDA required.

Intel corporation disclaims all warranties and liabilities for use  of this document and the information contained herein, and assumes no responsibility for any errors which may appear in this document.  Intel makes no commitment to update the information contained herein.

Intel grants no express or implied license to any intellectual property rights of Intel Corporation or others based upon the information in this document.

Intel retains the right to make changes to these specifications at any time, without notice.

Table of Contents

Available Today

OEM Server Sales Tools

Entry Server Chassis

Mid-Range Server Chassis

Expandable Desktop Chassis

High Capacity Rack Mountable Chassis

Industrial Rack Mountable Chassis

Xpress Server Baseboard (6-slot)

Xpress Server Baseboard (8-slot)

Parity Memory Module

i486Ô Processor Module

60 MHz PentiumÔ Processor Module

66 MHz PentiumÔ Processor Module

EISA RAID Disk Controller

TokenExpressÔ EISA/32 LAN adapter

EtherExpressÔ Flash32 LAN adapter

LANDeskÔ Software Manager V1.51

Server Diagnostics

100 & 90 MHz PentiumÔ Processor Modules

Dual 66 MHz PentiumÔ Processor Module

ECC Memory Module

Available H2-94

i486Ô Processor Module, 512 KB L2

Server Management Server Monitor

Mid-Range Server Baseboard (Flat Design)

Entry Server Chassis

EtherExpressÔ PRO/100 LAN adapter

Available H1-95

Entry / Mid-Range Server Chassis

Mid-Range Expanded Feature Server Chassis

Cube Chassis

Xtended Xpress Entry Baseboard

Xtended Xpress Mid-Range Baseboard

Xtended Xpress High-End Baseboard

100 MHz XXpress PentiumÔ Processor Module

PCI RAID Disk Controller

Price / Performance PentiumÔ Processor Module

Dual PentiumÔ Processor Module, 1MB L2

Dual PentiumÔ Processor Module, 2MB L2

# OEM Server Sales Tools

1-800 548-4725

| Technical Product Summaries |  |  |
| --- | --- | --- |
|  | Systems Overview | 281397-004 |
|  | CPU Modules, Baseboards, RAID | 281562-002 |
|  | OmniRACK | 272401-001 |
|  | Xtended Xpress | 281609-001 |
| Xpress Benchmark Report |  | 281380-002 |
| Application Notes |  |  |
|  | Configuration Tips | 281398-002 |
|  | RAID disk array controller | 281563-001 |
|  | Dual Pentium Technology and ECC | 281606-001 |
|  | Server Monitor | 281625-001 |
| Fact Sheets |  |  |
|  | Desktop | 281377-004 |
|  | Deskside /MX | 281379-003 |
|  | Integrated /LX | 281607-001 |
|  | Deskside /LX | 281558-003 |
|  | RAID controller | 281559-001 |
|  | XpressRACK | 274136-001 |
|  | OmniRACK | 272400-001 |
| Complete Xpress Family Package |  | 281570-001 |
| Folder for custom package (Empty) |  | 281569-001 |
| External Product Specifications | Xpress Xtended Xpress 15" Dual PentiumÔ Processor Module ECC Memory Module i486Ô Processor Module 90 MHz Pentium Processor Module Server Management Specification (H/W, S/W, BIOS) | - Restricted Use - Call Account Manager |
| Other | Fold-out product overview Intel-Based Server Catalog | 280946-008 241894-001 |

# Entry Server Chassis

Product Code:   XLXBASE8TE8F (US)

EXLX8E8F220V (Europe)

(Code includes baseboard, other configurations available)

Description:

Entry Server Chassis

Positioning:

Expandable chassis design targeted for Entry server price bands

Key Features:

ü	Three external bays

ü	Support for i486Ô and PentiumÔtechnology processors

ü	OEM customizable bezel

Key Benefits:

ü	Robust Design (Security, thermal guardband, EMI protection)

ü	Expandable

Availability:

Volume 	Today

Technical Specifications:

| CPU Family | i486 CPU, Pentium Processor |
| --- | --- |
| I/O |  |
| Slots | 8 |
| Ports | 1 x Video, 2 x Serial, 1 x Parallel, Keyboard, Mouse |
| Bays |  |
| Internal | 5 x 3.5, 1.6" |
| External | 3 x 5.25" |
| Buttons | Reset, Front Power, Keyboard Inhibit |
| Indicators | 1 x Power on 2 x HDD Activity |
| Dimensions |  |
| Height | 20.4" |
| Width | 8.5" |
| Depth | 23" |
| Power Supply |  |
| AC Voltage / Freq | 100-120/200-240, 60/50Hz Switch Selectable |
| DC Power | 230 W |
| +5V | 30 A max continuous |
| +12V | 8 A max continuous |
| -12V | 0.5 A max continuous |
| -5V | 0.5 A max continuous |
| Noise | <45 db @ 25°C |
| Security | Rear lock cable hook, Front drive bay door lock, Master key available |
| Cooling | 1 x Power supply fan 1 x Interior fan (thermal) |
| FCC | Class B |
| Safety | UL, CSA, TUV, NORDIC |
| Target Baseboards | Xpress |

# Mid-Range Server Chassis

Product Code:   XBASE8TE8F-C

(Code includes baseboard, other configurations available)

Description:

Server Chassis

Positioning:

Highly expandable chassis design targeted for mid-range server price bands

Key Features:

ü	Three external bays

ü	Support for i486Ô and PentiumÔ technology generation of  processors

ü	Dual Processing

ü	OEM customizable bezel

Key Benefits:

ü	Robust design

ü	Highly expandable

Availability:

Volume 	Today

Technical Specifications:

| CPU Family | i486 CPU, Pentium Processor, Dual Pentium Processors |
| --- | --- |
| I/O |  |
| Slots | 8 |
| Ports | 1 x Video, 2 x serial, 1 x Parallel, Keyboard, Mouse |
| Bays |  |
| Internal | 8 x 3.5, 1.6" SCSI power and signal Distribution Board |
| External | 3 x 5.25" |
| Buttons | Reset, Front & Rear Power, Keyboard Inhibit |
| Indicators | 1 x Power on 1 x HDD activity Programmable LCD |
| Dimensions |  |
| Height | 24" |
| Width | 8.5" |
| Depth | 23" |
| Power Supply |  |
| AC Voltage / Freq | 100-120/200-240, 60/50Hz Autoranging |
| DC Power | 384 W |
| +5V | 45 A max continuous |
| +12V | 12 A max continuous |
| -12V | 1.0 A max continuous |
| -5V | 0.5 A max continuous |
| Noise | <45 db @ 25°C |
| Security | Rear lock, Front drive bay door lock Master key available |
| Cooling | 1 x Power supply fan 1 x interior fan (thermal) |
| FCC | Class B |
| Safety | UL, CSA, TUV, NORDIC |
| Target Baseboards | Xpress |

# Expandable Desktop Chassis

Product Code:   XBASE6E8F-B

(Code includes baseboard, other configurations available)

Description:

Expandable desktop chassis

Positioning:

Highly expandable chassis design targeted for mid-range desktop price bands

Key Features:

ü	Three external bays

ü	Support for i486Ô and PentiumÔ technology generation of  processors

ü	Mechanical security locks

Key Benefits:

ü	Robust design

ü	Highly expandable

Availability:

Volume 	Today

Technical Specifications:

| CPU Family | i486 CPU, Pentium Processor |
| --- | --- |
| I/O |  |
| Slots | 6 |
| Ports | 1 x Video, 2 x Serial, 1 x Parallel, Keyboard, Mouse |
| Bays |  |
| Internal | 1 x 3.5", 1.6" 1 x 3.5 ", 1" |
| External | 2 x 5.25" 1 x 3.5" |
| Buttons | Reset, Front Power |
| Indicators | 1 x Power on 1 x HDD activity |
| Dimensions |  |
| Height | 6.2" |
| Width | 17.3" |
| Depth | 17.1" |
| Power Supply |  |
| AC Voltage / Freq | 100-120/200-240, 60/50Hz Autoranging |
| DC Power | 215 W |
| +5V | 32 A max continuous |
| +12V | 4.0 A max continuous |
| -12V | 0.5 A max continuous |
| -5V | 0.2 A max continuous |
| Noise | <45 db @ 25°C |
| Security | Front door lock |
| Cooling | 1 x Power supply fan 1 x Interior fan (thermal) |
| FCC | Class B |
| Safety | UL, CSA, TUV, NORDIC |
| Target Baseboards | Xpress |

# High Capacity Rack Mountable Chassis

Product Code:   	RXBASE6E4P1				RXBASE8E8P2C

(Code includes baseboard, other configurations available)

Description:

Industrial rack mountable chassis

Positioning:

Robust chassis designed to support large power supplies, drives, and on board SCSI.

Key Features:

ü	Four external bays (5 1/4")

ü	Support for i486Ô and PentiumÔ technology generation of processors

ü	Dual Processing

ü	Meets industrial environmental specifications

ü	Fits standard 19" industrial rack

Key Benefits:

ü	Robust design

ü	Highly expandable

Availability:

Volume 	Today

Technical Specifications:

| CPU Family | i486 CPU, Pentium Processor |
| --- | --- |
| I/O |  |
| Slots | 6 or 8* |
| Ports | 1 x Video, 2 x serial, 1 x Parallel, Keyboard, Mouse |
| Bays |  |
| External | 4 x 5.25" Can be recessed |
| Buttons | Reset, Front Power |
| Indicators | 1 x Power on 1 x HDD activity Programmable LCD |
| Dimensions |  |
| Height | 7" |
| Width | 19" |
| Depth | 25.6" |
| Power Supply |  |
| AC Voltage / Freq | 100-120/200-240, 60/50Hz Autoranging / Selectable* |
| DC Power | 396 W / 300 W* |
| +5V A max cont. | 45 / 35* |
| +12V A max cont. | 13 / 10* |
| -12V A max cont. | 1.0 / 0.5* |
| -5V A max cont. | 0.5 / 0.5* |
| Cooling | Dual fans |
| Other | Captive fasteners Air filter (optional) Positive pressure enclosure |
| Certifications | FCC Class A CISPR 22 Class B VDE Class B VCCI Class 2 |
| Safety | UL, CSA, TUV |
| Target Baseboards | Xpress, Xtended Xpress 12" |

* Depending on baseboard

# Industrial Rack Mountable Chassis

Product Code:   	ORBASE6DT4

ORBASE8LX8

(Code includes baseboard, other configurations available)

Description:

Industrial rack mountable chassis

Positioning:

Mid-range compact chassis designed to support i486Ô and PentiumÔ technology baseboards

Key Features:

ü	Four external bays

ü	Support for i486Ô and PentiumÔ technology generation of  processors

ü	Meets industrial environmental specifications

ü	Fits standard 19" industrial rack

ü	Locking security door

ü	Temperature controlled fans

Key Benefits:

ü	Robust design

ü	Supports heavy +5V loading

ü	Ease of integration / maintenance

Availability:

Volume 	Today

Technical Specifications:

| CPU Family | i486 CPU, Pentium Processor |
| --- | --- |
| I/O |  |
| Slots | 6 or 8* |
| Ports | 1 x Video, 2 x serial, 1 x Parallel, Keyboard, Mouse |
| Bays |  |
| External | 2 x 5.25" 1 x 3.5", 1.6" 1 x 3.5", 1" |
| Buttons | Reset, Front Power Keyboard lock* |
| Indicators | 1 x Power on 1 x HDD activity |
| Dimensions |  |
| Height | 7" |
| Width | 19" |
| Depth | 17.8" |
| Power Supply |  |
| AC Voltage / Freq | 100-120/200-240, 60/50Hz User selectable |
| DC Power | 230 W |
| +5V | 30 A max continuous |
| +12V | 8 A max continuous |
| -12V | .5 A max continuous |
| -5V | .5 A max continuous |
| Cooling | Dual fans Temperature controlled |
| Other | Removable drive cage Standard air filter Positive pressure enclosure Mounting rails optional |
| Certifications | FCC Class A subpart B CISPR 22 Class B VDE Class B VCCI Class 2 |
| Safety | UL, CSA, TUV |
| Target Baseboards | Xpress, Xtended Xpress 12" |

* Depending on baseboard

# Xpress Server Baseboard (6-slot)

Product Code:   BXBASE6E0-B

Description:

Xpress  6-slot server baseboard

Positioning:

Highly integrated server baseboard designed for entry server applications

Key Features:

ü 	384 MB memory capacity (ECC Optional)

ü	Scalable from i486Ô to PentiumÔ processors

ü	Integrated I/O

Key Benefits:

ü	Highest performance sub-systems

ü	90 MHz Pentium processor / IntelDX4Ô CPU ready

Availability:

Volume 	Today

Technical Specifications:

| CPU Family |  |
| --- | --- |
| Family | i486, Pentium Processor |
| CPU Slots | 1 |
| Memory |  |
| Architecture | 64 bit interleaved |
| Bus Width | 32 bit |
| Transfer Bandwidth | 133 MB/s (peak) |
| Capacity | 384 MB |
| Type | Parity or ECC, STD x36 SIMMs |
| SIMM sites | 4, 128 MB |
| Memory Expansion | Parity Module w/ 8 SIMM sites, 256 MB |
| BIOS |  |
| Type | Enhanced Phoenix |
| Component | 256 KB Flash 8 KB User Block |
| Languages | 5 language support |
| Key Features | Memory Remapping, Error Logging |
| Graphics |  |
| Controller | WD90C31 |
| Bus | ISA |
| Memory | 512 KB DRAM expandable to 1MB |
| Resolutions | 640x480, 800x600, 1024x768, 1280 x 1024 |
| Colors | 16 Million |
| Output Ports | 15 pin analog 8514/A VESA Connector |
| Disk Controller |  |
| IDE | DMA transfers |
| I/O |  |
| EISA Slots | 6 Bus Master |
| Other | 2 x Serial w/ FIFOs 1 x Parallel (ECP, bi-directional) 1 x PS/2 Keyboard, 1 x PS/2 mouse |
| Dimensions |  |
| Length / Width | 10" / 13" |

# Xpress Server Baseboard (8-slot)

Product Code:   BXBASE8E0-C

Description:

Xpress  server baseboard

Positioning:

Highly integrated server baseboard designed for mid range server applications

Key Features:

ü	Dual Processing support

ü 	384 MB memory capacity (ECC Optional)

ü	Scalable from i486Ô to Dual PentiumÔ processor

ü	Integrated I/O

Key Benefits:

ü	Highest performance sub-systems

ü	New Technology throughout 1994

ü	90 MHz Pentium processor / IntelDX4Ô CPU ready

Availability:

Volume 	Today

Technical Specifications:

| CPU Family |  |
| --- | --- |
| Family | i486, Pentium Processor, Dual Pentium processor |
| CPU Slots | 1 |
| Memory |  |
| Architecture | 64 bit interleaved |
| Bus Width | 32 bit |
| Transfer Bandwidth | 133 MB/s (peak) |
| Capacity | 128MB Baseboard 256MB w/ Expansion Module 384MB Total |
| Type | Parity or ECC, STD x36 SIMMs |
| SIMM sites | 4, 128 MB |
| Memory Expansion | Module w/ 8 SIMM sites ,256 MB |
| BIOS |  |
| Type | Enhanced Phoenix |
| Component | 256 KB Flash 8 KB User Block |
| Languages | 5 language support |
| Key Features | Memory Remapping, Error Logging |
| Graphics |  |
| Controller | WD90C31 |
| Bus | ISA |
| Memory | 512 KB DRAM expandable to 1MB |
| Resolutions | 640x480, 800x600, 1024x768, 1280 x 1024 |
| Colors | 16 Million |
| Output Ports | 15 pin analog 8514/A VESA Connector |
| Disk Controller |  |
| SCSI | 2 x Fast Adaptec 7770-C Bus Master |
| IDE | DMA transfers |
| I/O |  |
| EISA Slots | 8 Bus Master |
| Other | 2 x Serial w/ FIFOs 1 x Parallel (ECP, bi-directional) 1 x PS/2 Keyboard, 1 x PS/2 mouse |
| Dimensions |  |
| Length / Width | 12" / 13" Also available 10" / 13" (6-slots) |

# Parity Memory Module

Product Code:   BXMEM0

Description:

An Parity memory expansion board designed for use in Xpress based systems.

Positioning:

Provides memory expansion for the Xpress line of server platforms.

Key Features:

ü	256 MB Memory Capacity

ü	Standard X36 SIMMs

ü	64 bit architecture

ü	32 bit data transfers

Key Benefits:

ü	Provides memory expansion

ü	No custom SIMMs required

ü	Modular approach does not burden baseboard

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

| Memory |  |
| --- | --- |
| Sockets | 8 |
| Capacity | 256 MB |
| Technology | 1Mb through 16Mb DRAM 36 bit, Parity, single or double density, 4-32MB SIMMS |
| Expansion | Increments of two, same size SIMMs |
| Architecture | 64 bit, 2 way interleave |
| Interface | Xpress 32 bit |
| Mechanical | Width - 12.4" |
|  | Height - 4.85" |
| Performance | Same as baseboard memory |
| Packaging | Single w/ manual Bulk Qty 20 no manual |
| Configuration | Software memory sizing |
|  | Bad SIMM remapping |
| Environmental |  |
| Temperature | 0 - 55° C air flow 200 LFM |
| Voltage | +5 VDC operation at 25 Watts maximum |
| FCC | Class B |
| Target Baseboards | Xpress |

# i486Ô Processor Module

Product Code:   BXCPU486DX266-B

BXCPU48633-B

Description:

Xpress CPU module for the Intel486Ô Microprocessors.

Positioning:

Entry and mid-range performance CPU modules designed for file server and compute intensive applications.

Key Features:

ü	Intel verified for PentiumÔ OverDriveÔ processor

ü	Up to 128 KB L2 cache

ü	Multiple frequencies supported

Key Benefits:

ü	Multiple price/performance points

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

| CPU |  |
| --- | --- |
| Family | i486 Technology |
| i486DX Int/Ext Freq MHz L1 Cache Bus data/address Voltage | 33/33 MHz 8 KB WB 32/32 bit 5 V |
| i486DX2 Int/Ext Freq MHz L1 Cache Bus data/address Voltage | 66/33 MHz 8 KB WB 32/32 bit 5 V |
| Cache Controller | Intel |
| Mode | Write-through |
| Cache |  |
| Size (KB) | 0, 128 |
| Parity | None |
| Voltage | 5V |
| Interface | Xpress 32 bit |
| OverDrive | 237 pin ZIF |
| Mechanical | Width - 7.2" |
|  | Height - 4.85" |
| Performance (SPECint92/fp92) |  |
| i486DX2/66 | 31.8 / 14.4 |
| i486DX/33 | 17.6 / 7.3 |
| Packaging | Single, Bulk Qty 20 |
| FCC | Class B |
| Target Baseboards | Xpress |

# 60 MHz PentiumÔ Processor Module

Product Code:   BXCPUPENT60

Description:

Xpress CPU module for the Intel 60 MHz  PentiumÔ Microprocessor.

Positioning:

High performance Pentium Processor module designed for transaction processing applications.

Key Features:

ü	60 MHz Internal/External CPU speed

ü	256 KB Write-back L2 cache

ü	0 Wait state two way set associative L2

ü	Backward compatible to installed base

Key Benefits:

ü	New price/performance point

ü	Highest performance L2 cache

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

| CPU |  |
| --- | --- |
| Family | PentiumÔ Processor Class |
| Frequency MHz L1 Cache Bus data/address Voltage | 60 MHz 8 KB data WB 8 KB code 64/32 bit 5.0 V |
| Cache Controller | Intel 82496, C5 |
| Mode | Write-Back Two way set associative 0 wait state |
| Cache |  |
| Size (KB) | 256 |
| Component | Intel 82491, C8 |
| Parity | None |
| Quantity | 8 |
| Voltage | 5.0 V |
| Interface | Xpress 32 bit |
| Mechanical | Width - 7.325" |
|  | Height - 4.85" |
| Performance |  |
| SPECint92 | 50.4 |
| SPECfp92 | 28.3 |
| MIPs | 100 |
| iCOMPÔ | 567 |
| Packaging | Single |
| FCC | Class B |
| Target Baseboards | Xpress, Xtended Xpress |

# 66 MHz PentiumÔ Processor Module

Product Code:   BXCPUPENT66

Description:

Xpress CPU module for the Intel 66 MHz  PentiumÔ Microprocessor.

Positioning:

Highest performance Pentium Processur module designed for transaction processing applications.

Key Features:

ü	66 MHz Internal/External CPU speed

ü	256 KB Write-back L2 cache

ü	0 Wait state two way set associative L2

ü	Backward compatible to installed base

Key Benefits:

ü	New price/performance point

ü	Highest performance L2 cache

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

| CPU |  |
| --- | --- |
| Family | Pentium Processor Class |
| Frequency MHz L1 Cache Bus data/address Voltage | 66 MHz 8 KB data WB 8 KB code 64/32 bit 5.4 V |
| Cache Controller | Intel 82496, C5 |
| Mode | Write-back Two way set associative, 0 wait state |
| Cache |  |
| Size (KB) | 256 |
| Component | Intel 82491, C8 |
| Parity | None |
| Quantity | 8 |
| Voltage | 5.0 V |
| Interface | Xpress 32 bit |
| Mechanical | Width - 7.325" |
|  | Height - 4.85" |
| Electrical | Power converter +5V to 5.4 V |
| Performance |  |
| SPECint92 | 67.4 |
| SPECfp92 | 63.6 |
| MIPs | 112 |
| iCOMPÔ | 567 |
| Packaging | Single |
| FCC | Class B |
| Target Baseboards | Xpress, Xtended Xpress |

# EISA RAID Disk Controller

Product Code:   XRAID4

Description:

Two channel, SCSI RAID (Redundant Array of Inexpensive Disks) controller to provide fault tolerant I/O.

Positioning:

Provides proactive fault detection and prevention when the server is running.

Key Features:

ü  Two I/O channels support up to 14 drives

ü  Fast SCSI-2 for 10 MB/s data transfers

ü  Support for standard I/O devices

ü  RAID 0,1,5, + other combinations

ü  Flash firmware and BIOS

ü  Up to 64MB Write back cache

ü  External device support

ü  Simultaneous data transfer

ü  Automatic drive failure detection / hot spare, background rebuild

ü  Complete utilities

Key Benefits:

ü  High performance

ü  Redundancy / availability

ü  Zero unscheduled downtime

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

| Board |  |
| --- | --- |
| CPU | i960CA 32-bit microprocessor |
| Cache option | 4, 16 or 64 MB |
| Standard | 4 MB |
| Cache support | Write-through |
|  | Write-back |
|  | Cache flush |
| SCSI support | SCSI II fast , 8-bit wide |
| SCSI Channels | 2 (7 devices per channel) |
| SCSI Controller | NCR 53C720 PT |
| EISA Bus Master | Supports EISA burst mode transfers of up to 33 MB/s |
| Onboard BIOS | Flash EEPROM (16K x 8) |
| Firmware | Flash EEPROM (16K x 8) |
| RAID Levels | 0 - Block striping, no redundancy |
|  | 1 - Mirroring |
|  | 5 - Block striping with rotated XOR redundancy |
|  | Striping & mirroring |
| Multi-threaded | 64 simultaneous commands |
| Scatter/Gather |  |
| Fault Tolerance | Automatic drive failure detection |
|  | Hot spare |
|  | On-line rebuilding of replacement drive |
| Power Sequencing |  |
| Environment | (System Level) |
| Ambient Temperature: |  |
| Operating | +10oC to +35oC ambient |
| Non-Operating | -40oC to +70oC ambient |
| Static Discharge | 25 kV no component damage |
| Regulations |  |
| Meets or exceeds the following requirements: |  |
| EMI/RFI |  |
| U.S. | FCC Class B |
| Software |  |
| OS Support | SCO* UNIX 3.2 version 4 |
|  | Novell Netware* 3.11, 3.12, 4.0 |
|  | Windows* NT 3.1 |
|  | IBM* OS 2.0, 2.1 |
|  | USG UNIX* SV R4.2 |
|  | Banyan 5.53 |
| Tools |  |
| Configuration | Menu driven DOS utility |
| OS Utilities | Utilities for all supported operating systems |

# TokenExpressÔ

# EISA/32 LAN adapter

Product code:  EILA8245

Description:

32-bit, high performance EISA Token

Ring LAN adapter card

Positioning:

Provides high performance network connection for Token Ring environments.

Key Features:

ü  True 32-bit high performance

ü	Interoperability with all IBM computing environments

ü	Fully software configurable

ü  Supports all major network operating systems

ü  Supports RJ-45 and DB-9 connections at 4 and 16 Mbps

ü  SNMP-manageable through Intel’s LANDeskÔ SNMP gateway

ü	High integration for superior reliability

ü	Limited lifetime warranty

Key Benefits:

ü	Increases network performance with speed-optimized design

ü	Provides flexible installation options

ü	Easy to install, use and manage

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

High performance architecture

Throughput is up to twice that of other EISA adapters for frames under 256 bytes.

32-bit bus mastering enables high speed data transfers with direct communication to and from host memory

Highly tuned drivers for maximum performance

512K on-board high-speed RAM

Low CPU utilization, ideal for servers

Flexible installation options

Supports interrupt & DMA sharing, allowing up to 8 boards in one server

Supports shielded twisted pair (DB-9) and unshielded twisted pair (RJ-45) at 4 and 16Mbps

Supports all major network operating systems

Novell NetWare*

Microsoft* LAN Manager

Microsoft Windows* for Workgroups

Microsoft Windows NT

IBM* DLC

IBM OS/2

IBM LAN Server & Extended Edition

IBM Communications Manager 2.1

IBM LAN Enabler

IBM PC LAN

IBM AS/400 support

SCO UNIX V rel. 3.2.4

SCO Unix System V (UnixWare)

Interactive Unix rel 3.2 version 3.0

Banyan VINES* NDIS client

Supports IEEE 802.2 and 802.5 standards

Management support

SNMP-manageable with Intel’s LANDeskÔ SNMP Gateway

Intel LANSight Express remote node management software available free on Intel’s BBS

For more information:

US: 800-538-3373 UK: +44-793-431155 Australia: +61-2-975-3300 Singapore: +65-250-7811

# EtherExpressÔ Flash32 LAN adapter

Product code:  EILA8225

Description:

32-bit high performance EISA Ethernet adapter card

Positioning:

Provides high performance network connection for Ethernet environments.

Key Features:

ü  True 32-bit high performance

ü  Fully software configurable; uses 256K of Flash memory for fast, autosensing installation

ü  Supports all major network operating systems

ü  BNC, RJ-45 & AUI connectors on every board for maximum flexibility

ü  SNMP-manageable through Intel’s LANDeskÔ SNMP gateway

ü	High integration for superior reliability

ü	Includes remote boot software

ü	Limited lifetime warranty

Key Benefits:

ü	Increases network performance with speed-optimized design

ü	Provides superior reliability & low price thanks to high level of integration

ü	Easy to install, use and manage

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

High performance architecture

32-bit direct bus mastering enables high speed data transfers with direct communication to and from host memory

On-board Intel 82596 Ethernet coprocessor with internal four-channel DMA controller

One-step, autosensing installation

Setup program automatically configures the card for interrupt-free request lines & addresses

RJ-45, BNC & AUI connectors on-board: card auto-senses the connector type used

High integration for superior reliability & low price

Highly integrated hardware design

256K Flash memory

On-board diagnostics

Supports all major network operating systems

Novell NetWare*

Microsoft* LAN Manager

Microsoft Windows* for Workgroups

Microsoft Windows NT

Artisoft LANtastic

Banyan VINES*

SCO UNIX

DEC PathWorks*

IBM* LAN Server

Management support

SNMP manageable via Intel LANDeskÔ SNMP Gateway

Intel LANSight Express remote node management software available free on Intel’s BBS

Supports IEEE 802.2 and 802.3 standards

For more information:

US: 800-538-3373 UK: +44-793-431155 Australia: +61-2-975-3300 Singapore: +65-250-7811

# LANDeskÔ Software Manager V1.51

Description:

Central management console for managing networked PCs.

Positioning:

Provides completely integrated suite of solutions to manage your desktops, servers, segments, and services

Key Features:

ü  Desktop Access

ü  Traffic Monitor

ü  Inventory Manager

ü  Application monitor

ü  Virus Scan

ü  Server Monitor, Status

ü  Remote Connection Manager

ü  Network Printer Manager

ü  Queue Monitor

ü  Alert Log

ü  Software Probe

Key Benefits:

ü	Manage proactively with an integrated suite of solutions

ü	Centrally access desktops, servers, segments, and services

ü	Windows* based network management software for Netware*

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

Simple network administration

Manage LAN from a single point of control

Real-time summary of all LAN HW/SW resources

Remote workstation control for fast troubleshooting

Centrally view network information to anticipate problems

Provides drag & drop printer selection

Sends alert log relays via beeper, alpha pager, cc:Mail (VIM compatible), or executable file

New Version 1.51 features

Integrates with Novell NMS* 2.0 and SynOptics Optivity* v2.1 for NMS

Supports NetWare 3.11, 3.12, and 4.01 (bindery emulation)

Asynchronous support for monitoring and controlling your network from a remote location

Supports VLMs that provide manageable TSRs at the workstation

Proactive LAN management

Monitors network health indicators (packet rates, utilization, error levels, etc)

Enables network performance tuning

Real-time graphic summary of application-related traffic

Protects workstations from common, stealth, and polymorphic viruses

Automatic network redirection to minimize printer failure impact

Provides open, standards-based environment for future expansion

Compatible with industry standards such as SNMP and Dynamic Data Exchange (DDE)

Complete Solution

Includes tools for server application monitoring, network activity monitoring, hardware and software inventory, virus scanning and removal, printer management, and remote control.

Windows-based control panel lets you seamlessly jump from one tool to another and makes analyzing information easy.

For more information:

Product: 800-538-3373

FaxBack: 800-525-3019

FaxBack Document numbers: Virus Protect 9558 SNMP Gateway 9626 OpenView for Window 5500 Response 5505 Traffic Analyst 9551

# Server Diagnostics

Product Code:   XDIAGNOSTICS

Description:

Comprehensive diagnostic package to test and diagnose all major server subsystems.

Positioning:

Diagnostic package originally designed for Intel's manufacturing and repair facilities has been packaged for OEM, integrator, and troubleshooting use.

Key Features:

ü	Test all major system components

ü	Windowed or command line interface

ü	Modular and Configurable

ü	Runs under DOS

ü	Complete on-line help documentation

ü	Auto-sensing option

ü	Developer's kit and Source code available

Key Benefits:

ü	Complete testing

ü	Quick trouble shooting and debug

ü	Easy customization

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

Testview Executive Environment

Windowed user interface

Test selection and configuration

Load and save custom test suites

Complete online - context sensitive help

Reminders of interactive, hardware required, and destructive tests when user enables each

Customizable sign-on line, colors, default tests suite, and command line parameters

Error and failure summaries

Testview Menus & Utilities

Test - list of test and subtests

Error - Allows viewing, clearing, saving, and redirecting summary list - Red flashing indicator of errors until viewed

Summary - List of test run, iterations, last time of execution - Allows viewing, clearing, saving, and redirecting

Configuration - Configure test menu from file or list - Save configuration - Self-configuration based on H/W

Options - Printer redirection - Halt execution on error - Prompt to abort or continue on error - Run all tests - Use or define a test run list - Interactive run time flags

Help - Complete on-line documentation - Print options

Utilities - Including cache interrogation, CMOS/EISA configuration tools, BIOS check, elapse time monitoring, etc.

Test Modules

Cache, CPU, DMA, Dual Processor, ECC, EEPROM, Floppy, Front Panel, Hard Drive, Keyboard Controller, LED Displays, Numeric Processing Unit, Modem, System Memory and Memory Controller, Parallel I/O, Programmable Interrupt Controller, Programmable Interrupt Timer, Power-on Self Test, Memory Size Check, Real Time Clock, Serial Controller, Western Digital Video Controller, and others

Target Platforms: Xpress, Xtended Xpress, Other OPSD, Other

# 100 & 90 MHz PentiumÔ Processor Modules

Product Code:      BXCPUPENT90

BXCPUPENT100*

Description:

Xpress CPU module for Pentium processor at iCOMPÔ index 815\100 MHz and Pentium processor at iCOMP index 735\90 MHz.

Positioning:

Highest performance CPU module in the Pentium family designed for transaction processing applications.

Key Features:

ü	90/60 MHz Internal/External CPU speed

ü	100/66 MHz price point also available

ü	512 KB Write-back L2 cache

ü	0 Wait state two way set associative L2

ü	Backward compatible to installed base

Key Benefits:

ü	Time to Market with next generation PentiumÔ processors

ü	New price/performance point

ü	Highest performance L2 cache

Availability:

| Volume | Today** |
| --- | --- |

Technical Specifications:

| CPU |  |
| --- | --- |
| Family | Pentium Processor Class |
| Int/Ext Freq MHz L1 Cache Bus data/address Voltage | 90/60 MHz 8 KB code 8 KB data WB 64/32 bit 3.3 V |
| Int/Ext Freq MHz L1 Cache Bus data/address Voltage | 100/66 MHz 8 KB code 8 KB data WB 64/32 bit 3.3 V |
| Cache Controller | Intel 82497, C5C' |
| Mode | Write-back Two way set associative 0 wait state |
| Cache |  |
| Size (KB) | 512 |
| Type | Synchronous SRAMs |
| Component | Intel 82492, C8C' |
| Parity | None |
| Quantity | 16 |
| Voltage | 3.3V / 5.0 V |
| Interface | Xpress 32 bit |
| Mechanical | Width - 12.34" |
|  | Height - 4.85" |
| Electrical | Power converter +5V to 3.3 V |
| Performance |  |
| iCOMP | 735 / 815 |
| AIM Technology (unaudited) | 70.9, 540 User loads |
| BAPCo93 | TBD |
| Packaging | Single |
| FCC | Class A - May 15 Class B - After May 15 |
| Target Baseboards | Xpress |

* H2-94 availability based on corporate component commitment.

** 90 MHz only

# Dual 66 MHz PentiumÔ Processor Module

Product Code:   BXCPU2XPENT66

Description:

Xpress Multi-Processing CPU module with two 66 MHz PentiumÔ  Microprocessors operating in symmetric or asymmetric mode.

Positioning:

Highest performance CPU module designed for the most demanding transaction processing applications.

Key Features:

ü	Two 66 MHz Pentium Microprocessors

ü	Two 256KB L2 Write-back caches

ü	Cache to cache transfer (data snarfing)

ü	Symmetric mode support ("-C" baseboard)

ü	Backward compatible to installed base

ü	Support for major operating systems

Key Benefits:

ü	Extends Xpress platforms into multi-processing market

ü	Over 180 MIPs processing power

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

| CPUs |  |
| --- | --- |
| Family | PentiumÔ Processor |
| Internal Freq. | 66 MHz |
| External freq | 66 MHz |
| L1 Cache | Data 8KB WB Code 8KB |
| Voltage | 5.4V (self-regulated) |
| Cache Controllers | Intel 82496 |
| Mode | Write back Two way set associative |
| Interrupt Controllers | Intel 82489DX APICs PC+MP compliant or Intel 82C59 (asymmetric) |
| Cache |  |
| Size (KB) | 2 x 256 KB |
| Component | Intel 82491 |
| Parity | None |
| Voltage | 5.0 volts |
| Interface | Xpress 32 bit |
| OverDrive Socket | Electrically compatible 2 x LIF |
| Mechanical | Width - 12.3" |
|  | Height - 4.85" |
| Performance |  |
| SPECint92/fp92 | TBD |
| MIPS | 180 |
| Packaging | Single |
| Server Management | None |
| OS Support |  |
| Symmetric* (*Requires -C baseboard or later) | SCO* MPX MS Windows* NT IBM* OS/2* 2.1 Solaris |
| Asymmetric | Banyan* Vines 5.54 |
| FCC | Class B |
| Target Baseboards | Xpress, Xtended Xpress |

# ECC Memory Module

Product Code:   BXECCMEM0

BXECCMEM0BLK

Description:

An Error Checking and Correcting (ECC) memory board designed for use in Xpress and Xtended Xpress systems.  The board provides in line, real time correction of single bit errors from a DRAM memory array.

Positioning:

Provides fault tolerance to the memory sub-system for mission critical applications.

Key Features:

ü	384 MB Memory Capacity

ü	Standard X36 SIMMs

ü	On-the-fly single bit correction

ü	Double bit detection*

ü	Error logging*

ü	128 bit architecture

ü	32 or 64 bit data transfers

Key Benefits:

ü	Provides Fault Tolerance

ü	No custom SIMMs required

ü	Modular approach does not burden baseboard

Availability:

| Volume | Today |
| --- | --- |

Technical Specifications:

| Memory |  |
| --- | --- |
| Sockets | 12 |
| Capacity | 384 MB |
| Technology | 1Mb through 16Mb DRAM 36 bit, Parity, single or double density, 4-32MB SIMMS |
| Expansion | Increments of four, same size SIMMs |
| Organization | Two banks of 72 bits, Total width 144 bits |
| Architecture | 128 bit, 2 way interleave |
| ECC Features | Real time detection |
|  | Correct single bit errors |
|  | Detect double bit errors |
|  | Bad nibble detect |
| Interface | Xpress 32 bit Xtended Xpress 64 bit |
| Mechanical | Width - 12.4" |
|  | Height - 4.85" |
| Performance (recommend Pentium or newer module) | Line Fill / Write |
| XX board, 64 bit cpu | 150 / 180 MB/s |
| X board, 64 bit cpu | 96 / 90 MB/s |
| X board, 32 bit cpu | 28 / 28 MB/s |
| Packaging | Single w/ manual Bulk Qty 20 no manual |
| Server Management | Diagnostic registers |
|  | Error logging |
|  | Board and ASIC revision |
| Configuration | Software memory sizing |
|  | Bad SIMM remapping |
| Environmental |  |
| Temperature | 0 - 55° C air flow 200 LFM |
| Voltage | +5 VDC operation at 25 Watts maximum |
| FCC | Class B |
| Target Baseboards | Xpress - must be (PBA# ³ 614727-003), Xtended Xpress |

*  (OS drivers required - Initial support for  Novell*,  SCO* UNIX*, Windows* NT, UNIVEL*, OS/2*)

# i486Ô Processor Module, 512 KB L2

Product Code:     BXCPU486DX266-C

Internal code:      Roadrunner

Description:

Xpress CPU module for Intel's next generation i486 Microprocessor running at upto 100 MHz.  Improved L2 Cache options for i486 microprocessors

Positioning:

Highest performance CPU module in the i486 CPU family designed for compute intensive applications.  Replacement for existing modules.

Key Features:

ü	66/33 MHz Internal/External CPU speed

ü	Write-back L2 cache, Standard SRAM

ü	256KB L2 Cache upgradeable to 512KB

ü	OverDriveÔ processors support

ü	Optimized for ECC bus transfers

ü	Backward compatible to installed base

ü	Server management hooks

Key Benefits:

ü	Higher performance w/ ECC module

ü	Improved L2 cache for higher performance

Availability:

| Limited | September-94 |
| --- | --- |
| Volume | October - 94 |

Technical Specifications:

| CPU |  |
| --- | --- |
| Family | i486 CPU Class |
| Intel486Ô DX2 Int/Ext Freq MHz L1 Cache Bus data/address Voltage | 66/33 MHz 8 KB WT 32/32 bit 5 V |
| Electrical support for: 100 Mhz IntelDX4 66 MHz Intel P24D 33 MHz Intel486 DX 33 MHz Intel486 SX | 33 MHz external bus Not Verified Not Verified Not Verified |
| Cache Controller | Intel |
| Mode | Write-back |
| Cache |  |
| Size (KB) | 0, 256, 512 |
| Type | Synchronous SRAMs |
| Component | Micron MT58LC 64K18B2 15ns (12ns for 50 Mhz designs) |
| Parity | None |
| Voltage | 3.3V |
| Interface | Xpress 32 bit |
| OverDrive | 237 pin ZIF |
| Mechanical | Width - 7.2" |
|  | Height - 4.85" |
| Performance | IntelDX4 |
| SPECint92/fp92 | 52 / 25 |
| iCOMP | 735 / 815 |
| Packaging | Single |
| Server Management | Performance monitoring registers |
|  | SMM mode support Xtended Xpress only |
|  | Configuration registers |
| FCC | Class B |
| Target Baseboards | Xpress, Xtended Xpress |

# Server Management Server Monitor

Product Codes:

SERVERMONPP- Pre-production

SERMONUSBLK- NA Modem, QTY 25

SERMONEURBLK - Int. Modem, QTY 25

Internal Code:   Hobbes

Description:

Integrated hardware and software system to provide manageability for Xpress as well as standard servers.

Positioning:

Provides proactive fault detection when the server is running.  Additionally, provides emergency diagnostics and control when the server has failed.

Key Features:

ü	Back-up battery

ü	Multi-Level Alerting

ü	PCMCIA slot for modem communication when the network is down

ü	Voltage and dual temperature sensors

ü	Error logging

ü	Agent software for server

ü	Windows-based management console

ü	LANDeskÔ Manager 'Snap-in'

ü	Host console redirection

ü	Reboot / reset when system hangs

ü	Enhanced support for Xpress and Xtended Xpress platforms

Key Benefits:

ü	Proactive server management

ü	Supports industry standards

Availability:

| Limited | August-94 |
| --- | --- |
| Volume | Q4-94 |

Technical Specifications:  Hardware

| Processor | 8086 compatible (VG230) |
| --- | --- |
| Interface | 16-bit ISA bus master PCMCIA modem RS-232C Serial port (9 pin) i2C bus support |
| Battery |  |
| Type | Rechargeable NiCd |
| Full operation | 30 minutes |
| Stand-by | 4 hours |
| Recharge time | 2 hours |
| Data Storage |  |
| Non-volatile Flash | 1 MB |
| RAM | 1 MB expandable to 2 MB |
| ROM | 256 KB |
| Sensors | Temperature (1 x on card, 1 x user placeable) Voltage (+5, -5, 3.3**, +12, -12) |
| Utilities | Flash BIOS update |

Console and Agent Software

| Agent Software | Novell 3.1X, 4.X In band support IPX/SPX OS parameters monitored Disk storage Configuration Memory Network information Failure diagnosis |
| --- | --- |
| Console | International Pager support Windows interface Control parameters (thresholds, severities, destinations, history) In-band / out of band support Multiple server control Alert notification (mail, pager, script) Scheduling LANDesk companion product |
| Firmware | Console and NA Pager alerting Access from remote console Host console redirection POST code capture Security (user name + password) 3-level (monitor, control, admin) Hard Reset* Power On / Off** Battery monitor OS Watchdog timer |

* Xpress baseboard ³ PBA# 614727-003, ** Xtended Xpress baseboard

# Mid-Range Server Baseboard (Flat Design)

Product Code:   B1F90B2M16PP (board)

T1F90B2M16PP (tower)

Internal Code:   Altair

Description:

AT footprint EISA / PCI server baseboard based on the Intel Neptune chipset

Positioning:

Highly integrated baseboard with performance PCI I/O, On-board server management features, and complete network certifications  to compete in the Mid/Entry server price bands

Key Features:

ü	Single or Dual Pentium Processor

ü	OverDriveÔ ready baseboard

ü	100/66, 90/60 or 75/50 MHz performance

ü	EISA and PCI I/O

ü	Integrated I/O (Graphics, SCSI)

ü	Server management hooks

ü	Standard AT form factor

Key Benefits:

ü	Feature rich design at entry price point

ü	Server Management features

ü	Complete network certifications

Availability:

Early		September -94

Volume	November - 94

Technical Specifications:

| CPU |  |
| --- | --- |
| Family | Pentium Technology |
| Int/Ext Freq MHz L1 Cache Bus data/address Voltage OverDriveÔ Proc. | 75/50, 90/60, 100/66 MHz 8 KB code, 8 KB data WB 64/32 bit 3.3 V P54CT |
| Cache Controller | Neptune - PCMC |
| Mode | Write-back |
| Cache |  |
| Size (KB) | 256, 512 |
| Component | Synchronous or Asynchronous |
| Parity | None |
| Quantity | 1 SIMM |
| Memory |  |
| Architecture | 128 bit interleaved |
| Bus Width | 64 bit |
| Capacity | 256 MB |
| Type | Parity |
| BIOS |  |
| Type | Enhanced AMI |
| Component | 512 KB Flash, 8 KB User Block |
| Languages | Flash update for 5 languages |
| Key Features | Memory Remapping, Error Logging Flash ECU, inactivity lockout, reset disable, Boot restriction |
| Graphics |  |
| Controller | Cirrus 5430 |
| Bus | PCI |
| Memory | 512KB DRAM exp. to 1MB |
| Resolutions | 640x480, 800x600, 1024x768 |
| Colors | 256, 65 K*, 1 M* ( * 1 MB DRAM) |
| Output Ports | 15 pin analog, Header |
| Disk Controller |  |
| SCSI | 1 x Fast Adaptec 7850, Ext. conn., 8 bit |
| IDE | Intel AIP, DMA type F |
| I/O | 8 slots |
| Dedicated EISA | 5 Bus Master |
| Dedicated PCI | 2 |
| Shared PCI/EISA | 1 Bus Master |
| Other (I/O connectors or headers on simm connector) | 2 x Serial w/ FIFO 1 x Parallel (ECP, bi-directional) 1 x PS/2 Keyboard, 1 x PS/2 Mouse |
| Dimensions |  |
| Length / Width | 12" / 13" |
| Other | Plug'N Play Server Management Features; (Voltage, Temperature, Watchdog timer, Chassis intrusion, Power cycle, Power/Reset disable, FDD write protect, Video blanking) |

# Entry Server Chassis

Product Code:   TBD

Internal Code:   Columbus

Description:

Entry Server Chassis

Positioning:

Cost driven chassis design targeted for entry server price bands

Key Features:

ü	Four external bays

ü	Support through P6 generation of   processors

ü	OEM customizable bezel

Key Benefits:

ü	Office Footprint

ü	Manageability

Availability:

Early		Q3-94

Volume	Q4-94

Technical Specifications:

| CPU Family | i486 CPU, PentiumÔ Processor, Dual 100 MHz Pentium Processor*, Single P6* *DC-DC converter required |
| --- | --- |
| I/O |  |
| Slots | 8 |
| Knockouts | 1 x Video, 2 x serial, Kybd, 1 x Parallel, 1 x SCSI, Mouse |
| Bays |  |
| Internal | 4 x 3.5, 1" |
| External | 1 x 3.5" 3 x 5.25" |
| Buttons | Reset |
|  | Power On/Off |
| Indicators | 1 x Power on 1 x HDD activity |
| Dimensions |  |
| Height | 19.3" |
| Width | 8.3" (With Feet 10") |
| Depth | 17.7" |
| Power Supply |  |
| AC Voltage / Freq | 100-120/200-240, 60/50Hz User selectable |
| DC Power (max continuous) | 230 W / 300 W |
| +5V | 30 A / 30A |
| +12V | 8 A / 12A (4 A w/ DC to DC yeild 10 A 3.3) |
| -12V | .5 A / .5 A |
| -5V | .5 A / .5A |
| Noise | <45 db @ 25°C |
| Security | Rear cable lock hook |
| Management | Support for door switch |
| FCC | Class B |
| Safety | UL, CSA, TUV, NORDIC |
| Target Baseboards | Xpress, Xtended Xpress 12", Standard AT |

# EtherExpressÔ PRO/100 LAN adapter

Product code:  EISA version:  EILA8265

PCI version:    PILA8465

Description:

The first 100Mbps Fast Ethernet adapter supporting both 10 & 100 Mbps operation.

Positioning:

Allows easy migration from 10 to 100Mbps Ethernet operation.

Key Features:

ü  High performance 32-bit architecture

ü	Supports both 10 and 100Mbps operation using Ethernet protocols

ü  Supports all major network operating systems

ü	Switchless and jumperless, uses 256K Flash memory for fast, easy installation

ü	Includes diagnostic suite for on-board and on-network testing

ü	DMI-compliant, SNMP-manageable

ü	Limited lifetime warranty

Key Benefits:

ü	Allows seamless migration from 10 to 100Mbps Fast Ethernet

ü	Co-developed with SynOptics for a completely interoperable Fast Ethernet solution

ü	Easy to install, use and manage

Availability:

| Volume | October 1994 |
| --- | --- |

Technical Specifications:

High performance architecture

32-bit direct bus mastering uses scatter gather memory access, burst transfers, extended high speed FIFO interface & dynamic transmit chaining for high throughput with low CPU utilization.

Uses derivative of the Intel 82596 Ethernet controller with a companion chip to handle MAC layer interface for both 10 & 100Mbps operation

16K on-board memory for transmit and receive buffer

Seamless migration from 10 to 100Mbps Ethernet

Uses CSMA/CD protocol for both 10 and 100Mbps operation, leveraging existing Ethernet infrastructure

The board auto-senses the speed of the hub port to which it is attached, and configures itself accordingly.

Single RJ-45 connector for 10 and 100Mbps

All drivers work at 10 and 100Mbps

With a SynOptics 10/100Mbps switching hub, you can run some nodes at 100Mbps and others with older adapters at 10Mbps.

Supports all major network operating systems

Novell NetWare*

Microsoft* LAN Manager*

Microsoft Windows* for Workgroups

Microsoft Windows NT

Artisoft LANtastic*

Banyan VINES*

DEC PathWorks*

IBM* LAN Server*

Management support

SNMP-manageable with Intel’s LANDeskÔ SNMP Gateway

Supports the Desktop Management Interface (DMI) for management by DMI-compliant applications

Standards compliance

Supports IEEE 802.3 100Base-TX draft standard for 100Mbps operation with 2 pair of UTP CAT5 data grade wiring

Supports IEEE 802.2 and 802.3 10Base-T for 10Mbps operation

For more information:

US: 800-538-3373 UK: +44-793-431155 Australia: +61-2-975-3300 Singapore: +65-250-7811