# PD440FX

Pentium® II processor-based ATX Board

Time to Market with Pentium II processor performance

The PD440FX motherboard offers OEMs and system integrators the opportunity to deliver “Time to Market Solutions” featuring the performance of the Pentium II processor.  The Pentium II processor delivers the best of Intel’s next-generation processors -- the power of the Pentium Pro processor plus the capabilities of MMX™ Technology for better, smoother, and more realistic media experiences.

The motherboard was designed with flexibility in mind to meet the needs for high performance platforms.  The PD440FX features optional on-board Yamaha OPL3-SA3 audio and dual Universal Serial Bus (USB) connectors enabling  the board for today’s multimedia applications and connectivity to tomorrow’s.  The PD440FX is based on the ATX form factor, which provides easy OEM integration, convenient access for service and upgrades, and superior thermal and emission performance.

Enhanced Manageability: Lowering the Total Cost of Ownership

With the corporate desktop in mind, the PD440FX motherboard was designed with mangeability in mind.  The PD440FX integrates an on-board hardware management ASIC, enhanced Desktop Management Interface (DMI) BIOs, and LANDesk® DMI instrumentation that help reduce support costs through manageability and serviceability.   Through secure remote monitoring, these features, coupled with management software such as LANDesk Client Manager, allow the network administrator to remotely access the user’s system and perform fault prediction, recovery, software installation and configuration tasks.  This reduces the need for service calls which aids in lowering  the total cost of ownership (TCO) in the business environment. Error Checking and Correction (ECC) memory to guard against data corruption.

| Features |  | Benefits |
| --- | --- | --- |
| · Pentium® II processor | · The power of the Pentium Pro plus the capabilities of MMX technology |  |
| · Hardware Management ASIC | · Remote monitoring of system conditions for lower total cost of ownership |  |
| · Universal Serial Bus (USB) connectors | · Expand and simplify PC connectivity while enabling new peripherals |  |
| · 4 72-pin SIMM† sockets | · Flexibility for support up to 256 MB of extended data out (EDO) memory |  |
| · ATX form factor | · Easy integration and upgradeability, reduces total system costs |  |
| · 3 PCI, 2 ISA, and 1 shared slot | · Allows ample room for custom configuration of peripherals |  |
| · Error Checking & Correction (ECC) memory | · Guards against data corruption |  |

| PROCESSOR/CACHE |  |
| --- | --- |
| Processor Supported | Pentium II ® processor in S.E.C. cartridge |
|  | with 512 KB or 256KB L2 cache |
|  | 233 Mhz, 266 Mhz, and future |
|  | upgrades |
| SYSTEM MEMORY |  |
| Memory Capacity | Four 72-pin sockets for tin-lead DRAM SIMMs |
|  | 2 banks (SIMMs filled in pairs for 64 bit bus) |
| Memory Size | Supports up to 256 MB of memory |
|  | 8 MB minimum (two 4 MB SIMMs) |
|  | 256 MB maximum (four 64 MB SIMMs) |
| SIMM Sizes | 4 MB, 8 MB, 16 MB, 32 MB, 64 MB |
| Access Speed | 50ns or 60ns Extended Data Out (EDO) , |
|  | ECC support with ECC and Parity memory |
| Memory Voltage | 5 volts |
| INTEGRATED AUDIO* | Yamaha OPL3-SA3 (YMF715) |
|  | Plug and Play with 16-bit addressing |
| INTEGRATED PCI/ISA IDE Xccelerator |  |
| Dual Channel | 2 independent channels, 4 IDE devices, |
| Fast-IDE Interface | Mode 0, PIO Mode 3 and Mode 4 IDE |
|  | Hard Drive and CD-ROM support |
| USB* | 2 standard stacked USB connectors |
| INTEGRATED SUPER I/O |  |
| Controller | National 87307 ISA Plug and Play |
| Two Serial Ports | Async, RS-232C, PC16C550A compatible |
| Parallel Port | Compatible / ECP / EPP |
| IEEE 1284 standard |  |
| Floppy Controller | 1.44 MB, 2.88 MB, 3-Mode support |
| Keyboard/Mouse Cntrler | 8042A compatible |
| Real Time Clock | Century Reg.  13 minutes/year accuracy |
| APM/APC | Power On, Off, Sleep, Resume features |
| Infra Red Interface | IrDA compatible and consumer IR compliant |
| SYSTEM BIOS |  |
| BIOS Type | 2 Mb Flash EEPROM with Intel BIOS |
| Special Features | Plug and Play; IDE drive autoconfigure; |
|  | APM 1.2, DMI 2.0, ECC/Parity support; |
|  | Multilingual support |
| PCI CHIP SET | Intel 82440FX PCIset |
|  | Intel 82441FX PMC |
|  | Intel 82442FX DBX |
|  | Intel 82371SB PIIX3 |
| JUMPERS AND FRONT PANEL CONNECTORS |  |
| Connectors | Speaker, Reset, Power LEDs, HD LED, infrared, power on/off, sleep/resume |
| Jumpers | Password clear, CMOS clear, setup access, BIOS recovery, CPU/Bus frequency |
| HARDWARE MONITOR* | On-board sensors for remote monitoring of PS voltages, Fan speed, motherboard temperature and chassis intrusion |
| EXPANSION SLOTS (all full length) |  |
| Description | Three dedicated PCI slots |
|  | Two dedicated ISA slots |
|  | One shared slot for either ISA or PCI |
| MECHANICAL |  |
| Board Style | ATX standard mounting holes |
| Integrated I/O connectors include: | two serial ports, one parallel port, PS/2 style keyboard and mouse connectors, audio stack and two USB connectors |
| Board Size | 9.6” x 12.0” (24.4 cm x 30.5 cm) |
|  | ATX information available at http://www.teleport.com/~atx |
| BASEBOARD POWER REQUIREMENTS |  |
|  | +3.3V, Tolerance 5%, typical current 0.6A |
|  | +5V, Tolerance 5%, typical current 7.0A |
|  | +12V, Tolerance 5%, typical current 0.6A |
|  | -5V, Tolerance 10%, typical current 0.01A |
|  | -12V, Tolerance 10%, typical current 0.1A |
| ENVIRONMENT |  |
| Operating Temperature | 0°C to +55°C |
| Storage Temperature | -40°C to +70°C |
| REGULATIONS |  |
| Safety regulations: |  |
| U.S & Canada | UL 1950 - CSA 950-95, |
|  | U.S. & Canadian recognition component marks |
| Europe | UL Classified to IEC 950 |
| EMI/RFI regulations: |  |
| Europe | CE Mark |

This product complies with European Directive 89/336/EEC via standards EN 55 022 and EN 50 082-1 when tested in a representative chassis.

ORDERING INFORMATION

For specific configurations please contact your ION sales representative.

Information in this document is provided in connection with Intel products. No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in Intel’s Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel products are not intended for use in medical, life saving, or life sustaining applications. Intel retains the right to make changes to specifications and product descriptions at any time, without notice. This product may contain design defects or errors known as errata. Current characterized errata are available on request.

*Third-party brands and names are the property of their respective owners.

ãINTEL CORPORATION 1997		ORDER NUMBER 282981-001