Pentium Processor Power

# Premiere/PCI Low Profile

Pentiumä Processor-based PCI Desktop

in a Low Profile

The Premiere/PCI Low Profile Desktop combines the computing power of the Pentium processor with the PCI local bus in a platform designed for high system and graphics performance, plus flexibility for the future.

The Premiere/PCI Low Profile is based on the Pentium processor, and is upgradable to future generation CPUs.  The Pentium processor provides data transaction rates of greater than 132 MB per second. With a highly featured chip set, 256 KB of second level cache, accelerated local bus graphics, and up to 128 MB of system memory, Premiere/PCI Low Profile puts the performance required by today's compute- and graphics-intensive applications in a sleek chassis that easily fits on any desktop.

SP5D60LP8HF

PCI Local Bus - Performance and Compatibility for the Future

PCI provides industry-leading performance and compatibility. The 32-bit, 33 MHz pathway to the CPU offers performance unmatched by other local bus architectures.

The PCI local bus specification is a true industry standard.  Backed by over 200 leading PC, add-in card, and chip manufacturers, the standard is clearly defined to ensure complete compatibility. A PCI add-in card available today will work in any PCI-compliant system in the future – a competitive advantage that other local bus technologies can't offer.

The PCI local bus standard also provides a roadmap to the future. With provisions for 64-bit data paths and 3.3 volt CPUs, you can feel comfortable basing your solution on the PCI local bus.

Plug N' Play with PCI Auto-configuration

In addition to the security of a true standard, PCI add-in cards feature auto-configurability for easy integration. The user-friendly BIOS automatically allocates system resources for add-in cards and configures hard disks, memory, and other peripherals. No more hassles with settings, jumpers, or switches. Just plug in the card and go.

Expandability in a sleek design

The Premiere/PCI Low Profile chassis has a slim, attractive look that still offers expansion using three dedicated ISA slots and two PCI slots on a riser card. The motherboard integrates two serial ports, an Enhanced Capabilities parallel port, and the floppy, IDE, keyboard and mouse controllers.

The system also offers one internal and three external peripheral bays. All power and cooling requirements are met by the Premiere/PCI Low Profile's 145 watt power supply and fan.

Meeting Tomorrow's Needs

Premiere/PCI Low Profile Desktop

The Premiere/PCI Low Profile is an ideal desktop for your Pentium processor-based solution. The sleek form factor keeps the system small, while the Pentium processor, PCI connections, integrated ATI local bus graphics, and integrated PCI high performance IDE controller provide the horsepower needed for the applications of today and tomorrow.

The Premiere/PCI Low Profile easily delivers capabilities such as voice, full motion video, and high-speed networking - bringing users into the new era of computing.

The graphics performance is lightning-fast, achieving up to 50 million Winmarks. A PCI-connected ATI 68800AX controller forms the core of a graphics subsystem that delivers resolutions up to 1280 x 1024 x 256 colors using 2 MB of fast VRAM. A Premiere/PCI Low Profile also is available with 1 MB of VRAM for resolutions up to 1024 x 768.

Intel's High Standards

Intel platforms are sold only to OEMs and custom solution providers - allowing us to focus on their special quality, reliability, and compatibility needs.

We understand that customers often stretch the limits of PC performance by using a wide variety of peripherals, operating systems, and applications, so our products are thoroughly tested in a wide variety of environments. Furthermore, we have policies to ensure product stability.  For example, we can provide 60 days advance notice of engineering changes.

Outstanding Service and Support

IntelTechDirect provides factory direct support to help you get to market faster. The service includes monthly product updates, FaxBACK service, a technical hotline staffed with trained engineers, and a 24-hour electronic bulletin board.

Every Intel system comes with a 12-month warranty and has several options available for extended coverage and service.

Intel is committed to exceptional product support, and delivers the services you need.

Specifications

Premiere/PCI Low Profile Desktop

PROCESSOR/CACHE

Processors	Pentium microprocessor; 60 MHz

Performance Upgrade	OverDrive processor for Pentium 60 MHz

OverDrive processor for Pentium 66 MHz

Cache Option	Intel 82434LX controller

256 KB direct mapped, second level

SYSTEM MEMORY

Standard Memory	4 72-pin sockets for tin-lead DRAM SIMMs

8 MB standard with two open sockets

Burst mode read/write support, 64-bit path
		to memory, fast page DRAM devices only

Memory Size	2 MB minimum (two 1 MB SIMMs)

128 MB maximum (four 32 MB SIMMs)

SIMM Types	1 Mb, 2 Mb, 4 Mb, 8 Mb, 16 Mb, 32 Mb

(Must fill RAM in pairs.)

Access time	70 ns or faster

Architecture 	64-bit interleaved

INTEGRATED GRAPHICS

High Resolution Config.	1280 x 1024 x 256 colors

(2 MB VRAM)	1024 x 768 x 64K colors

800 x 600 x 16.7 million colors

Low Resolution	1024 x 768 x 256 colors

(1 MB VRAM)	800 x 600 x 64K colors

640 x 480 x 16.7 million colors

Compatible with	8514/A, VGA, EGA, HGC, MDA

Controller	PCI-based ATI 68800AX

Video port	15-pin connector, analog

VESA compliant	8514/A connector to off-board DAC only

INTEGRATED I/O

I/O Controller	SMC 37C665 Super I/O controller

Two serial ports	Async, RS-232C, 9-pin 16C552 compatible

Parallel port	Centronics compatible, 25-pin, bidirectional

Enhanced Compatibilities Port

Hard disk controller	Type F DMA compatible (ISA)

Floppy Controller	2.88 MB, 765A compatible

Keyboard controller	Intel 8742

Keyboard port	PS/2 compatible

Mouse port	PS/2 compatible

Real Time Clock	Dallas DS12887, 128 bytes CMOS RAM

EXPANSION SLOT

One riser slot in standard LPX location, using connector which

allows PCI/ISA signal connection or the use of existing ISA

riser cards. Three dedicated ISA slots; two dedicated to PCI.

Connector Type	120-pin EISA style

SYSTEM BIOS

BIOS Type	Flash AMIBIOS

Hard Disk driver	IDE, auto-configuring, supports IDE DMA

Special features	PCI add-in card auto-configurable

JUMPERS	Flash reload, Flash WP, password enable,

CMOS clear,  CPU speed, CMOS setup

disable,  color/mono

SYSTEM

Height	4.3" (11.7 cm)

Width	17.2" (43.7 cm)

Depth	16.2" (41.1 cm)

Weight	20 lbs (9.1 kg) without peripherals

Drive Bays

External	2 @ 5.25"; 1/2 hieght

1 @ 3.5"; 1/3 height (1.0")

Internal	1 @ 3.5"; 1/2 height (1.6")

ELECTRICAL

AC Voltage/Frequency	115v/60Hz; 230v/50Hz (switchable)

DC Power Supply	145w

+5V	18.0A maximum continuous

+12V	4.2A maximum continuous

-5V	0.3A maximum continuous

-12V	0.3A maximum continuous

ENVIRONMENT

Ambient Temperature

Operating	+10°C to +35°C

Non-Operating	-40°C to +70°C ambient

Relative Humidity

Operating	85% RH @ 35°C

Non-Operating	95% @ 35°C

Altitude

Operating	to 10,000 feet (3,048 m)

Non-Operating	to 50,000 feet (15,240 m)

Noise	40 dB (peripherals idle, at 1 meter)

Static Discharge	25 kv with no component damage

REGULATIONS

Meets or exceeds the following requirements:

Safety

U.S.	UL 1950, 1st edition

Canada	CSA C 22.2 No. 950M-89

Europe	TUV to EN 60950 with ZHI/618

Ergonomics IEC950 with IEC 380

EMI/RFI

U.S.	FCC CFR 47 Part 15, Level B

Canada	DOC CRC c, 1374 Class B

Europe	VDE 0871 Class B, CISPR-B

ORDERING INFORMATION

High Resolution Graphics/High Performance IDE

SP5D60LP8HF	Pentium/60 MHz LPX system, 8 MB RAM,

2 MB VRAM, 256 KB cache, floppy

For more information or the number of your nearest Intel sales office,

call 800-628-8686 (good in the U.S. and Canada).

Intel Corporation disclaims all warranties and liabilities for the use of this document and the information contained herein, and assumes no responsibility for any errors which may appear in this document. Intel makes no commitment to update the information contained herein, and may make changes at any time without notice. i486, i487, OverDrive, Pentium and IntelTechDirect are trademarks of Intel. AT is a trademark of IBM.

ãINTEL CORPORATION 1993		FaxBack 4259 (2/2/94, Ver 1.2)