# Premiere/PCI LX Desktop

Featuring the Pentiumä processor at

iCOMPä index 735 \ 90 MHz

µ §Pentium Processor Power

SP54D90LPLX8F

in a Low Profile

The Premiere/PCI LX combines the computing power of the Pentium processor at iCOMP index 735 \ 90 MHz with the PCI local bus in a platform designed for high system and graphics performance, plus flexibility for the future. Premiere/PCI LX is even upgradable to future generation OverDriveä processors.

| Features |  | Benefits |
| --- | --- | --- |

| · Pentium processor 735 \ 90 |  | · Fastest processor performance available at iCOMPä index 735 |
| --- | --- | --- |

| · Upgradable to future generation Pentium OverDriveä processors |  | · Protects investment by extending the useful life of the system |
| --- | --- | --- |

| · Cirrus 5434 PCI graphics controller |  | · High performance for graphics-intensive applications such as full motion video |
| --- | --- | --- |

| · Two PCI slots on riser card |  | · Tap into the power of the CPU with high performance PCI add-in cards |
| --- | --- | --- |

| · ISA Plug and Play, PCI and IDE autoconfiguration |  | · Simplified integration and configuration |
| --- | --- | --- |

| · Integrated PCI IDE |  | · Performance comparable to SCSI |
| --- | --- | --- |

With a highly featured chip set, 256 KB write back second level cache, accelerated local bus graphics, and up to 136 MB of system memory, Premiere/PCI LX offers the performance required by compute- and graphics-intensive applications in a sleek chassis that fits on any desktop.

PCI Local Bus - Performance and Compatibility for the Future

Premiere/PCI LX Desktop

PCI local bus provides industry-leading performance and compatibility. The 32-bit, 33 MHz pathway to the CPU offers performance unmatched by other local bus architectures.Backed by over 200 leading PC, add-in card, and chip manufacturers, PCI is clearly defined to ensure complete compatibility. The PCI local bus also provides a roadmap to the future. With provisions for 64-bit data paths and 3.3 volt CPUs, you can feel comfortable basing your solution on the PCI local bus.

Autoconfiguration Cuts

Support Costs

Adding expansion cards, memory and hard drives is as simple as plugging them in. Support for ISA Plug and Play and full PCI autoconfiguration helps dramatically reduce end user problems and support costs. The system also provides automatic IDE and memory sizing and configuration.

Plug and Play Today; Ready for Windows 4.0 Tomorrow

The Premiere/PCI LX's user-friendly BIOS automatically allocates system resources for add-in cards and configures hard disks, memory, and other peripherals. These features, the ISA Configuration Utility and PCI auto-configuration mean no more hassles with settings, jumpers, or switches. Just plug in the card and go.

The Premiere/PCI LX also is designed to be compatible with the next generation of Windows*. The BIOS will interact with Windows 4.0 for extended Plug and Play capability.

Expandability and Flexibility

The Premiere/PCI LX is an ideal desktop for your high-performance Pentium processor-based solution.

The Premiere/PCI LX chassis has a slim, attractive look that still offers expansion using three dedicated ISA slots and two PCI slots on a riser card. The motherboard integrates two serial ports, an EPP/ECP parallel port, and the floppy, IDE, keyboard and mouse controllers.

The system offers one internal and three external peripheral bays. All power and cooling requirements are met by the Premiere/PCI LX's 145 watt power supply and internal CPU fan.

Meeting Tomorrow's Needs

The Premiere/PCI LX has the features and horsepower to easily support capabilities such as voice, full motion video, and high-speed networking - bringing users into the new era of computing.

The core of the graphics subsystem is a PCI-connected Cirrus Logic 5434 controller and 1 MB of DRAM video memory for standard resolutions up to 1024 x 768 x 256 colors. By upgrading to 2 MB of memory, resolutions up to 1280 x 1024 x 256 colors are possible.

Overall, the graphics performance is lightning-fast, achieving up to 42 million Winmarks(Winbench 4.0).

Intel's High Standards

We understand that OEMs often stretch the limits of PC performance by using a wide variety of peripherals, operating systems, and applications, so our products are thoroughly tested in a wide variety of environments.

Furthermore, policies are in place to ensure product stability. For example, we can provide up to 60 days advance notice of engineering changes.

Outstanding Service and Support

IntelTechDirect provides factory direct support to help you get to market faster. The service includes monthly product updates, FaxBack* service, a technical hotline staffed with trained engineers, and a 24-hour electronic bulletin board.

Every Intel system comes with a 12-month warranty and has several options available for extended coverage and service.

PROCESSOR/CACHE

Premiere/PCI LX Desktop Specifications

Processors	Pentium processor

at iCOMP index 735 \ 90 MHz

Performance Upgrade	Pentium OverDrive processors

Cache	Intel 82434LX controller

256 KB write-back, second level

SYSTEM MEMORY

Standard Memory	8 MB soldered on board. Four open

72-pin sockets for tin-lead DRAM SIMMs.

Burst mode read/write support, 64-bit path
		to memory, fast page DRAM devices only

Memory Size	136 MB maximum

(8 MB plus four 32 MB SIMMs)

SIMM Types	1 Mb, 2 Mb, 4 Mb, 8 Mb, 16 Mb, 32 Mb

(Must fill RAM in pairs.)

Access time	70 ns or faster

Architecture 	64-bit non-interleaved

INTEGRATED GRAPHICS

Graphics Controller	PCI-based Cirrus Logic 5434

1 MB Video Memory	1024 x 768 x 256 colors

(Standard)	800 x 600 x 64K colors

640 x 480 x 16.7 million colors

2 MB Video Memory	1280 x 1024 x 256 colors

(User Upgrade)	1024 x 768 x 64K colors

800 x 600 x 16.7 million colors

Compatible with	8514/A, VGA, EGA, HGC, MDA

Video port	15-pin connector, analog

VESA compliant	8514/A connector to off-board DAC only

INTEGRATED I/O

I/O Controller	SMC 37C665 Super I/O controller

Two serial ports	Async, RS-232C, 9-pin NS16550 compatible

Parallel port	Centronics compatible 25-pin

bi-directional or output only

Enhanced Parallel Port (EPP)

Extended Capability Port (ECP)

Primary IDE Controller	PC Tech RZ1000 (PCI)

Secondary IDE Controller	Type F DMA compatible (ISA)

Floppy Controller	2.88 MB, 765A compatible

Keyboard controller	Intel 8242

Keyboard port	PS/2 compatible

Mouse port	PS/2 compatible

Real Time Clock	Dallas DS12887, 128 bytes CMOS RAM

EXPANSION SLOT

One riser slot in standard LPX location, using connector which

allows PCI/ISA signal connection or the use of ISA riser cards.

Two dedicated ISA slots; one PCI  dedicated; one combo ISA/PCI

Connector Type	120-pin EISA style

SYSTEM BIOS

BIOS Type	Flash AMIBIOS

Hard Disk Driver	IDE, auto-configuring, supports IDE DMA

Special Features	PCI add-in card auto-configurable

ISA Plug and Play auto-configurable

Windows 4.0 Plug and Play support

JUMPERS	Flash reload, Flash WP, password enable,

CMOS clear,  CPU speed, CPU Voltage

CMOS setup disable, color/mono

SYSTEM

Height	4.3" (11.7 cm)

Width	17.2" (43.7 cm)

Depth	16.2" (41.1 cm)

Weight	20 lbs (9.1 kg) without peripherals

Drive Bays

External	2 @ 5.25"; 1/2 hieght

1 @ 3.5"; 1/3 height (1.0")

Internal	1 @ 3.5"; 1/3 height (1.6")

ELECTRICAL

AC Voltage/Frequency	115v/60Hz; 230v/50Hz (switchable)

DC Power Supply	145w

+5V	18.0A maximum continuous

+12V	4.2A maximum continuous

-5V	0.3A maximum continuous

-12V	0.3A maximum continuous

ENVIRONMENT

Ambient Temperature

Operating	0°C to +35°C

Non-Operating	-40°C to +70°C ambient

Relative Humidity

Operating	85% RH @ 35°C

Non-Operating	95% @ 35°C

Altitude

Operating	to 10,000 feet (3,048 m)

Non-Operating	to 50,000 feet (15,240 m)

Noise	< 40 dB (peripherals idle, at 1 meter)

Static Discharge	20 kv with no component damage

REGULATIONS

Meets or exceeds the following requirements:

Safety

U.S.	UL 1950, 1st edition

Canada	CSA C 22.2 No. 950M-89

Europe	TUV to EN 60950 with ZHI/618

Ergonomics IEC950 with IEC 380

EMI/RFI

U.S.	FCC CFR 47 Part 15, Level B

Canada	DOC CRC c, 1374 Class B

Europe	VDE 0871 Class B, CISPR-B

Japan	VCCI

ORDERING INFORMATION

SP54D90LPLX8F	LPX system, Pentium processor-735\90, 8 MB

RAM, 1 MB video memory, 256 KB cache, floppy

For more information or the number of your nearest Intel sales office,

call 800-628-8686 (good in the U.S. and Canada).

Intel Corporation disclaims all warranties and liabilities for the use of this document and the information contained herein, and assumes no responsibility for any errors that may appear in this document. Intel makes no commitment to update the information contained here, and may make changes at any time without notice. There are no express or implied licenses granted hereunder to any intellectual property rights of Intel Corporation or others to design or fabricate Intel integrated circuits or integrated circuits based on the information in this document. i486, i487, OverDrive, Pentium, and iCOMP are trademarks of Intel.

*Other brands and trademarks are the property of their respective owners.

ãINTEL CORPORATION 1994		FaxBack document 4361 (Rev 1.1, 6/13/94)