Xpress Disk Array ControllerHigher I/O Performance & Data Loss Protection

µ §

High performance compute platforms, like the clock doubling i486Ô DX/2 and PentiumÔ microprocessors, require faster disk sub-systems.  To meet this increased performance need, and to satisfy the need for protection against data loss, Intel is now offering a two channel, SCSI RAID (Redundant Array of Inexpensive Disks) solution to complement the Xpress platform line of scalable servers.  Intel's EISA disk array controller, based on the Intel i960Ô microprocessor, provides a high performance hardware RAID solution supporting data striping  for performance, mirroring for data redundancy, and distributed striping with parity for the highest data availability at minimal cost.

Designed for network server, application server, and multi-user UNIX environments, Intel's disk array controller can improve data throughput, balance I/O performance, and ensure availability for critical data.  Best of all, the disk array controller has been tested and certified for SCO Unix and Novell Netware* environments to insure the quality and reliability you expect from Intel.

µ §

FAMILY OF PLATFORMS

The Xpress disk array controller is one of many accessories designed specifically for Xpress platforms.  The base platforms are available in a range of different form factors - each utilizing the same scalable baseboard technology with supports Intel486Ô to Pentium processors.  Chassis options include an expandable desktop, a mainstream deskside server (Xpress/LX), a rack mountable chassis, and a premium server tower (Xpress/MX).  Xpress servers are designed with snap-in drive bays to reduce integration time.  The Xpress/MX includes a SCSI signal and power distribution board designed to eliminate the need for long cables and increase signal reliability when used with the Xpress disk array controller.

COMPATIBILITY, SERVICE AND SUPPORT FOR OEMS

Our support services are geared to the unique needs of OEMs and custom solution providers.  IntelTechDirectÔ helps you get to market faster and keep all of your Intel platform products running smoothly. The service includes monthly product updates, FaxBack service, factory hotline, and an electronic bulletin board.

RAID Level 0,1,5 Hardware Support

Drives can be paired and mirrored providing 100% data duplication for redundancy

Striping			  Mirroring		  Parity-Striping  µ §Block striping segments data between several disks providing higher performance than with individual drives

Data is striped and distributed with parity across several disks to provide high data availability with less total disks (lower cost) than mirroring

| Feature | Function | Benefit |
| --- | --- | --- |
| Two I/O channels | Supports up to 14 drives | Highest possible performance with industry standard compatibility |
| Fast SCSI-2 | 10 MB/s data transfer | Leading edge performance |
| Intel i960 processor | 32 bit RISC processor | Highest possible performance |
| Support for standard I/O devices | Support for high performance disk drives, CD-ROM, Tape, and optical drives | Industry compatibility |
| Support for: RAID Level 0 RAID Level 1 RAID Level 5 Other combinations | Striping Mirroring Striping w/ distributed parity 0/1 and spanning | Performance Redundancy Performance/Availability Flexibility |
| Flash firmware and BIOS | Programmable BIOS | Easy to support and upgrade |
| Up to 64 MB write back cache | 4,16, and 64MB read/write cache with standard SIMMs | Highest performance at minimum cost |
| External device support | External SCSI-2 connector | Flexibility |
| Simultaneous Data transfer | Data transfer between I/O bus/controller and controller/Disk at same time | Highest performance |
| Automatic drive failure detection | Constant monitoring of disk array | Instant notification of system drive problems |
| Hot spare option | Spare drive to replace failing drive | Zero unscheduled down time |
| Automatic drive rebuild with spare | Replaced drive / spare drive will be rebuilt in the background | Rebuild time is optimized based on amount of requests to the I/O subsystem |
| Complete utilities | Drive card is easy to configure and install | Minimum time investment |

Specifications

| Board |  | Environment (System Level) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CPU | i960CA 32-bit microprocessor | Ambient Temperature: |  |  |  |  |
| Cache option | 4, 16 or 64 MB | Operating | +10oC to +35oC ambient |  |  |  |
| Standard | 4 MB | Non-Operating | -40oC to +70oC ambient |  |  |  |
| Cache support | Write-through |  |  |  |  |  |
|  | Write-back | Relative Humidity: |  |  |  |  |
|  | Cache flush | Operating | 85% @ 35oC |  |  |  |
| SCSI support | SCSI II fast , 8-bit wide | Non-Operating | 95% @ 35oC |  |  |  |
| SCSI Channels | 2 (7 devices per channel) |  |  |  |  |  |
| SCSI Controller | NCR 53C720 PT | Altitude: |  |  |  |  |
| EISA Bus Master | Supports EISA burst mode transfers of up to 33 MB/s | Operating | to 10,000 ft. (3048 m) |  |  |  |
| Onboard BIOS | Flash EEPROM (16K x 8) | Non-Operating | to 50,000 ft. (15,240 m) |  |  |  |
| Firmware | Flash EEPROM (16K x 8) | Static Discharge | 25 kV no component damage |  |  |  |
| RAID Levels | 0 - Block striping, no redundancy |  |  |  |  |  |
|  | 1 - Mirroring |  |  |  |  |  |
|  | 5 - Block striping with rotated XOR redundancy | Regulations |  |  |  |  |
|  | Striping & mirroring | Meets or exceeds the following requirements: |  |  |  |  |
| Multi-threaded | 64 simultaneous commands | EMI/RFI |  |  |  |  |
| Scatter/Gather |  | U.S. | FCC Class B |  |  |  |
| Fault Tolerance | Automatic drive failure detection |  |  |  |  |  |
|  | Hot spare |  |  |  |  |  |
|  | On-line rebuilding of replacement drive |  |  |  |  |  |
| Power Sequencing |  |  |  |  |  |  |
| Software |  | Tools |  |  |  |  |

i486,  Intel486, Pentium, IntelTechDirect and OverDrive are trademarks of Intel corporation.  Banyan is a trademark of Banyan Systems Inc., OS/2 is a trademark of IBM, Netware is a trademark of Novell, SCO is a trademark of Santa Cruz Operations, UNIX is a trademark of AT&T, Windows is a trademark of Microsoft corporation.

| OS Support | SCO Unix 3.2 version 4 | Configuration | Menu driven DOS utility |
| --- | --- | --- | --- |

Ó Intel Corporation 1993

|  | Novell Netware 3.11, 4.0 | OS Utilities | Utilities for all supported operating systems |  |
| --- | --- | --- | --- | --- |
|  | Windows NT (when available) |  |  |  |
|  | IBM OS 2.0 (H2-93) |  |  |  |

Order # 281559-001

|  | Banyan (H2-93) |  |  |
| --- | --- | --- | --- |

Order Code	XRAID4	2 Channel SCSI disk array controller with 4 MB cache

| UNITED STATES | EUROPE |
| --- | --- |
| Intel Corporation | Intel Corporation (U.K.) Ltd |
| 3065 Bowers Avenue | Pipers Way |
| Santa Clara, CA 95051 | Swindon |
| Tel : 800-548-4725 | Wiltshire SN3 1RJ |
|  | United Kingdom |
| JAPAN | Tel: +44 793 696000 |
| Intel Japan K.K. |  |
| 5-6 Tokodai, Tsukuba-shi |  |
| Ibaraki, 300-26 |  |
| Tel : +81-29847-8522 |  |