# Advanced/ZP

Pentiumä processor-based Baby-AT Board

Great Value and Performance

Advanced/ZP90

Advanced/ZP75

with New Capabilities

The Advanced/ZP Baby-AT baseboards deliver exceptional value for entry-level 75 MHz or 90 MHz Pentiumä processor-based systems.

New features - like 100 MB/second PCI Data Streaming, Mode 4 Bus Mastering IDE, and pipelined memory performance using Fast Page Mode or EDO DRAM - provide great performance and flexibility for today’s and tomorrow’s multimedia and communications applications. The Advanced/ZP ensures ease-of-use and built-in readiness for Windows*95 using the latest software and BIOS capabilities, including Plug and Play and Advanced Power Management.

This small, yet highly integrated board includes all standard I/O, 4 ISA and 3 PCI slots (one is shared), and a 3.3V regulator so you can get to market quickly with performance, flexibility, and new advanced capabilities.

|  | Features |  |  | Benefits |  |
| --- | --- | --- | --- | --- | --- |
| · 75 or 90 MHz Pentium processor |  |  | · High performance computing |  |  |
| · PCI Data Streaming |  |  | · Extra bandwidth for multitasking and Native Signal Processing-based multimedia and communications |  |  |
| · Bus Mastering IDE |  |  | · Disk performance scales with Windows 95 |  |  |
| · Completely Plug and Play |  |  | · Easy to configure and setup; ready for Windows 95 |  |  |

| · Advanced family software kit | · Native Signal Processing and multimedia support, ready-to-run showcase applications, on-line documentation and help, and more! |
| --- | --- |
| - Advanced Power Management | - Ready for Energy Star* and Windows 95 Instant On systems |
| · 3 ISA slots, 2 PCI slots, plus 1 shared | · High degree of flexibility for expansion |
| - Upgradable to a faster Pentium OverDriveä Processor | · Upgradable to meet future computing needs. Protect user’s investment |

Advanced/ZP Baseboard Specifications

PROCESSOR/CACHE

Processor	Pentium Microprocessor

at iCOMPä index 735 \ 90 MHz

Pentium microprocessor

at iCOMP index 615 \ 75 MHz

Performance Upgrade	Intel Verified upgradable for a

faster Pentium OverDrive processor

Cache Configuration	256 KB direct mapped, write back

Voltage	On board 3.3V regulator (for CPU/cache)

SYSTEM MEMORY

Memory Capacity	4 72-pin sockets for tin-lead DRAM SIMMs

2 banks (SIMMs filled in pairs for 64 bit bus)

Memory Size	8 MB minimum (two 4 MB SIMMs)

128 MB maximum (four 32 MB SIMMs)

SIMM Sizes	4 MB, 8 MB, 16 MB, 32 MB

Access Speed	70 ns Fast Page Mode or 70 ns

Extended Data Out. Standard 32 bit

SIMM module. SIMMs can be mixed by

bank; system will auto-detect and

optimize for EDO if present.

Memory Voltage	5 volts

PCI IDE

Number of Devices	2 independent channels, 4 IDE devices

Device Types	Mode 3, Mode 4 IDE hard drives

ATAPI CD-ROM supported

Extended Modes	Master Mode IDE for Windows 95

INTEGRATED SUPER I/O

Two Serial Ports	Async, RS-232C, 9-pin 16C550 compatible

Parallel Port	Centronics compatible, 25-pin, ECP, EPP

Floppy Controller	2.88 MB, 765A compatible

Infrared Port	IrDA compliant interface to IR TX/RX

RTC, Keyboard, I/O Cont.	National 82C306

SYSTEM BIOS

BIOS Type	1 Mb Flash with Intel BIOS

Special Features	PC ‘95 compliant, including Windows 95-

ready Plug and Play; APM 1.1 with

Windows 95 Instant-On; ATAPI CD-ROM,

IDE drive autoconfigure; five language

BIOS, modifiable through software update.

PCI CHIP SET 	Intel Triton PCISet

82437FX, 82438FX, 82371FB

JUMPERS AND FRONT PANEL CONNECTORS

Connectors	Reset switch, keylock, external speaker,

LEDs (power, HDD, turbo), 12V CPU fan, 		infrared, sleep

Jumpers	Flash Recovery

Switches	Password  enable, CMOS clear and setup

disable, CPU bus speed and multiplier, 			voltage regulation, ISA bus speed

MECHANICAL

Chassis 	Small Baby-AT board, standard mounting

holes and external connector placement,

including AT-style keyboard connector

Board size	8.6" x 10.0"

EXPANSION SLOTS

Description	Two dedicated PCI slots

Three dedicated ISA slots

One full-length combo slot for ISA or PCI

MOTHERBOARD MAXIMUM POWER REQUIREMENTS

+5V	Tolerance ± 5%, current 2.7 Amps

+12V	Tolerance ± 10%, current 780 mA

-5V	Tolerance ± 10%, current 120 mA

-12V	Tolerance ± 10%, current 90 mA

ENVIRONMENT

Operating Temperature	0°C to +55°C

Storage Temperature	-40°C to +70°C

Relative Humidity	92% RH @ 36°C

Airflow Requirements	100 LFM

REGULATIONS

Intended for use in systems meeting the following EMI/RFI regulations:

U.S.	FCC Class B

Canada	DOC Class B

Europe	VDE Class B, CISPR-B

Japan	VCCI Class 2 ITE

ORDERING INFORMATION

BP90AT8LCS	Pentium processor-735\90, Baby-AT board,

8 MB Fast Page RAM, 256 KB asynchronous cache

BP90AT0LCS	Pentium processor-735\90, Baby-AT board,

256 KB asynchronous cache, no memory

BP75AT8LCS	Pentium processor-615\75, Baby-AT board,

8 MB Fast Page RAM, 256 KB asynchronous cache

BP75AT0LCS	Pentium processor-615\75, Baby-AT board,

256 KB asynchronous cache, no memory

BEXXAT0LCS	Baby-AT board (supports 75 MHz or 90 MHz),

256 KB asynchronous cache, no processor or memory

(Configurations subject to change. Others may be available)

For more information or the number of your nearest Intel sales office,

call 800-628-8686 (U.S. and Canada) or +44 1793 696000 (Europe).

Intel Corporation disclaims all warranties and liabilities for the use of this document and the information contained herein, and assumes no responsibility for any errors that may appear in this document. Intel makes no commitment to update the information contained here, and may make changes at any time without notice. There are no express or implied licenses granted hereunder to any intellectual property rights of Intel Corporation or others to design or fabricate Intel integrated circuits or integrated circuits based on the information in this document.

*other product and corporate names may be trademarks or registered trademarks of other companies, and are used only for explanation and to the owners’ benefit, without intent to infringe.

ãINTEL CORPORATION 1995		ORDER NUMBER 281664-001