Technical Product Summary

Classic E-Series

Expandable Desktop

Version 1.0

March, 1993

Order Number xxxxxx-001

Intel Corporation makes no warranty for the use of its products and assumes no responsibility for any errors which may appear in this document nor does it make a commitment to update the information contained herein.

Intel Corporation retains the right to make changes to these specifications at any time, without notice.

Contact your local sales office to obtain the latest specifications before placing your order.

The following are trademarks of Intel Corporation and may only be used to identify Intel products:

376Ô	i750â	MAPNETÔ

AboveÔ	i860Ô	MatchedÔ

ActionMediaâ	i960â	MCSâ

BITBUSÔ	Intel287Ô	Media MailÔ

Code BuilderÔ	Intel386Ô	NetPortÔ

DeskWareÔ	Intel387Ô	NetSentryÔ

Digital StudioÔ	Intel486Ô	OpenNETÔ

DVIâ	Intel487Ô	OverDriveÔ

EtherExpressÔ	Intelâ	ParagonÔ

ETOXÔ	intel inside.Ô	PentiumÔ

ExCAÔ	Intellecâ	ProSolverÔ

Exchange and GoÔ	iPSCâ	READY-LANÔ

FaxBACKÔ	iRMXâ	Reference Pointâ

FlashFileÔ	iSBCâ	RMX/80Ô

Grand ChallengeÔ	iSBXÔ	RxServerÔ

iâ	iWARPÔ	SatisFAXtionâ

ICEÔ	LANDeskÔ	SnapIn 386Ô

iLBXÔ	LANPrintâ	Storage BrokerÔ

InboardÔ	LANProtectÔ	SuperTunedÔ

i287Ô	LANSelectâ	The Computer Inside.Ô

i386Ô	LANShellâ	TokenExpressÔ

i387Ô	LANSightÔ	Visual EdgeÔ

i486Ô	LANSpaceâ	WYPIWYFâ

i487Ô	LANSpoolâ	IntelTechDirectÔ

MDS is an ordering code only and is not used as a product name or trademark. MDS is a registered trademark of Mohawk Data Sciences Corporation.

CHMOS and HMOS are patented processes of Intel Corp.

Intel Corporation and Intel's FASTPATH are not affiliated with Kinetics, a division of Excelan, Inc. or its FASTPATH trademark or products.

TRADEMARK ACKNOWLEDGMENTS

AutoCAD is a trademark of Autodesk Inc.

BAPCo and Sysmark92 are trademarks of the Business Applications Performance Corporation.

Centronics is a trademark of Centronics Data Computer Corp.

IBM, MDA, MCGA, EGA, VGA, Personal System/2, PS/2, PC/XT and PC/AT are trademarks of International Business Machines.

Microsoft, MS-DOS, OS/2, Xenix and Windows are trademarks of Microsoft Corporation.

Novell is the trademark of Novell, Inc.

PolySwitch is a registered trademark of Raychem Corporation.

SCO is a trademark of Santa Cruz Operations, Inc.

SPEC, SPECratio, SPECint92 and SPECfp92 are trademarks of the Standard Performance Evaluation Corporation.

TEAC is a trademark of TEAC Corp. of Japan.

UNIX is a trademark of AT&T Bell Laboratories.

Western Digital is a trademark of Western Digital Corp.

All other trademarks are the property of their respective owners.

Classic E-Series

Preliminary Technical Product Summary

March 29, 1993

Table of Contents

µIntroduction	4

Board Level Features	5

CPU

Performance Upgrades

System Memory

Super I/O (National PC87311)

OPTi Chipset

Keyboard Controller

Real-Time Clock, CMOS RAM, Battery

Phoenix Technologies System BIOS

System Level Features	9

Peripheral Bays

Expansion Slots

Front Panel

Back Panel

Power Supply

Floppy Drive

Security

Performance Benchmarks	12

BAPCo Suite

PC Laboratories Suite, version 7.01

PC Laboratories Suite, Winbench version 3.11

Dhrystone Suite

MIPS Computation

SPECmark Suite

Appendices

A	-	User Installable Upgrades	15

B	-	Jumper Settings	17

C	-	Setup Options	18

D	-	Connectors	20

E	-	BIOS Recovery	22

F	-	Memory Map	23

G	-	System Interrupts	23

H	-	Baby AT Chassis Suppliers	23

I	-	Environmental Specifications	24

J	-	Reliability Data	24

K	-	Physical Specifications	25

L	-	Customer Support	25

M	-	Product Codes	26

# Introduction

The Intel486ä Classic E-Series opens the door to solid high performance, expandability, and upgradeability for users requiring slots and watts for their add-in devices. For OEMs and VARs adding a lot of value to the base platform, the Classic E-Series is a logical successor to systems with Intel 386 CPUs.

Standard configurations of the E-Series come with either an i486 SX/25 or an i486 DX/33 microprocessor, but the platform also provides a 238-pin socket for upgrading CPU performance with any i486 OverDriveä Processor or future OverDrive components based on Pentiumä microprocessor technology.

The Classic E-Series makes the integrator's job easy by providing seven ISA bus slots and five peripheral expansion bays. The seven ISA slots support six full-length ISA cards and one half-length ISA card. In addition, one of the slots accepts a VESA Local Bus slave add-in card. The Classic-E's 200 watt power supply and a secondary fan provide ample power and cooling for a full load of peripherals and devices.  The E-Series may be purchased as either a board or system product. The platform ships with one 4 MB SIMM installed, and supports up to 64 MB of system memory using the motherboard's four 36-bit SIMM sockets.

All standard I/O features are integrated on the motherboard. Included are the floppy drive and IDE hard disk controllers and connectors, two DB9 RS-232 serial port connectors, a bi-directional parallel port, an integrated speaker and an AT style keyboard connector.

The Classic E-Series is available as both a board and system product. The motherboard complies with the Baby-AT form factor, allowing OEMs to purchase their own off-the-shelf chassis, if desired.

## Baby At Form Factor

The motherboard is designed to the Baby AT standard, as shown in Figure 1. The standard specifies board size, board mounting locations and connector locations for the keyboard. For a list of several chassis suppliers that support the Baby AT standard, refer to Appendix G.

µ §

Figure 1. Baby AT Board Dimensions

# Board Level Features

### A	-	Offboard battery connectorB	-	AT-style keyboard connectorC	-	Serial port 2 connectorD	-	Serial port 1 connectorE	-	Power supply connectorsF	-	System memory SIMM sockets (4)G	-	Parallel port connectorH	-	Diskette Drive connectorI	-	IDE hard disk drive connectorJ	-	OPTi 82C206 integrated peripheral controllerK	-	OPTi 82C495 ISA system controllerL	-	OPTi 82C392 data buffer controllerM	-	Flash EPROM containing the BIOSN	-	Keyboard controllerO	-	Secondary cache memory data bank 0P	-	Secondary cache tag bit RAMQ	-	Secondary cache memory data bank 1R	-	Front panel connectorsS	-	Secondary cache dirty bit RAMT	-	Front fan connectorU	-	OverDrive-compatible 238-pin socketV	-	Main CPU siteW	-	Add-in VESA local bus slotX	-	Add-in board slots

## CPU

Intel486ä Classic E-Series is offered with two base CPU options: an i486ä SX CPU running at  25 MHz or an i486 DX CPU running at 33 MHz. This provides a  price/performance range to fit a variety of customer application needs.

The Classic-E supports all i486 CPU functionality. Common features of the two CPUs include backward compatibility with the 8086, 80286,  and i386ä CPUs, burst mode bus cycles, and an on-chip 8 KB cache. The cache is 4-way set associative, uses a write-through policy, and can be disabled via software. The i486 DX contains an on-chip numeric coprocessor to increase the speed of floating point operations. This coprocessor is backward code compatible with i387 DX and i387 SX math coprocessors and complies with ANSI/IEEE standard 754-1985. The i486 SX CPU does not include the numeric coprocessor.

## Performance UpgradeS

Upgrades to higher performance are easy using the Performance Enhancement Socket incorporated into the Classic E-Series platform. The socket accepts Intel OverDrive processors for significantly faster CPU performance and number-crunching capabilities. To improve floating-point performance alone, an i487 SX component can be added to systems based on the i486 SX CPU.  A detailed description of the upgrade process is included in Appendix A.

## OPTional SECOND LEVEL Cache

The Classic E-Series supports an optional secondary cache using industry-standard SRAM.  DIP sockets on the motherboard allow cache sizes of 64 KB, 128 KB, or 256 KB. The OPTi 486SXWB chipset includes a direct-mapped, write-back cache controller. Installation details and a list of SRAM vendors are included in Appendix A.

## System Memory

The Classic E-Series memory subsystem consists of four logical banks, each containing one 36-bit SIMM socket. Standard configurations ship with one 4 MB (1 Mb x 36) SIMM installed in Bank 0. The user may expand system memory to a maximum of 64 MB by adding SIMMs sequentially to Bank 0 (U5), followed by Bank 1 (U12), Bank 2 (U16), and Bank 3 (U20). Each bank supports 80 ns fast page mode SIMMs. Faster DRAM can be installed, but there will be no improvement in system performance. For a complete list of possible memory configurations, refer to Appendix A.

## National PC87311 (SuperI/O)

The serial ports, parallel port, floppy and IDE hard drive interface are incorporated into a single component, the National PC87311AVF. This component provides:

· Two NS16450 UARTs (No 16-bit FIFO support)

· IBM and Centronics compatible bi-directional parallel port

· Industry standard floppy controller (2.88 MB floppy support)

· IDE hard disk decode and chip select

## OPTi Chipset

The OPTi 486SXWB is a three-chip solution for i486 DX and i486 SX ISA-based systems.

### 82C495 System Controller

·	CPU reset control

·	CPU internal cache control

·	CPU burst mode control

·	CPU interface control

·	Integrated write-back cache controller with tag comparator

·	Page-mode DRAM controller

·	Burst line fill control logic

### 82C206 Integrated

### Peripheral Controller

·	DMA control

·	Interrupt controller

·	Real Time Clock

·	Timers

·	CMOS RAM

### 82C392 Data Buffer Controller

·	Data bus conversion

·	Parity generation/detection

·	AT-BUS direction control

·	Clock source for 82C206 and keyboard controller

·	Chip select for keyboard controller and RTC

·	Speaker control

·	NMI logic

·	Floating-point coprocessor interface

·	Keyboard reset and gate A20 emulation logic

## Keyboard Controller

An Intel 8242 surface-mount microcontroller (at location U4) contains the Phoenix Technologies PS/2 compatible keyboard controller. An AT-style keyboard connector is located on the back panel. The microcontroller supports Power-On/Reset (POR), and keyboard password protection. The POR password can be set via the SETUP program. In addition, the keyboard controller provides these "hot key" sequences:

·	CTRL-ALT-DEL: System software reset. This sequence performs a software reset of the system by jumping to the beginning of the BIOS code and running POST, excluding memory tests.

·	CTRL-ALT-1 and CTRL-ALT-2: Turbo mode selection. CTRL-ALT-1 sets the system for de-turbo mode (emulation of an 8 MHz 80286 CPU using wait states) and CTRL-ALT-2 sets the system for turbo mode (normal operation). Changing the Turbo mode may be prohibited by an operating system or application software.

·	CTRL-ALT-+ and CTRL-ALT--: Keyclick.

## Real Time Clock, Battery-backed CMOS RAM, and Battery

The real-time clock (RTC) and battery-backed CMOS RAM are contained within the OPTi 82C206.  This provides a clock accurate to 12 minutes/year, 14 bytes of CMOS RAM for time keeping, and 50 bytes of general non-volatile RAM for storage of setup information. All CMOS RAM is reserved for BIOS use. A jumper on the baseboard or a setup option can be used to set system CMOS to its default values.  For jumper configurations, see Appendix B.

Power is provided by a 4.5V Ray-O-Vac alkaline battery attached to the inside of the chassis. The expected life of the battery is 3 years. A replacement battery providing 3.6V, 4.5V or 6.0V can be installed using the connector at J17.

## Phoenix Technologies System BIOS

The E-Series uses a Phoenix Technologies BIOS to provide ISA compatibility. A 1 Mbit FLASH EEPROM device (Intel 28F001B at location U1) incorporates the BIOS and facilitates easy BIOS upgrades. Upgrading the BIOS is as simple as running a utility from the floppy drive. Upgrades will be available for download on iPAN, the electronic bulletin board service of IntelTechDirectä. Classic-E supports video and system BIOS shadowing, allowing for the execution of any BIOS routine out of fast 32-bit onboard DRAM memory instead of the slower 8-bit FLASH device.

### BIOS IDENTIFIER/REVISION LEVEL

The E-series BIOS sign-on during POST is 1.00 01 AB0.  As BIOS updates occur, the BIOS revision number will increment by one (i.e.; 1.00 01 AB0 moves to 1.00 02 AB0).

### FLASH Implementation

### tc  \l 3 "Flash Implementation"§

The Intel 28F001B 1 Mbit FLASH component is organized as 128 Kbit X 8 (128 KB). The Flash device is divided into five areas, as described in Table 2.

| System Address |  | FLASH Memory Area |
| --- | --- | --- |
| F0000H | FFFFFH | 64 KB Main BIOS |
| EE000H | EFFFFH | 8 KB Boot Block (Not FLASH erasable) |
| ED000H | EDFFFH | User available |
| EC000H | ECFFFH | 4 KB Parameter Block (BIOS Only) |
| E8000H | EBFFFH | CMOS Setup |
| E0000H | E7FFFH | User available |

Table 2. Flash Memory Organization

The FLASH device resides in system memory in two 64 KB segments starting at E0000H, and is distributed in two different organizations, depending on the mode of operation. In Normal Mode address line A16 is inverted, switching the E000H and F000H segments so that the BIOS is organized as shown in the system address column above. Recovery mode removes the inversion on address line A16, swapping the E000H and F000H segments so that the 8 KB boot block resides at FE000H where the i486 expects the bootstrap loader to exist. This mode is only necessary in the unlikely event that a BIOS upgrade procedure is interrupted, causing the BIOS area to be left in an unusable state. For information on recovering the BIOS in the event of a catastrophic failure, refer to Appendix D.

### Upgrade Utility

### tc  \l 3 "Upgrade Utility"§

The disk-based FLASH upgrade utility (FMUP.EXE; downloadable from iPAN) has three options for BIOS upgrades:

· The FLASH BIOS can be updated from a file on a disk;

· The current BIOS code can be copied from the FLASH EEPROM to a disk file as a backup in the event that an upgrade cannot be successfully completed; and

· The BIOS in the FLASH device can be compared with a disk file to ensure the system has the correct BIOS version.

### Setup UTILITY

Classic E-Series incorporates many commonly used system setup features into the FLASH EEPROM. The setup utility is accessible only during the Power-On Self Test by pressing the <F1> key. A screen prompt for this key press can be enabled or disabled. The ROM-based setup allows the system configuration to be modified without opening the system for most basic changes. Setup options are detailed in Appendix C.

# System Level Features

## Chassis

The Classic E-Series easily provides the expandability required for most traditional PC applications - five peripheral bays, seven ISA slots, an ample 200 Watt power supply and an additional fan, located at the front left side of the chassis, to help keep the system cool. The user can access all five peripheral bays from the front of the chassis. System security is provided by a mechanical keyboard lock and a BIOS password feature. Classic-E also meets stringent environmental requirements.

## Peripheral Bays

Five expansion bays are available for peripherals and other add-in devices. These include three 5¼" half-height bays and two 3½" one-inch bays. All of the drive bays are accessible from the front panel.

________________________________________________________________________

Figure 2: E-Series Front Panel

## FAN

The Classic E-Series has two fans to keep the system cool. One fan within the power supply provides 28 cfm of air movement. The second fan, located behind the card guide, provides 26 cfm across the add-in cards and the CPU. The second fan receives 12 Vdc directly from the baseboard at stake pin location J29.

## Expansion slots

The Classic E-Series has seven 16-bit ISA card slots, as shown in Figure 3. Six slots (J21, J22, J23, J24, J26 & J28) support full-length ISA cards, while a seventh slot (J20) supports a half-length ISA card. Tandem slots J27 & J28 support an ISA card, but also accept a target VESA Local Bus card.

### VESA LOCAL BUS SLOT

The VESA Local Bus slot supports any target (slave) VL card that meets the VESA Local Bus specification. The slot is a direct connection to the CPU local bus, allowing the installed VL card to operate at the processor speed instead of the slower speed of the ISA bus. I/O add-in peripheral cards designed to support the VL bus benefit from the direct connection to the processor bus.  Specifically, graphics cards, disk controllers and LAN controllers benefit significantly from the faster bus and 32-bit data path.

## Front Panel

The Classic E-Series front panel consists of a power switch, a hardware reset switch, a power-on LED,  a hard disk access LED and a keyboard lock. All five peripheral bays also are accessed from the front panel, as shown in Figure 2.

## Back panel

The back panel consists of access panels for the expansion slots (one location is unused), two DB9 RS-232 serial ports, a bi-directional parallel port, an AT-style keyboard connector, a 115/230 voltage switch, a power supply input, an auxiliary power output and the power supply fan.

µ §

________________________________________________________________________

Figure 3. Classic E-Series Back Panel.

## Power Supply

The Classic E-Series integrates a 200 watt switchable power supply for all onboard resources, add-in cards, and peripherals. The Astec Corp. model SA201-3440 supports operating settings at 100-120 VAC (5 Amps AC) or 200-240 VAC (3 Amps AC). The input voltage is selected using a switch on the back of the system.

### AC Power input SPEcifications

| Input frequency 50/60 Hz |  |
| --- | --- |
| AC Voltage | Current |
| 90-135 | 5.0 A |
| 180-265 | 3.0 A |

Table 3. Power Supply Input Specifications.

### DC output SPECIFICATIONS

| DC Voltage | Max. Continuous Current | Peak Current 15 Seconds | Minimum Current Load |
| --- | --- | --- | --- |
| +5V | 22.0A | - | 3.0A |
| -5V | 0.5A | - | 0A |
| +12V | 8.0A | 9.0 A | 0A |
| -12V | 0.5A | - | 0A |

Table 4. Power Supply DC Output Specifications.

Table 5 lists the wattage used by system resources. This information is preliminary and is provided only as a guide for calculating approximate total system power usage with additional resources added.

| Resource | Typical Current | Maximum Current |
| --- | --- | --- |
| E-Series baseboard, i486 DX-33 CPU, 4MB, 256 K cache | 3255.26 mA | 4139.33 mA |

Table 5. Current Use by System Resources (Calculated)

| Resource | Measured Current |
| --- | --- |
| +5 V | 2.19 A |
| +12 V | 0.24 A |
| -12 V | 0.06 A |
| -5 V | 0.01 A |

Table 6. Current Use by System Resources (Measured)

Measurements in Table 6 were obtained using a  Classic E-Series motherboard with 4 MB of memory, an i486 DX-33 CPU, no secondary cache, a TEAC 3½" floppy drive and the ASTEC 200w supply.

## Floppy Drive

Classic E-Series integrates a 3½" Teac Floppy drive Model FD-235HF into the right 3½" bay. This is the same proven floppy disk drive that has been used on many previous Intel systems.

## Speaker

The standard system ships with an external speaker installed. The user may enable/disable the device using the Setup utility or program the speaker via port 61H. The speaker provides error beep code information during POST if the system cannot use the video interface. The E-Series product guide (Order # 469281-001) contains detailed beep and error code information.

## Security

### SYSTEM Keylock

When engaged, the system keylock  prevents keyboard input. Each system is provided with a set of two identical keys.

### BIOS Password

A BIOS password feature provides security during the boot process. A password can be entered using the Setup utility and must be re-entered prior to disk boot each time the system is reset. The password can be changed by entering <old password>/<new password>/<new password><enter>. If the password is forgotten, it can be cleared by turning off the system and setting jumper J7 to 1-2. After the system has finished the Power-On Self Test, turn the system off and reset the jumper to 2-3. This allows the user to again access the password feature, but with the forgotten password cleared.

## Chassis color

The chassis color is beige.  The bottom and back of the chassis are not painted.

# Performance Benchmarks

## BAPCo Suite

The BAPCo tests were developed in 1992 by a consortium of more than 14 companies (Business Applications Performance Corporation). The SYSmark92 suite is a set of application scripts, data files  MS-DOS batch files and executable programs designed to measure overall system performance by realistically using all the computer subsystems. This collection of program scripts is based solely on actual user patterns for a large variety of DOS and Windows applications. The BAPCo scripts are written for use under MS-DOS 5.0 and Windows 3.0.  The six areas in the BAPCo suite:

| Word Processing | Database | Spreadsheet | Desktop Graphics | Desktop Publishing | S/W Development |
| --- | --- | --- | --- | --- | --- |
| Word 2.0 for Windows | dBase IV v1.5 | Lotus 1-2-3 v 3.1+ | Harvard Graphics v3.0 | PageMaker 4.0 for Windows | Microsoft C v6.0 |
| WordPerfect v5.1 | Paradox v3.5 | Excel 3.0 for Windows |  |  | Borland C++ v2.0 |
|  |  | QuattroPro v3.0 |  |  |  |

While the SYSmark92 result is based on popular applications, there are very few Windows or 32-bit applications. The BAPCo test suite is available to anyone, yet it is somewhat difficult to use (over 21 3½" diskettes) and takes up to 10 hours to test a single system configuration. Future enhancements include a test suite based on 32-bit windowing applications with graphical floating point and CPU intensive programs. A standalone 16-bit and 32-bit Networking Benchmark Test also is planned.

| System | Word Proc. | Spreadsheet | Database | Graphics | DTP | S/W Devel. | SysMARK 92 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DX/33 w/256 KB | 110.23 | 111.80 | 113.15 | 94.68 | 119.95 | 105.17 | 108.91 |
| SX/25 w/256 KB | 94.49 | 98.64 | 95.16 | 81.53 | 98.42 | 94.08 | 91.73 |
| DX/33, no cache | 90.21 | 97.29 | 88.80 | 89.82 | 103.17 | 93.63 | 92.99 |
| SX/25, no cache | 71.32 | 74.83 | 70.42 | 73.32 | 83.81 | 81.19 | 74.03 |

Note: Specific configuration information (AUTOEXEC.BAT and CONFIG.SYS files) for the results are available on request.

## PC Laboratories Suite, Version 7.01

Developed by PC Magazine ZD Labs in late 1992, the version 7.0 suite is considered to be one of the most widely used utilities to measure system performance. PC Magazine consulted CPU component vendors to analyze DOS, Windows and OS/2 16-bit applications to determine the frequency of the instructions executed by the i486 CPU. The ZD Labs utilized new software tools to bring the level of source code and compilers up to date when compared to version 6.0. In March 1993, ZD Labs released the revised version 7.01 and WinBench 3.11 test suites. Specific alterations include memory test script computation formulas, DosMark computation formulas and developments in compilier techniques.

Processor Tests in Release 7.01 have been recompiled to test both real mode and protected mode instructions, focusing on the instruction mix test scripts. The synthetic application scripts are broken into a small instruction mix (1 KB) and a medium instruction mix (48 KB). The small instruction mix will easily fit into the CPU's 8 KB internal cache, while the medium instruction mix utilizes the internal cache and any second level cache. There is no 32-bit instruction mix in this release, but ZD Labs feels confident it will complete a profile of 32-bit application compilation and synthetic 32-bit instruction mix test scripts in a future benchmark (sometime after Windows NT is released).

The Memory Test includes five scripts to determine raw memory transfer speed, but only the extended and conventional memory tests are factored when comparing to other computer systems. An average of all extended and conventional memory tests determines the performance result. The extended memory test performs sequential read/write operations in 8-bit, 16-bit and 32-bit blocks in protected mode into every memory location. The conventional memory test performs sequential read/write operations in 8-bit and 16-bit blocks in real mode to every memory location below 640 KB (A0000H).

The Video Tests in Release 7.01 are identical to Release 6.0.

| PC Labs Suite ver 7.01 | i486DX2/66 w/o cache | i486DX2/66w/ 256 kB | i486DX/33 w/o cache | i486DX/33 w/ 64 kB | i486DX/33 w/ 128 KB | i486DX/33 w/ 256 KB |
| --- | --- | --- | --- | --- | --- | --- |
| Processor Harmonic | 8474.01 | 14713.56 | 6753.85 | 8932.40 | 8929.59 | 8932.40 |
| Video Harmonic | 9886.67 | 10646.26 | 9862.48 | 10343.22 | 10558.58 | 10597.60 |
| Memory Harmonic | 6206.84 | 5933.79 | 5077.32 | 4640.17 | 4722.29 | 4727.18 |
| Disk Harmonic | 36.96 | 33.40 | 36.03 | 34.50 | 33.80 | 32.71 |
| DOSmark Rating | 33.57 | 35.35 | 29.98 | 31.13 | 30.87 | 30.31 |

| PC Labs Suite ver 7.01 | i486SX/25 w/o cache | i486SX/25 w/ 64 kB | i486SX/25 w/ 128 kB | i486SX/25 w/ 256 KB |
| --- | --- | --- | --- | --- |
| Processor Harmonic | 5169.66 | 6905.39 | 6763.67 | 6919.58 |
| Video Harmonic | 7679.91 | 8098.52 | 8289.68 | 8467.88 |
| Memory Harmonic | 3908.08 | 3572.65 | 3565.83 | 3674.27 |
| Disk Harmonic | 34.53 | 33.96 | 31.76 | 30.55 |
| DOSmark Rating | 25.65 | 27.34 | 26.26 | 26.06 |

Each system tested with 8MB system memory, 1MB video DRAM and the WD 2340 hard drive. Tests were conducted by Intel Corp.

## PC Laboratories Suite,  WINBENCH Version 3.11

The Disk WinMark is based on testing techniques adapted from PC Magazine's DOS disk test to reflect operation within the Windows 3.1 environment. Each system contained 2 MB of SmartDrive cache. This test determines how effectively the disk processes data files of varying sizes up to 32 MB.  This will force disk cache misses. The test simulates large database accesses, large graphic files, and reading/writing small files typically used in word processors and electronic mail. Multi-user, multi-tasking and networking disk access patterns also are simulated.

| WINBENCH Version 3.11 | i486DX2/66 w/o cache | i486DX2/66w/ 256 kB | i486DX/33 w/o cache | i486DX/33 w/ 256 kB | i486SX/25 w/o cache | i486SX/25 w/ 256 kB |
| --- | --- | --- | --- | --- | --- | --- |
| 640 x 480 x 16 Graphics WINmark | 13.274 | 15.806 | 10.166 | 11.096 | 8.011 | 8.987 |
| 640 x 480 x 256 Graphics WINmark | 13.015 | 15.407 | 10.208 | 11.000 | 7.958 | 8.939 |
| 800 x 600 x 256 Graphics WINmark | 13.027 | 15.144 | 10.239 | 11.307 | 8.108 | 9.147 |
| 1024 x 768 x 256 Graphics WINmark | 13.046 | 15.243 | 10.151 | 11.331 | 8.047 | 9.080 |
| 1280 x 1024 x 16 Graphics WINmark | 13.442 | 15.401 | 10.156 | 11.243 | 8.130 | 9.121 |

Each system tested with 8MB system memory, 2MB video VRAM on an ATi Ultra Pro VL, and the WD 2340 hard drive.

Tests conducted by Intel Corp.

## Dhrystone Suite

Dhrystone versions 1.1 and 2.1 are measurements of processor speed when executing a 'typical' program.  The typical program was designed by taking a mix of C instructions and measuring statistics on a large number of real applications. The typical program was written using these statistics. In 1988, after an evaluation of the Dhrystone in achieving its original goals, version 2.1 was released. Although expected to be close, Dhrystone numbers for version 2.1 are lower than those in version 1.1. Version 2.1 is intended to provided a more realistic benchmark by defeating some compiler optimizations that were becoming common, but not representative of those applied to real programs. The benchmark allows the use of internal registers (r) or no registers (nr) when executing and is dependent on the 32-bit compiler used. Dhrystone tests only fixed-point calculations and is weighted towards the CPU/Cache/Memory subsystem and serves as a compiler optimization intensive benchmark which tests virtually no I/O transfers across the EISA/ISA bus. The benchmark is a small program which easily fits into a 64 KB cache. While the Dhrystone suite is a good indication of performance for a single processor system, it is a somewhat poor indication of overall throughput. The VAX MIPS number is a derivative of the Dhrystone version 1.1 benchmark, obtained by dividing by 1757 (the Dhrystone result on a VAX 11/780).

| SCO ODT 2.0 Intel C v5.1.1 beta | Dhry1.1nr4 | Dhry1.1r4 | Dhry2.1nr4 | Dhry2.1r4 |
| --- | --- | --- | --- | --- |
| DX/33 / 256 kB | 56,179 | 56,179 | 39,370.1 | 39,215.7 |
| SX/25 / 256 kB | 42,194 | 42,194 | 30,395.1 | 30,395.1 |

Each system tested with 16 MB (80ns) system memory, Adpatec 1542B SCSI controller and Maxtor 535S MB Hard Drive.

## MIPS Computation

The VAX MIPS (million of instructions per second) is a measure used in conjunction with the Dhrystone benchmark. Different packages contain MIPS rating benchmarks which can give very contrary results. The MIPS rating depends on the CPU frequency, instruction mix executed, system response and compiler. To calculate the MIPS rating, vendors compile and execute the Dhrystone program to determine how many times it can be executed in one second. The result is divided by 1757 (the rating for a VAX 11/780 executing the same Dhrystone program). The same method can be used to determine a MIPS rating using Dhrystone Version 2.1 by dividing by 1657.

| SCO ODT 2.0 Intel C V5.1.1 (beta) | DX/33 w/ 256 KB cache | SX/25 w/ 256KB cache |
| --- | --- | --- |
| MIPS w/ Dhry 1.1 | 31.97 | 24.01 |
| MIPS w/ Dhry 2.1 | 23.75 | 18.34 |

Each system tested with 16 MB (80ns) system memory, Adpatec 1542 SCSI controller and Maxtor 535S MB Hard Drive.

SCO ODT 2.0 is based on X-Windows Version 11 Release 4.0 and Open Software Foundation's Motif Release 1.1

## SPECmark Suite

SPECmark was developed by the Standard Performance Evaluation Corporation in 1989 in a cooperative effort by various companies to introduce a benchmark involving UNIX applications. The ten SPEC 1.2b programs perform CPU intensive, fixed integer arithmetic, floating point arithmetic and some disk I/O tests in a technical environment. SPEC publishes a comprehensive comparison of tested computer systems to document the configuration, compiler, operating system, and benchmark results. The suite executes each test twice for accuracy on a 32-bit tested system. Many times the SPECmark tests are divided into three separate areas to distinguish between CPU-intensive system performance (SPECint), floating point performance (SPECfp), and overall system performance (SPECmark). While the SPECmark89 suite established itself as the standard for UNIX testing, in some cases it is not the best predictor for commercial applications in a business environment.

Recently, SPEC released a new suite of 32-bit UNIX benchmarks to replace the SPEC 1.2b tests. The Cint92 suite is a collection of six compute-intensive integer benchmarks intended to stress the CPU/memory subsystem, cache subsystem, and software compiler. The result is the geometric mean of the six integer benchmarks called SPECint92.

| SCO ODT 2.0 Intel 'C' v5.1.1 (beta) | espresso | li | eqntott | compress | sc | gcc | SPECint92 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DX/33 w/256 cache | 15.4 | 22.2 | 17 | 10.2 | 25.8 | 16.8 | 17.1 |
| SX/25 w/256 cache | 12 | 16.8 | 12.9 | 8.1 | 5.7 | 12.9 | 10.8 |

# Each system tested with 16 MB (80ns) system memory, an Adpatec 1542B SCSI controller and a Maxtor 535S MB Hard Drive.

# SCO ODT 2.0 is based on X-Windows version 11 Release 4.0 and the Open Software Foundation's Motif Release 1.1

# Appendix A - User-Installable Upgrades

## Performance Upgrades

### SX-25

The Classic E-Series i486 SX/25 model incorporates a two-site CPU design, providing for a variety of upgrade possibilities. The first socket site (U59) contains the i486 SX/25 PQFP CPU soldered to the baseboard. The second socket site (U58) is the OverDrive Processor Socket, and will accept an OverDrive processor or an i487 SX math coprocessor. Refer to Table A-1 below when installing any upgrade processor or validating existing processor jumper positions.

### DX-33

The Classic E-Series i486 DX/33 does not use the PQFP CPU site (U59).  Instead, the i486DX CPU is  installed in the 238-pin upgrade socket (U58). Upgrading the i486 DX model requires removing the main CPU from the low insertion force socket and replacing it with an upgrade processor. Table A-1 lists the variety of CPU upgrade options for the Classic-E.

| Upgrade Processor... | ...should be in CPU Socket... | ...and jumper settings should be... |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | J5 | J6 | J32 | J33 |
| i486SX-25 | (U59) PQFP | 1-2 | 2-3 | Don't Care | 1-3, 2-4 |
| i486SX-25 | U58 | 1-2 | 2-3 | 3-4 | 4-6 |
| i486DX-33 | U58 | 2-3 | 2-3 | 1-2, 5-6 | 3-5, 4-6 |
| i486DX2-66 | U58 | 2-3 | 2-3 | 1-2, 5-6 | 3-5, 4-6 |
| ODP486SX-20 | U58 | 2-3 | 1-2 | 1-2, 5-6 | 1-3, 2-4 |
| ODP486SX-25 | U58 | 1-2 | 2-3 | 1-2, 5-6 | 1-3, 2-4 |
| ODP486DX-33 | U58 | 2-3 | 2-3 | 1-2, 5-6 | 1-3, 2-4 |
| ODPR486DX-25 | U58 | 1-2 | 2-3 | 1-2, 5-6 | 3-5, 4-6 |
| ODPR486DX-33 | U58 | 2-3 | 2-3 | 1-2, 5-6 | 3-5, 4-6 |
| i487SX-25 | U58 | 1-2 | 2-3 | 1-2, 5-6 | 1-3, 2-4 |

Table A-1. Classic E-Series CPU Upgrade Options.

## OverDrive PROCESSORS

Table A-2 lists the OverDrive Processors supported by the Classic E-Series. Contact Intel's Personal Computer Enhancement Division (800-538-3373) for more information.

| Product Code | Pins | External Clock Speed | Internal CPU Speed |
| --- | --- | --- | --- |
| ODP486SX-20 | 169 | 20 MHz | 40 MHz |
| ODP486SX-25 | 169 | 25 MHz | 50 MHz |
| ODP486SX-33 | 169 | 33 MHz | 66 MHz |
| ODP486DX-33 | 169 | 33 MHz | 66 MHz |
| ODPR486DX-25 | 168 | 25 MHz | 50 MHz |
| ODPR486DX-33 | 168 | 33 MHz | 66 MHz |
| Future OverDrive | 238 | Future OverDrive based on Pentium microprocessor technology |  |

Table A-2. Intel OverDrive Processors supported by Classic E-Series.

## SYSTEM MEMORY

Classic-E supports only single-sided page mode SIMMs in these sizes: 256 Kb x 36, 1 Mb x 36 and 4 Mb x 36. All SIMMs are 80 ns or faster. Table A-3 shows all possible memory combinations. Boldface rows indicate configurations using the 4 MB SIMM supplied with each system.

| Bank 0 SIMM Type | Bank 1 SIMM Type | Bank 2 SIMM Type | Bank 3 SIMM Type | Total Memory |
| --- | --- | --- | --- | --- |
| 256 Kb X 36 | Empty | Empty | Empty | 1 MB |
| 256 Kb X 36 | 256 Kb X 36 | Empty | Empty | 2 MB |
| 256 Kb X 36 | 256 Kb X 36 | 256 Kb X 36 | Empty | 3 MB |
| 256 Kb X 36 | 256 Kb X 36 | 256 Kb X 36 | 256 Kb X 36 | 4 MB |
| 1 Mb X 36 | Empty | Empty | Empty | 4 MB |
| 1 Mb X 36 | 1 Mb X 36 | Empty | Empty | 8 MB |
| 1 Mb X 36 | 1 Mb X 36 | 1 Mb X 36 | Empty | 12 MB |
| 1 Mb X 36 | 1 Mb X 36 | 1 Mb X 36 | 1 Mb X 36 | 16 MB |
| 1 Mb X 36 | 256 Kb X 36 | Empty | Empty | 5 MB |
| 1 Mb X 36 | 256 Kb X 36 | 256 Kb X 36 | Empty | 6 MB |
| 1 Mb X 36 | 256 Kb X 36 | 256 Kb X 36 | 256 Kb X 36 | 7 MB |
| 1 Mb X 36 | 4 Mb X 36 | Empty | Empty | 20 MB |
| 1 Mb X 36 | 4 Mb X 36 | 1 Mb X 36 | Empty | 24 MB |
| 1 Mb X 36 | 4 Mb X 36 | 256 Kb X 36 | Empty | 21 MB |
| 1 Mb X 36 | 4 Mb X 36 | 256 Kb X 36 | 256 Kb X 36 | 22 MB |
| 1 Mb X 36 | 4 Mb X 36 | 1 Mb X 36 | 1 Mb X 36 | 28 MB |
| 1 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | Empty | 36 MB |
| 1 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | 256 Kb X 36 | 37 MB |
| 1 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | 1 Mb X 36 | 40 MB |
| 1 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | 52 MB |
| 4 Mb X 36 | Empty | Empty | Empty | 16 MB |
| 4 Mb X 36 | 4 Mb X 36 | Empty | Empty | 32 MB |
| 4 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | Empty | 48 MB |
| 4 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | 4 Mb X 36 | 64 MB |

Table A-3. Possible SIMM Memory Configurations.

## Cache SRAM

The Classic E-Series can be upgraded with a second level cache by adding industry-standard SRAM components to DIP sockets on the baseboard. The design requires 20 ns data SRAM and 15 ns tag and dirty bit SRAM. Appendix B lists required jumper settings.

|  | SRAM Size |  |  |  |
| --- | --- | --- | --- | --- |
| Cache Size | Bank 0 U42-U45 | Bank 1 U54-U57 | Tag U50 | Dirty Bit U62 |
| 64 KB | (4) 8Kb x 8 | (4) 8Kb x 8 | (1) 8Kb x 8 | (1) 64Kb x 1 |
| 128 KB | (4) 32Kb x 8 | none | (1) 32Kb x 8 | (1) 64Kb x 1 |
| 256 KB | (4) 32Kb x 8 | (4) 32Kb x 8 | (1) 32Kb x 8 | (1) 64Kb x 1 |

Table A-4. Possible Second Level Cache Combinations.

Table A-5 lists the part numbers for cache components from several vendors.

| Device Type | Motorola | Cypress | IDT | Samsung | Mitsubishi |
| --- | --- | --- | --- | --- | --- |
| 8 Kb x 8, 15 ns | MEM6264PC15 | CY7C185-15PC | 7164S15TP | KM68685BP-15 | N/A |
| 8 Kb x 8, 20 ns | MEM6264PC20 | CY7C185-20PC | 7164S20TP | KM68685BP-20 | N/A |
| 32 Kb x 8, 15 ns | MEM6206PC15 | CY7C199-15PC | 71256S15TP | KM68257BP-20 | N/A |
| 32 Kb x 8, 20 ns | MEM6206PC15 | CY7C199-20PC | 71256S20TP | KM68257BP-20 | N/A |
| 64 Kb x 1, 15 ns | MEM6287P15 | CT7C187A-15PC | 7187S15P | N/A | M5M5187BP-15 |

Table A-5. Sampling of Cache Component Vendors.

# Appendix B - Jumpers (* denotes default setting)

## FLASH BOOT BLOCK (J3)

1-2	Boot from Boot Block (recovery mode)		2-3*	Boot from standard BIOS

## SUPER I/O PC87311AVF (J4)

1-2*	Enable						2-3	Disable

## CPU SPEED SELECT (J5, J6)

|  | J5 | J6 |
| --- | --- | --- |
| 16 MHz | 1-2 | 1-2 |
| 20 MHz | 2-3 | 1-2 |
| 25 MHz | 1-2 | 2-3 |
| 33 MHz | 2-3 | 2-3 |

Table B-1. CPU Speed Select Jumper Settings.

## PASSword jumper (J7)

1-2	Clear Password					2-3*	Enable Password function

## PArallel port direction via hardware (j9)

1-2*	Parallel port is output				2-3	Parallel port is input

## PArallel port direction hardware/SOFTWARE (J11)

1-2	Software select for redirection			2-3*	Hardware select for redirection

## BAttery type select (j17)

1-2	6 volt						2-3*	3.6 volt or 4.5 volt

## FAN Power (J29)

1-2	No connect					2-3	Connect secondary 12V DC fan*

## FLASH WRITE (J30)

1-2*	Enable +12V to FLASH (erasable)		2-3	Disable +12V to FLASH

## LOCAL BUS READY MODE (J31)

1-2*	Delayed Local Bus Ready			2-3	Direct Local Bus Ready

## UPGRADE SOCKET CPU TYPE (J32, J33)

| Upgrade Socket (U58) CPU Type | J32 |  | J33 |  |
| --- | --- | --- | --- | --- |
| i486DX | 1-2 | 5-6 | 3-5 | 4-6 |
| i487SX | 1-2 | 5-6 | 1-3 | 2-4 |
| i486DX2 | 1-2 | 5-6 | 3-5 | 4-6 |
| OverDrive processor (ODP) | 1-2 | 5-6 | 1-3 | 2-4 |
| i486SX | 3-4 |  | 4-6 |  |
| None | Don't Care |  | 1-3 | 2-4 |

Table B-2. Upgrade Socket CPU Type Jumper Settings.

## CACHE SIZE (J34)

| Cache Size | Jumpers |  |  | SRAM Required |
| --- | --- | --- | --- | --- |
| 0 KB | Don't Care |  |  | None |
| 64 KB | Out |  |  | 8K x 8 (both banks) |
| 128 KB | 1-2 | 5-6 |  | 32K x 8 (bank 0) |
| 256 KB | 1-2 | 3-4 | 7-8 | 32K x 8 (both banks) |

Table B-3. Cache Size Jumper Selections.

## CMOS CLEAR (J25)

|  | 1 | 3 | 5 |  | 7 | 9 | 11 | 13 | 15 | 17* |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 | 4 | 6 |  | 8 | 10 | 12 | 14 | 16 | 18* |  |
| No jumper |  |  |  | Clear CMOS on boot-up |  |  |  |  |  |  |  |
| 17-18* |  |  |  | Use saved CMOS settings on boot-up |  |  |  |  |  |  |  |

* Note: The CMOS clear jumper shares the same connector header with the speaker, reset, hard drive LED and power LED circuitry. The J25:17-18 connection is located on the side closest to the cache SRAM sockets.

# Appendix C - Setup Options

## Setup Page 1

| Choice | Default Settings | Comments |
| --- | --- | --- |
| System Time | Current time | Can also change from DOS |
| System Date | Current date | Can also change from DOS |
| Onboard Floppy | Enabled |  |
| Diskette A | 3 1/2-inch, 1.44 MB |  |
| Diskette B | Not Installed |  |
| Onboard IDE | Disabled |  |
| Hard Disk 1 | Not Installed |  |
| Hard Disk 2 | Not Installed |  |
| User Definable Drives | 48 and 49 |  |
| Boot Device | Floppy or Hard Drive |  |
| Post Setup Prompt | Enabled |  |

## Setup Page 2

| Video Card | VGA/EGA | Display mode |
| --- | --- | --- |
| Primary Video Display | Color |  |
| Keyboard | Not Installed |  |
| Numlock On at Boot | No |  |
| Password | Not Installed |  |

## Setup Page 3

| Base Memory | Set by system | Display only; cannot be changed. |
| --- | --- | --- |
| Extended Memory | Set by system | Display only; cannot be changed. |
| Base Memory Above 512K | Enabled |  |
| Speaker | Enabled |  |
| Parallel Port | Base Address 378H: Compatible |  |
| Serial Port 1 | Enabled |  |
| Serial Port 2 | Enabled |  |
| Console Redirection/Serial Port 1 | Disabled |  |
| Console Redir/ Serial Port 2 | Disabled |  |

## Setup Page 4

| CPU Speed | Fast |  |
| --- | --- | --- |
| Cache | Enabled | Optimum performance |
| Shadow C0000H to C3FFFH | Disabled |  |
| Shadow C4000H to C7FFFH | Disabled |  |
| Shadow C8000H to CBFFFH | Disabled |  |
| Shadow CC000H to CFFFFH | Disabled |  |
| Shadow D0000H to D3FFFH | Disabled |  |
| Shadow D4000H to D7FFFH | Disabled |  |
| Shadow D8000H to DBFFFH | Disabled |  |
| Shadow DC000H to DFFFFH | Disabled |  |

# Appendix D - Connectors

## Serial ports COM1 & COM2 (J16 & J18)

| J16 & J18 | DB9 Pin | Name | Description |
| --- | --- | --- | --- |
| 1 | 1 | DCD | Data Carrier Detected |
| 3 | 2 | RXD | Receive Data |
| 5 | 3 | TXD | Transmit Data |
| 7 | 4 | DTR | Data Terminal Ready |
| 9 | 5 | GND | Ground |
| 2 | 6 | DSR | Data Set Ready |
| 4 | 7 | RTS | Request to Send |
| 6 | 8 | CTS | Clear to Send |
| 8 | 9 | RIA | Ring Indication Active |

Table C-1. Serial Ports Pin-Out

## parallel port (j19)

| J19 Pin | DB25 Pin | Name | J19 Pin | DB25 Pin | Name |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | STROBE* | 2 | 14 | AUTOFD* |
| 3 | 2 | D0 | 4 | 15 | ERROR* |
| 5 | 3 | D1 | 6 | 16 | INIT* |
| 7 | 4 | D2 | 8 | 17 | SLCTIN* |
| 9 | 5 | D3 | 10 | 18 | GND |
| 11 | 6 | D4 | 12 | 19 | GND |
| 13 | 7 | D5 | 14 | 20 | GND |
| 15 | 8 | D6 | 16 | 21 | GND |
| 17 | 9 | D7 | 18 | 22 | GND |
| 19 | 10 | ACK* | 20 | 23 | GND |
| 21 | 11 | BUSY | 22 | 24 | GND |
| 23 | 12 | PE | 24 | 25 | GND |
| 25 | 13 | SLCT |  |  |  |

Table C-2. Parallel Port Connector Pin-Out

tc "Table 0. Parallel Port Connector Pin-Out" \f t§

## floppy connector (J13)

| Pin | Name | Function |
| --- | --- | --- |
| 1 | (NC) | Docking drive only, usually GND |
| 2 | HD_In | High Density In |
| 3 | (NC) | Docking drive only, usually GND |
| 4 | (NC) | Docking drive only, usually GND |
| 5 | (Keyed) |  |
| 6 | ED_In | Extended Density In |
| 7 | +5V | Docking drive only, usually GND |
| 8 | INDEX* | L = Beginning of track |
| 9 | +5V | Docking drive only, usually GND |
| 10 | MOTENA* | Motor A select |
| 11 | +5V | Docking drive only, usually GND |
| 12 | DRVSELB* | Drive B select |
| 13 | GND |  |
| 14 | DRVSELA* | Drive A Select |
| 15 | GND |  |
| 16 | MOTENB* | Motor B Select |
| 17 | GND |  |

| Pin | Name | Function |
| --- | --- | --- |
| 18 | DIR* | Head Move to Center |
| 19 | GND |  |
| 20 | STEP* | Step (Supplies step pulses) |
| 21 | GND |  |
| 22 | WRDATA* | Write Data |
| 23 | GND |  |
| 24 | FLPYWE* | Enable Head to Write |
| 25 | GND |  |
| 26 | TRACK0* | Indicates Head on Track 0 |
| 27 | GND |  |
| 28 | WP* | Write Protected |
| 29 | ED_Out | Extended Density Out |
| 30 | RDDATA* | Read Data |
| 31 | GND |  |
| 32 | HDSEL* | Head Select side 1 |
| 33 | HD_Out | High Density Out |
| 34 | DSKCHNG* | Disk Changed |

Table C-3. Floppy Drive Connector Pin-Out

## IDE connector (J14)

| 1 | IDERST | Reset signal from CPU |
| --- | --- | --- |
| 2 | GND | Ground |
| 3 | ID7 | Data bit 7 |
| 4 | ID8 | Data bit 8 |
| 5 | ID6 | Data bit 6 |
| 6 | ID9 | Data bit 9 |
| 7 | ID5 | Data bit 5 |
| 8 | ID10 | Data bit 10 |
| 9 | ID4 | Data bit 4 |
| 10 | ID11 | Data bit 11 |
| 11 | ID3 | Data bit 3 |
| 12 | ID12 | Data bit 12 |
| 13 | ID2 | Data bit 2 |
| 14 | ID13 | Data bit 13 |
| 15 | ID1 | Data bit 1 |
| 16 | ID14 | Data bit 14 |
| 17 | ID0 | Data bit 0 |
| 18 | ID15 | Data bit 15 |
| 19 | GND | Ground |
| 20 | KEY | No connection |

| 21 | IDEDRQ - CHRDY | I/O Channel Ready |
| --- | --- | --- |
| 22 | GND | Ground |
| 23 | IDEIOW* | I/O write |
| 24 | GND | Ground |
| 25 | IDEIOR* | I/O read |
| 26 | GND | Ground |
| 27 | CHRDY | I/O channel ready |
| 28 | IDEBALE | Address latch enable |
| 29 | IDEDAK* |  |
| 30 | GND | Ground |
| 31 | IDEIRQ14 | IRQ 14 |
| 32 | CS16 |  |
| 33 | IDESA1 | Address bit 1 |
| 34 | PDIAG* |  |
| 35 | IDESA0 | Address bit 0 |
| 36 | IDESA2 | Address bit 2 |
| 37 | IDECS0* | Chip select 0 |
| 38 | IDECS1* | Chip select 1 |
| 39 | IDEHDACT*/ DR1PRES* | Disk activity light |
| 40 | GND | Ground |

## Power connector

## J1

| Pin | Name | Function |
| --- | --- | --- |
| 1 | PWRGD | Power Good |
| 2 | +5 V | + 5 volts |
| 3 | +12 V | + 12 volts |
| 4 | -12 V | - 12 volts |
| 5 | GND | Ground |
| 6 | GND | Ground |

Table C-5. Power Connector Pin-Out J1

## J2

| Pin | Name | Function |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | GND | Ground |
| 3 | -5 V | - 5 volts |
| 4 | +5 V | + 5 volts |
| 5 | +5 V | + 5 volts |
| 6 | +5 V | + 5 volts |

Table C-6. Power Connector Pin-Out J2

# Appendix E - BIOS Recovery

The Classic E-Series incorporates a Phoenix Technologies system BIOS on a FLASH component. FLASH BIOS allows easy upgrades without the need to replace an EPROM. The upgrade utility fits on a floppy diskette and provides the capability to save, verify, and update the system BIOS. The utility can be run from a hard drive or a network drive, but no memory managers can be installed during upgrades.

The latest upgrade utility and BIOS code are available in the public section of the iPAN bulletin board.

## Using the Upgrade Utility

If the utility is obtained from iPAN, UNZIP the archive and copy the files to a bootable MS-DOS 3.3, 4.01, 5.0, or 6.0 bootable diskette. Reboot the system with the upgrade diskette in the bootable floppy drive and follow the directions in the easy to use menu-driven program.

## Recovery Mode

In the unlikely event that a FLASH upgrade is interrupted, it is possible the BIOS may be left in an unusable state. Recovering from this condition requires the following steps.

1.	Change jumper J3 to position 1-2.

2.	Install the bootable upgrade diskette into drive A:

3.	Reboot the system.

4.	Because of the small amount of code available in the non-erasable boot block area, no video is available to direct the procedure. The procedure can be monitored by listening to the speaker and looking at the floppy drive LED. When the system beeps and the floppy drive LED is lit, the system is copying the recovery code into the FLASH device. As soon as the drive LED goes off, the system can be turned off.

5.	Reset jumper J3 to position 2-3.

6.	Leave the upgrade floppy in drive A: and turn the system on.

7.	Continue with the original upgrade.

# Appendix F - Memory Map

| Address Range (Decimal) | Address Range (hex) | Size | Description |
| --- | --- | --- | --- |
| 16384K-32768K | 1000000-2000000 | 16384K | Extended Memory |
| 16256K-16383K | FE0000-FFFFFF | 128K | System & Video BIOS Copy |
| 1024K-16255K | 100000-FDFFFF | 15232K | Extended Memory |
| 960K-1023K | F0000-FFFFF | 64K | Phoenix System BIOS |
| 952K-959K | EE000-EFFFF | 8K | FLASH Boot Block (Available as himem) |
| 948K-951K | ED000-EDFFF | 4K | User available |
| 928K-947K | E8000-ECFFF | 20K | Phoenix Setup Program (message disable via setup pre-boot only; with this option open to himem after boot) |
| 896K-927K | E0000-E7FFF | 32K | BIOS reserved |
| 800K-895K | C8000-DFFFF | 96K | User available |
| 768K-799K | C000-C7FFF | 32K | User available (many VGA cards use for video BIOS) |
| 736K-767K | B8000-BFFFF | 32K | VGA Display Memory (available unless VGA card present) |
| 704K-735K | B0000-B7FFF | 32K | VGA & Mono Display Mem (user available if no VGA/Mono) |
| 640K-703K | A0000-AFFFF | 64K | VGA Display Memory (available if no VGA present) |
| 639K | 9FC00-9FFFF | 1K | Extended BIOS Data (moveable by QEMM,386MAX) |
| 512K-638K | 80000-9FBFF | 127K | Extended conventional |
| 0K-511K | 00000-7FFFF | 512K | Conventional |

Table E-1. Classic E-Series Memory Map

# Appendix G - System Interrupts

| IRQ | System Resource |
| --- | --- |
| NMI | Parity Error |
| 0 | Reserved, Interval Timer |
| 1 | Reserved, Keyboard buffer full |
| 2 | Reserved, Cascade interrupt from slave PIC |
| 3 | On-board Serial Port 2 |
| 4 | On-board Serial Port 1 |
| 5 | User available |
| 6 | On-board Floppy controller |
| 7 | On-board Parallel Port |
| 8 | Real Time Clock |
| 9 | User available |
| 10 | User available |
| 11 | User available |
| 12 | User available |
| 13 | Reserved, Math coprocessor |
| 14 | IDE if enabled |
| 15 | User available |

# Appendix H - Baby AT Chassis Suppliers

Axxion Group Corporation

11 B. Leigh Fisher

El Paso, TX  79906

(915) 772-0088

Olsen Metal Products

1001 Crossroads Boulevard

Seguin, TX  78155

(512) 379-2799

Bermo Inc.

4501 Ball Road

Northeast Cir. Pines, MN

55014      (612) 786-7676

# Appendix I - Environmental Standards

| Parameter | Condition | Specification |
| --- | --- | --- |
| Temperature | Non-Operating | -40oC to +70oC |
|  | Operating | +10oC to +35oC |
| Humidity | Non-Operating | 95% Relative Humidity max. @ 35oC |
|  | Operating, no hard disk | 85% RH max. |
|  | Operating, w hard disk | Not to exceed 26oC wet bulb |
| Altitude | Non-Operating | 50,000 feet (15,240 meters) |
|  | Operating | 10,000 feet (3048 meters) |
| ESD | 1.0kV | No Errors |
|  | 2.5kV | No Errors |
|  | 5.0kV | 5% Soft Errors, 0% Hard Errors, No physical damage |
|  | 7.5kV | 10% Soft Errors, 0% Hard Errors, No physical damage |
|  | 10.0kV | 25% Soft Errors, 5% Hard Errors, No physical damage |
|  | 12.5kV | 50% Soft, 10% Hard, No physical damage |
|  | 15.0kV | 100% Soft, 25% Hard, No physical damage |
|  | 25.0kV | 100% Soft, 100% Hard, No physical damage |
| Shock | Non-Operating | 30.0G, 11ms, 1/2 sine |
| Acoustical Noise | 1 meter, peripherals idle | Less than 41 dB max. |

## Safety Certifications

| Underwriters Laboratories | UL1950 1st edition | Listed |
| --- | --- | --- |
| Canadian Standards Assn. | CSA C22.2 No.950-M89 | Certified |
| TUV Rheinland of N.A. | EN 60 950-1988 + A1/A2 | Certified |
| NEMKO (Norway) (includes SEMKO, Sweden, SETI, & Finland Certifications) | EN 60 950-1988 + A1/A2 | Certified |

## EMI Certifications

| Federal Communications Commission | CFR 47, Parts 2 and 15 Class B | Authorized |
| --- | --- | --- |
| German Post Office | CISPR 22, Class B | Registered to Vfg. 243/1991 |
| Canadian Dept. of Comm. | C.R.C., c.1374, Class B | Compliant |
| Voluntary Control Council | Class 2 I.T.E. | Registered |
| VDE (Germany) | VDE 0871 Class B | Tested/Compliant |
| NEMKO (Norway) (includes SEMKO, Sweden, SETI, & Finland Certifications) | EN 55 022, Class B | Certified |

# Appendix J - Reliability Data

The Mean-Time-Between-Failures (MTBF) data is calculated from predicted data @ 35C and 55C.

Classic E-Series baseboard (4 MB)		141,834 hours @ 55oC, 100% duty cycle

Power Supply (Astec SA201-3440)		100,000 hours @ 50oC, 100% duty cycle

Floppy drive (Teac FD-235HF-4240)		30,000 hours @ 50oC, 100% duty cycle

Classic E-Series system (4 MB, floppy)		82,772 hours @ 35oC

# Appendix K - Physical Specifications

## system

| Height | 15.875 cm | 6.25" (with feet installed) |
| --- | --- | --- |
| Width | 41.529 cm | 16.35" |
| Depth | 42.545 cm | 16.75" |
| Weight | 9.55 kg | 21 lbs (floppy installed) |

## board

| Length | 33.1216 cm | 13.040" |
| --- | --- | --- |
| Width | 21.8948 cm | 8.62" |
| Height (no heatsink) | 2.921 cm | 1.15" including 4MB SIMM. * |
| Weight |  | 1.42 pounds |

# *measured from the bottom of the PBA (not including the pins sticking out) to the top of an installed SIMM module.

# Appendix L - Customer Support

The Classic E-Series is backed by Intel's industry-leading support groups in the OEM Products and Services Division (OPSD), including IntelTechDirect.ä OPSD can support many of your network integration and service needs, including worldwide integration and system repair services. IntelTechDirect provides these services:

## iPAN (Intel Product Assistance Network)

An electronic Bulletin board with current product information, demo software and more...

- Available worldwide through direct-dial

- Modem speeds up to 14.4k baud with standard software

- FLASH BIOS upgrade files

- Modem set at no parity, 8 data bits, 1 stop bit.

## iPUB (Intel Product Update Bulletin)

- Monthly Product updates available 24 hours a day from iPAN

- Official notification of engineering changes and technical data

- Easy information retrieval using Windows Help file format

- Intel platform system, board, and BIOS revision histories

- Hardware and software compatibility notes

- Documentation updates, spare parts and order information

## iPALS (Intel Phone Action Line Support)

A direct telephone support line backed by highly qualified and well trained technical personnel.

- Toll-free access to Intel support engineers for problem resolution

- Responses within 24 hours Monday-Friday

- Expert assistance geared to the secial needs of OEMs and VARs

## FaxBack™

- Product descriptions and technical data sent to any fax machine from a touchtone phone

- Information on End-of-Life products

For information about IntelTechDirect please contact your local Intel Sales Representative.

# Appendix M - Product Codes

## Systems

DT486SX254F	i486 SX/25 system, 4 MB, 3.5" floppy

DT486DX334F	i486 DX/33 system, 4 MB, 3.5" floppy

*All systems ship with an accessories kit

## Boards

BDT486SX254	i486 SX/25 baseboard, 4 MB (Bulk shipped in quantities of 10)

BDT486DX334	i486 DX/33 baseboard, 4 MB (Bulk shipped in quantities of 10)

## Accessories

Accessory kit includes:

·	One power cord

·	One product guide (Order # 469281-001)

·	Two keys

·	One IDE cable

## documentation

The Classic-E system ships with a product guide, but also will have an online electronic Technical Reference manual using a Windows Help engine interface. The electronic Technical Reference manual will be available via the iPAN bulletin board system.

For a complete system memory map, consult Appendix E.