AMD K6-2 450~550 MHZ CPU settting table

( with PC-100 SDRAM combination, if early PC-66 SDRAM , please follow Async setting  BIOS ver : 1.0 08 )

- Voltage setting

| # Voltage | SW2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1-5 | 2-6 |  |  | 3 | 4 |  |
| 2.2 V | Off |  | On | Off |  |  | Off |
| 2.3 V | On |  | On | Off |  |  | Off |
| 2.4 V | Off |  | Off | On |  |  | Off |

- Frequency setting

| Frequency | SW1 |  |  |  |  |  | JP2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |
| K6-2 & III 450 | Off | Off | On | On | On | On | 1-2 |
| K6-2+ 450 | Off | Off | On | On | On | On | 1-2 |
| K6-2 475 | On | Off | On | Off | On | On | 1-2 |
| K6-2 500 | Off | Off | On | Off | On | On | 1-2 |
| K6-2 550 | Off | Off | On | Off | Off | On | 1-2 |

- other common setting :

JP1 :  1-3  ,  2-4  close

JP3 :  2-3    close

## JP4 :  1-2    close

***   Recommendation  :

- Do not plug too High-End AGP ( TNT series , Voodoo 3 ) card .

suggestion as below 8MB , such as : SiS 6326 , S3 Trio3D , Trident

9850  display cards .

2.  Shall use “ Bigger “  CPU FAN for high speed CPU .

1. AMD K6-2 450 MHZ  ( 100 x 4.5   2.3V )