AMD K6-2 450~550 MHZ CPU settting table

( with PC-100 SDRAM combination, if early PC-66 SDRAM , please follow Async setting  ,  BIOS ver: 2.0r02  )

- Voltage setting

| # Voltage | JP4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1-5 | 2-6 |  |  | 3-7 | 4-8 |  |
| 2.2 V | Open |  | Close | Open |  |  | Open |
| 2.3 V | Close |  | Close | Open |  |  | Open |
| 2.4 V | Open |  | Open | Close |  |  | Open |

- Frequency setting

| Frequency | SW1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| K6-2 450 | On | On | On | Off | Off | Off | Off |
| K6-III 450 | On | On | On | Off | Off | Off | Off |
| K6-2 475 | On | On | Off | Off | Off | On | Off |
| K6-2 500 | On | On | Off | Off | Off | Off | Off |
| K6-2 550 | On | Off | Off | Off | Off | Off | Off |

- other common setting :

JP1 :  2-3   close

JP2 :  1-2  close

JP3 :  2-3   close

## JP5 :  1-2 , 3-4 , 5-6   close

***   Recommendation  :

- Do not plug too High-End AGP ( TNT series , Voodoo 3 ) card .

suggestion as below 8MB , such as : SiS 6326 , S3 Trio3D , or PCI

display cards .

2.  Shall use Bigger CPU FAN for high speed CPU .

1. AMD K6-2 450 MHZ  ( 100 x 4.5   2.3V )