AMD K6-2 450~500 MHZ CPU settting table

( with PC-100 SDRAM combination, if early PC-66 SDRAM , please follow Async setting  ,  BIOS ver : 1.0 08 GS  )

- Voltage setting

| # Voltage | SW2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  |  | 3 | 4 |  |
| 2.2 V | Off |  | On | Off |  |  | Off |
| 2.3 V | On |  | On | Off |  |  | Off |
| 2.4 V | Off |  | Off | On |  |  | Off |

- Frequency setting

| Frequency | SW1 |  |  |  |  |  | JP7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |
| K6-2 450 | Off | Off | On | On | On | On | 1-2 |
| K6-III 450 | Off | Off | On | On | On | On | 1-2 |
| K6-2 475 | On | Off | On | Off | On | On | 1-2 |
| K6-2 500 | Off | Off | On | Off | On | On | 1-2 |

- other common setting :

JP5 :  1-2   close

JP6 :  2-3   close

## JP13 :  1-2 , 4-5   close

***   Note  :

- CPU support to AMD K6-2 500 MHZ ( message show 506 MHZ ,

the 550 MHZ show 531 MHZ ) .

- the ATC-5200 V1.0 support 90 MHZ frequency , V1.1 support 95 MHZ frequency ( for AMD K6-2 475 MHZ ) .