A-Trend Technical Notice                        No:TN000601

Subject	:	A-Trend BX mainboard , model ATC-6240 , two service issues :

(1). How to update the BIOS for ATC-6240 mainboard .

(2). Which version of ATC-6240 to support P-III/coppermine CPU ?

Index		:	BIOS V1.0 series , for ATC-6240 PCB R01 ~ R03 , no coppermine .

BIOS V2.0 series , for ATC-6240 PCB R05 , no coppermine supporting .

BIOS V3.0 series , for ATC-6240 PCB R06 , for coppermine supporting .

Content :

(1).	How to update the BIOS for ATC-6240 mainboard .

  - To record of your ATC-6240 mainboard’s current BIOS version , it is only

upgradable following its unique BIOS series , (for example : V1.0 series ,

V2.0 series , V3.0 series) , it can not mix .

  - How to recognize the ATC-6240 hardware version matching with BIOS

version V1.0 series , V2.0 series or V3.0 series ?

The BIOS V1.0 series : for Green_color PCB , PCB version R01 ~ R03.

The BIOS V2.0 series : for golden_color PCB , PCB version R05 .

The BIOS V3.0 series : for golden_color PCB , PCB version R06 .

(The PCB version is printed on the back side of PCB , near to DIMM socket)

For more information to differentiate BIOS V2.0 /V3.0 series on the ATC-6240 :

The BIOS V2.0 series is only for PCB version R05 ,

The BIOS V3.0 series is only for PCB version R06 ,

And how to differentiate ATC-6240 PCB R05 and ATC-6240 PCB R06 :

Please check the front side PCB of ATC-6240 mainboard , and its location is

between PCI slot 1 and PCI slot 2 .

A-Trend model ATC-6240		       A-Trend model ATC6240V-01

Test to comply 		                         Test to comply

With FCC standards                                  With FCC standards

For HOME or OFFICE USE                         For HOME or OFFICE USE

(For BIOS V2.0 series, PCB R05)                 (For BIOS V3.0 series, PCB R06)

(2). ATC-6240 (which BIOS is V3.0 series , PCB R06) supports P-III coppermine CPU .

A-Trend Technical service , Armin Lin . 2000-06-13