ABIT COMPUTER CORPORATION

PW4/PW4T REV. 1.3

Processor  80486SX/SL80486SX/CX486M7/AM486DX/80486DX/SL80486DX/CX486DX2/

AM486DX2/80486DX2/SL80486DX2/AM486DX4/80486DX4

Processor  25/33/40/50(internal)/50/66(internal)/75(internal)/80(internal)/100(internal)MHz

Speed

Chip Set   Unidentifed

Max.       192MB

Onboard

DRAM

Cache      128/256/512/1024KB

BIOS       Award

Dimensions 330mm x 218mm

I/O        32-bit VESA local bus slots (3), green PC connector

Options

NPU        None

Options

[Image]

CONNECTIONS

Purpose                   Location    Purpose                 Location

External battery             J1       Power LED & keylock       JW1

Green PC connector          JP47      Speaker                   JW2

Green PC connector          JP48      Turbo LED                 JW3

Green PC connector          JP49      Turbo switch              JW4

Green PC connector          JP50      Reset switch              JW5

Green PC connector          JP51      32-bit VESA local      SL1 - SL3

bus slots

Green PC connector          JP52

USER CONFIGURABLE SETTINGS

| Function | Jumper | Position |
| --- | --- | --- |
| >> Monitor type select color | JP5 | Closed |
| Monitor type select monochrome | JP5 | Open |
| >> Factory configured - do not Alter | JP6 | Open |
| >> Factory configured - do not Alter | JP41 | Open |
| >> CMOS memory normal operation | JP67 | pins 1 & 2 closed |
| CMOS memory clear | JP67 | pins 2 & 3 closed |

DRAM CONFIGURATION

| Size | Bank 0 | Bank 1 | Bank 2 |
| --- | --- | --- | --- |
| 1MB | (4) 256K x 9 | NONE | NONE |
| 1MB | NONE | (1) 256K x 36 | NONE |
| 1MB | NONE | NONE | (1) 256K x 36 |
| 2MB | (4) 256K x 9 | (1) 256K x 36 | NONE |
| 2MB | (4) 256K x 9 | (1) 256K x 36 | NONE |
| 2MB | NONE | (1) 256K x 36 | (1) 256K x 36 |
| 2MB | NONE | (1) 512K x 36 | NONE |
| 2MB | NONE | NONE | (1) 512K x 36 |
| 3MB | (4) 256K x 9 | (1) 512K x 36 | NONE |
| 3MB | (4) 256K x 9 | (1) 256K x 36 | (1) 256K x 36 |
| 3MB | (4) 256K x 9 | (1) 512K x 36 | NONE |
| 3MB | NONE | (1) 256K x 36 | (1) 512K x 36 |
| 4MB | (4) 1M x 9 | NONE | NONE |
| 4MB | NONE | (1) 1M x 36 | NONE |
| 4MB | NONE | (1) 512K x 36 | (1) 512K x 36 |
| 4MB | NONE | NONE | (1) 1M x 36 |
| 5MB | (4) 256K x 9 | (1) 1M x 36 | NONE |
| 5MB | (4) 1M x 9 | (1) 256K x 36 | NONE |
| 5MB | (4) 1M x 9 | (1) 256K x 36 | NONE |
| 5MB | (4) 256K x 9 | (1) 512K x 36 | (1) 512K x 36 |
| 5MB | NONE | (1) 256K x 36 | (1) 1M x 36 |
| 6MB | (4) 1M x 9 | (1) 512K x 36 | NONE |
| 6MB | (4) 1M x 9 | (1) 256K x 36 | (1) 256K x 36 |
| 6MB | (4) 256K x 9 | (1) 256K x 36 | (1) 1M x 36 |
| 6MB | (4) 1M x 9 | (1) 512K x 36 | NONE |
| 6MB | NONE | (1) 1M x 36 | (1) 512K x 36 |
| 8MB | (4) 1M x 9 | (1) 1M x 36 | NONE |
| 8MB | (4) 1M x 9 | (1) 512K x 36 | (1) 512K x 36 |
| 8MB | NONE | (1) 2M x 36 | NONE |
| 8MB | NONE | NONE | (1) 2M x 36 |
| 9MB | (4) 1M x 9 | (1) 256K x 36 | (1) 1M x 36 |
| 9MB | (4) 256K x 9 | (1) 1M x 36 | (1) 1M x 36 |
| 9MB | (4) 256K x 9 | (1) 2M x 36 | NONE |
| 11MB | (4) 256K x 9 | (1) 512K x 36 | (1) 2M x 36 |
| 12MB | (4) 1M x 9 | (1) 2M x 36 | NONE |

DRAM CONFIGURATION (CON’T)

| Size | Bank 0 | Bank 1 | Bank 2 |
| --- | --- | --- | --- |
| 12MB | (4) 1M x 9 | (1) 1M x 36 | (1) 1M x 36 |
| 12MB | (4) 1M x 9 | (1) 2M x 36 | NONE |
| 12MB | NONE | (1) 1M x 36 | (1) 2M x 36 |
| 16MB | (4) 4M x 9 | NONE | NONE |
| 16MB | NONE | (1) 4M x 36 | NONE |
| 16MB | NONE | (1) 2M x 36 | (1) 2M x 36 |
| 16MB | NONE | NONE | (1) 4M x 36 |
| 17MB | (4) 256K x 9 | (1) 4M x 36 | NONE |
| 17MB | (4) 4M x 9 | (1) 256K x 36 | NONE |
| 17MB | (4) 256K x 9 | (1) 2M x 36 | (1) 2M x 36 |
| 17MB | NONE | (1) 256K x 36 | (1) 4M x 36 |
| 18MB | (4) 4M x 9 | (1) 512K x 36 | NONE |
| 18MB |  |  |  |

(4) 4M x 9

(1) 256K x 36

(1) 256K x 36

18MB

(4) 256K x 9

(1) 256K x 36

(1) 4M x 36

18MB

(4) 4M x 9

(1) 512K x 36

NONE

18MB

NONE

(1) 4M x 36

(1) 512K x 36

20MB

(4) 1M x 9

(1) 4M x 36

NONE

20MB

(4) 4M x 9

(1) 1M x 36

NONE

20MB

(4) 1M x 9

(1) 2M x 36

(1) 2M x 36

20MB

(4) 4M x 9

(1) 512K x 36

(1) 512K x 36

20MB

NONE

(1) 1M x 36

(1) 4M x 36

21MB

(4) 4M x 9

(1) 256K x 36

(1) 1M x 36

21MB

(4) 1M x 9

(1) 256K x 36

(1) 4M x 36

21MB

(4) 256K x 9

(1) 1M x 36

(1) 4M x 36

24MB

(4) 4M x 9

(1) 2M x 36

NONE

24MB

(4) 4M x 9

(1) 1M x 36

(1) 1M x 36

24MB

(4) 1M x 9

(1) 1M x 36

(1) 4M x 36

24MB

(4) 4M x 9

(1) 2M x 36

NONE

24MB

NONE

(1) 4M x 36

(1) 2M x 36

32MB

(4) 4M x 9

(1) 4M x 36

NONE

32MB

(4) 4M x 9

(1) 2M x 36

(1) 2M x 36

32MB

NONE

(1) 4M x 36

(1) 4M x 36

32MB

NONE

(1) 8M x 36

NONE

32MB

NONE

NONE

(1) 8M x 36

33MB

(4) 256K x 9

(1) 8M x 36

NONE

33MB

(4) 4M x 9

(1) 256K x 36

(1) 4M x 36

33MB

(4) 256K x 9

(1) 4M x 36

(1) 4M x 36

33MB

(4) 256K x 9

(1) 8M x 36

NONE

33MB

NONE

(1) 256K x 36

(1) 8M x 36

34MB

NONE

(1) 512K x 36

(1) 8M x 36

35MB

(4) 256K x 9

(1) 512K x 36

(1) 8M x 36

36MB

(4) 1M x 9

(1) 8M x 36

NONE

36MB

(4) 4M x 9

(1) 1M x 36

(1) 4M x 36

36MB

(4) 1M x 9

(1) 4M x 36

(1) 4M x 36

DRAM CONFIGURATION (CON’T)

Size            Bank 0           Bank 1          Bank 2

36MB          (4) 1M x 9      (1) 8M x 36         NONE

36MB             NONE         (1) 1M x 36      (1) 8M x 36

38MB          (4) 1M x 9     (1) 512K x 36     (1) 8M x 36

40MB             NONE         (1) 2M x 36      (1) 8M x 36

41MB         (4) 256K x 9     (1) 2M x 36      (1) 8M x 36

44MB          (4) 1M x 9      (1) 2M x 36      (1) 8M x 36

48MB          (4) 4M x 9      (1) 8M x 36         NONE

48MB          (4) 4M x 9      (1) 4M x 36      (1) 4M x 36

48MB             NONE         (1) 4M x 36      (1) 8M x 36

56MB          (4) 4M x 9      (1) 2M x 36      (1) 8M x 36

64MB         (4) 16M x 9          NONE            NONE

64MB             NONE         (1) 16M x 36        NONE

64MB             NONE         (1) 8M x 36      (1) 8M x 36

64MB             NONE             NONE        (1) 16M x 36

65MB         (4) 256K x 9     (1) 16M x 36        NONE

65MB         (4) 16M x 9     (1) 256K x 36        NONE

65MB         (4) 16M x 9     (1) 256K x 36        NONE

65MB         (4) 256K x 9     (1) 8M x 36      (1) 8M x 36

65MB             NONE        (1) 256K x 36    (1) 16M x 36

66MB         (4) 16M x 9     (1) 512K x 36        NONE

66MB         (4) 16M x 9     (1) 256K x 36    (1) 256K x 36

66MB         (4) 256K x 9    (1) 256K x 36    (1) 16M x 36

66MB         (4) 16M x 9     (1) 512K x 36        NONE

66MB             NONE         (1) 16M x 36    (1) 512K x 36

68MB          (4) 1M x 9      (1) 16M x 36        NONE

68MB         (4) 16M x 9      (1) 1M x 36         NONE

68MB          (4) 1M x 9      (1) 8M x 36      (1) 8M x 36

68MB         (4) 16M x 9     (1) 512K x 36    (1) 512K x 36

68MB             NONE         (1) 1M x 36     (1) 16M x 36

69MB         (4) 16M x 9     (1) 256K x 36     (1) 1M x 36

69MB          (4) 1M x 9     (1) 256K x 36    (1) 16M x 36

69MB         (4) 256K x 9     (1) 1M x 36     (1) 16M x 36

72MB         (4) 16M x 9      (1) 2M x 36         NONE

72MB         (4) 16M x 9      (1) 1M x 36      (1) 1M x 36

72MB          (4) 1M x 9      (1) 1M x 36     (1) 16M x 36

72MB         (4) 16M x 9      (1) 2M x 36         NONE

72MB             NONE         (1) 16M x 36     (1) 2M x 36

74MB         (4) 16M x 9     (1) 512K x 36     (1) 2M x 36

80MB          (4) 4M x 9      (1) 16M x 36        NONE

80MB         (4) 16M x 9      (1) 4M x 36         NONE

80MB         (4) 16M x 9      (1) 2M x 36      (1) 2M x 36

81MB         (4) 16M x 9     (1) 256K x 36     (1) 4M x 36

81MB          (4) 4M x 9     (1) 256K x 36    (1) 16M x 36

81MB         (4) 256K x 9     (1) 4M x 36     (1) 16M x 36

DRAM CONFIGURATION (CON’T)

Size

Bank 0

Bank 1

Bank 2

84MB

(4) 16M x 9

(1) 1M x 36

(1) 4M x 36

84MB

(4) 4M x 9

(1) 1M x 36

(1) 16M x 36

84MB

(4) 1M x 9

(1) 4M x 36

(1) 16M x 36

96MB

(4) 16M x 9

(1) 8M x 36

NONE

96MB

(4) 4M x 9

(1) 4M x 36

(1) 16M x 36

96MB

NONE

(1) 16M x 36

(1) 8M x 36

128MB

(4) 16M x 9

(1) 16M x 36

NONE

128MB

(4) 16M x 9

(1) 8M x 36

(1) 8M x 36

128MB

NONE

(1) 16M x 36

(1) 16M x 36

129MB

(4) 16M x 9

(1) 256K x 36

(1) 16M x 36

129MB

(4) 256K x 9

(1) 16M x 36

(1) 16M x 36

132MB

(4) 16M x 9

(1) 1M x 36

(1) 16M x 36

132MB

(4) 1M x 9

(1) 16M x 36

(1) 16M x 36

144MB

(4) 16M x 9

(1) 4M x 36

(1) 16M x 36

144MB

(4) 4M x 9

(1) 16M x 36

(1) 16M x 36

192MB

(4) 16M x 9

(1) 16M x 36

(1) 16M x 36

CACHE CONFIGURATION

Size          Bank 0          Bank 1           TAG

128KB        (4) 32K x 8        NONE        (1) 32K x 8

256KB        (4) 32K x 8     (4) 32K x 8    (1) 32K x 8

256KB        (4) 64K x 8        NONE        (1) 64K x 8

512KB        (4) 64K x 8     (4) 64K x 8    (1) 64K x 8

512KB       (4) 128K x 8        NONE        (1) 128K x 8

1MB        (4) 128K x 8    (4) 128K x 8    (1) 128K x 8

CACHE JUMPER CONFIGURATION

Size

JP60

JP61

JP62

JPX3

128KB

pins 2 & 3

closed

pins 1 & 2

closed

pins 2 & 3

closed

pins 9 - 16

closed

256KB

pins 2 & 3

closed

pins 1 & 2

closed

pins 2 & 3

closed

pins 13 - 20

closed

256KB

pins 2 & 3

closed

pins 2 & 3

closed

pins 2 & 3

closed

pins 3 - 10

closed

512KB

pins 2 & 3

closed

pins 1 & 2

closed

pins 2 & 3

closed

pins 1 - 8

closed

512KB

pins 2 & 3

closed

pins 1 & 2

closed

pins 2 & 3

closed

pins 3 - 10

closed

1MB

pins 1 & 2

closed

pins 1 & 2

closed

pins 1 & 2

closed

pins 1 - 8

closed

CPU TYPE CONFIGURATION

Type            JP15          JP16          JP17          JP37

80486SX          Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

SL80486SX         Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

CX486M7          Open        pins 1 & 2   pins 1 & 2       Open

closed        closed

AM486DX          Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

80486DX          Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

SL80486DX         Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

CX486DX2-50        Open        pins 2 & 3   pins 2 & 3       1 & 2

closed        closed

CX486DX2-66        Open        pins 1 & 2   pins 1 & 2       Open

closed        closed

CX486DX2-80        Open        pins 1 & 2   pins 1 & 2       Open

closed        closed

AM486DX2 (3.45v)   pins 2 & 3    pins 2 & 3   pins 2 & 3       Open

closed        closed        closed

AM486DX2          Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

80486DX2          Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

SL80486DX2         Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

AM486DX4          Open        pins 1 & 2   pins 1 & 2       Open

closed        closed

80486DX4-75        Open        pins 2 & 3   pins 2 & 3     pins 1 & 2

closed        closed        closed

80486DX4-100        Open        pins 2 & 3   pins 2 & 3       Open

closed        closed

Note: DX4 CPU is only available on the PW4T mainboard.

CPU TYPE CONFIGURATION

Type

JP38

JP39

JP40

RN8

80486SX

Open

Open

Open

Installed

SL80486SX

Open

Open

Open

Installed

CX486M7

Open

Open

Open

Installed

AM486DX

Open

Open

Open

Installed

80486DX

Open

Open

Open

Installed

SL80486DX

Open

Open

Open

Installed

CX486DX2-50

Open

Open

Open

Not installed

CX486DX2-66

Open

1 & 2

Open

Not installed

CX486DX2-80

Open

Open

1 & 2

Not installed

AM486DX2 (3.45v)

1 & 2

Open

Open

Not installed

AM486DX2

Open

Open

Open

Installed

80486DX2

Open

Open

Open

Installed

SL80486DX2

Open

Open

Open

Installed

AM486DX4

1 & 2

Open

1 & 2

Not installed

80486DX4-75

Open

Open

Open

Not installed

80486DX4-100

1 & 2

Open

1 & 2

Not installed

Note: Pins designated should be in the closed position. DX4 CPU is only

available on the PW4T mainboard.

CPU TYPE CONFIGURATION

Type          RN5           RN6           RN7         JP20     JP26

80486SX          Not       Installed         Not       Closed    1 & 2

installed                   installed

SL80486SX         Not       Installed         Not       Closed    1 & 2

installed                   installed

CX486M7      Installed         Not       Installed      Open     1 & 2

installed

AM486DX          Not       Installed     Installed      Open     1 & 2

installed

80486DX          Not       Installed     Installed      Open     1 & 2

installed

SL80486DX         Not       Installed     Installed      Open     1 & 2

installed

AM486DX2         Not       Installed     Installed      Open     1 & 2

installed

80486DX2         Not       Installed     Installed      Open     1 & 2

installed

SL80486DX2        Not       Installed     Installed      Open     1 & 2

installed

AM486DX4         Not       Installed     Installed      Open     Open

installed

80486DX4         Not       Installed     Installed      Open     Open

installed

Note: Pins designated should be in the closed position. DX4 CPU is only

available on the PW4T mainboard.

CPU TYPE CONFIGURATION (CON’T)

Type

JP42

JP65

JPX1

JPX2

80486SX

Open

Open

pins 5 - 12

closed

Open

SL80486SX

Closed

Open

pins 5 - 12

closed

Open

CX486M7

Closed

Closed

pins 1 - 8

closed

pins 1 - 8

closed

AM486DX

Open

Closed

pins 5 - 12

closed

Open

80486DX

Open

Closed

pins 5 - 12

closed

Open

SL80486DX

Closed

Closed

pins 5 - 12

closed

Open

AM486DX2

Open

Closed

pins 5 - 12

closed

Open

80486DX2

Open

Closed

pins 5 - 12

closed

Open

SL80486DX2

Closed

Closed

pins 5 - 12

closed

Open

AM486DX4

Closed

Closed

pins 5 - 12

closed

Open

80486DX4

Closed

Closed

pins 5 - 12

closed

Open

Note: DX4 CPU is only available on the PW4T mainboard.

CPU SPEED CONFIGURATION(U30 CLOCK GENERATOR)

Speed        JP27        JP28         JP29

20MHz        Open        Open        Closed

25MHz       Closed       Open        Closed

33MHz        Open       Closed       Closed

40MHz       Closed       Open         Open

50iMHz      Closed       Open        Closed

66iMHz       Open       Closed       Closed

75iMHz      Closed       Open        Closed

80iMHz      Closed       Open         Open

100iMHz       Open       Closed       Closed

CPU SPEED CONFIGURATION(U31 CLOCK GENERATOR)

Speed

JP27

JP28

JP29

20MHz

Open

Open

Open

25MHz

Open

Open

Closed

33MHz

Closed

Closed

Closed

40MHz

Open

Closed

Closed

50iMHz

Open

Open

Closed

66iMHz

Closed

Closed

Closed

75iMHz

Open

Open

Closed

80iMHz

Open

Closed

Closed

100iMHz

Closed

Closed

Closed

VESA WAIT STATE CONFIGURATION

Wait states          JP9

0 wait states        Open

1 wait state        Closed

BUS SPEED CONFIGURATION

CPU speed

JP10

<= 33MHz

Open

> 33MHz

Closed