# TEST REPORT                               Dec . 27 . 1999

##### 5VIA73

ACORP  ELECTRONICS CORP.

8F, No.123-5 , Shing Teh RD. San Chung City , 
Taipei Hsien , Taiwan R.O.C.
Tel:886-2-2999-7188  Fax:886-2-2999-7177

http://www.acorp.com.tw

E-Mail:acorp@ms2.hinet.net

SPECIFICATION

01. Processor : Socket 7 Compatible Processor.

02. Chipset : VIA82C501/686 Chip set.

03. CPU Support : Intel , AMD , Cyrix and IDT.

04. Voltage Regulator : DIP SW ( 2.0V to 3.5V )

05. Bus CLK : 66/75/83/95/100/124 ... MHz.

06. Bus Architecture : 3 x 32 bit PCI Bus, 1 x 16 Bit ISA Bus.

07. L2 Cache Memory : On Board Pipeline SRAM 1 MB.

08. USB : Support 4 x Universal Serial Bas Port .

09. System Memory : 3 x 168 pin DIMM Socket 768M ( Max ).

10. On Board I/O : 1 x Floppy, 2 x Serial Ports, 1 x Parallel Port ( ECC,EPP,SPP ) ,

1 x Infrared Port ( IR ).

11. Power Support : ATX Power Connector Support.

12. BIOS : AWARD AGP BIOS with ACPI, Plug and Play Features 2MB Flash ROM,

Support CD-ROM , SCSI , LAN , LS120 boot.

13. Special Features : Temperature Senor ( Option ) , Warning by Sound , Auto Detec CPU

Thermal pin , Wake On Lan, wake on Modem, Wake on Alarm BUS,

Hard - ware Monitor ( option ).

14. DMA : Support Ultra DMA 33/66 .

15. Multimedia on Board : AC-97 Basic Audio.

16. VGA on Board : Trident 2x AGP VGA on Board.

17. Board Size : 22 x 23 cm ( AT Form ).

ACORP ELECTRONICS CORPORATON.                                                                                    Page  1
http://www.acorp.com.tw

PART A. SPECIFICATION TEST

- System Specific

- System Specific Test

- System BIOS Test

- System Utility Test

- System Diagnostic & Certification Test

- Field (Stress) Test

PART B. OPERATING SYSTEM ENVIRONMENT TEST

- Microsoft Windows 95, 98 Operating System Test

- Microsoft Windows NT Operating System Test

- Operating System Field (Stress) Test

ACORP ELECTRONICS CORPORATON.                                                                                    Page  2
http://www.acorp.com.tw

PART A . SPECIFICATION TEST

2. System Specific Test

| CPU TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vender |  | Model |  | Voltage |  | Clock Multiplier |  | Bus Speed |  |  |
| Intel |  | AMD K6 – PR2 166 |  | 2.9V |  | 2.5 x |  | 66 MHz |  |  |
|  |  | AMD K6 – 233 |  | 3.2 V |  | 2.5 x |  | 66 MHz |  |  |
|  |  | AMD K6 II - 350 |  | 2.2 V |  | 3.5 x |  | 100 MHz |  |  |
|  |  | AMD K6 III – 450 |  | 2.4 V |  | 4.5 x |  | 100 MHz |  |  |
|  |  | Intel MMX – 166 |  | 2.8 V |  | 2.5 x |  | 66MHz |  |  |
|  |  | Intel MMX – 200 |  | 2.8 V |  | 3 x |  | 100 MHz |  |  |
| Memory Test |  |  |  |  |  |  |  |  |  |  |
| Type | Size |  | Vender |  | Component |  | Type |  | Amount | Status |
| 168 Pin DIMM module | 16 Mytes |  | NEC |  | D451682165-A10-7JF |  | PC66 |  | 3 | Pass |
|  | 32 Mytes |  | NEC |  | D4516821AG5-A10-7JF |  | PC66 |  | 3 | Pass |
|  |  |  | SIEMENS |  | HYB39S16800AT-8 |  | PC66 |  | 3 | Pass |
|  | 64 Mytes |  | Kti |  | JSV884T4A0A |  | PC100 |  | 3 | Pass |
|  |  |  | LGS |  | GM72V66S41CT75 |  | PC100 |  | 3 | Pass |
|  |  |  | SEC |  | KM48S8030CT-GH |  | PC100 |  | 3 | Pass |
|  |  |  | JMC |  | 8008KTW-SS |  | PC100 |  | 3 | Pass |
|  |  |  | ACORP |  | AP52836408313-7 |  | PC100 |  | 3 | Pass |
|  | 128 Mytes |  | HITACHI |  | HM5264805TTB60 |  | PC100 |  | 3 | Pass |
|  |  |  | MOSEL |  | V54C365864VCT75 |  | PC133 |  | 3 | Pass |
|  |  |  | NEC |  | D4564841G5-A80-9JF |  | PC100 |  | 3 | Pass |
|  |  |  | SEC (D) |  | KM48S8030CT-F10 |  | PC100 |  | 3 | Pass |
|  |  |  | SEC (S) |  | KM48S16030T-GL |  | PC100 |  | 3 | Pass |
|  |  |  | Kti |  | TSV884T4A0A |  | PC100 |  | 3 | Pass |
|  |  |  | Winbond |  | W986408BH-75 |  | PC133 |  | 3 | Pass |
|  | 256 Mytes |  | SEC |  | KM44S16030BT-GH |  | PC100 |  | 2 | Pass |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  3
http://www.acorp.com.tw

| FDD Devices |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  | Model |  | O.S. Environment | Status |  |  |
| 1.44 MB FDD |  | Mitsumi D353T6 |  | Win95 V950B Win NT V 4.0 | Pass |  |  |
|  |  | NEC FD1231H |  | Win98SE V4.10 | Pass |  |  |
|  |  | TEAC FD235FH |  | Win95 V950B | Pass |  |  |
|  |  | Panasonic JU-257A606P |  | MS-DOS V6.22 Win2000 2128 | Pass |  |  |
| 120MB FDD |  | Mitsubishi MF357G-111MA |  | Win95 V950B Win98SE V4.10 Windows NT 4.0 | Pass |  |  |
| Serial Port Devices |  |  |  |  |  |  |  |
| Type |  | Model | O. S. Environment |  | Status | Remark/Special setting |  |
| Mouse |  | Genius Serial Mouse | Win95 V 950B Win98SE V 4.10 Win NT V 4.0 |  | Pass |  |  |
|  |  | Logitech First Mouse | Win95 V 950B Win98SE V 4.10 |  | Pass |  |  |
|  |  | NEC King Mouse | Win95 V 950B Win98SE V 4.10 |  | Pass |  |  |
|  |  | DTK Mouse | Win95 V 950B MS-DOS V 6.22 Win2000 V2128 |  | Pass |  |  |
| PS2 Port Devices |  |  |  |  |  |  |  |
| Type |  | Model | O. S. Environment |  | Status | Remark/Special setting |  |
| Mouse |  | Microsoft PS/2 Mouse | Win NT V 4.0 Win98SE V 4.10 Win95 V950B |  | Pass |  |  |
|  |  | Logitech PS/2 Mouse | Win98SE V 4.10 Win2000 V2128 |  | Pass |  |  |
|  |  | AT&T PS/2 Mouse | Win95 V 950B Win98SE V 4.10 |  | Pass |  |  |
|  |  | DTK PS/2 Mouse | Win95 V 950B MS-DOS V 6.22 |  | Pass |  |  |
|  |  | Agiler PS/2 Mouse | Win98SE V4.10 Win95 V 950B MS-DOS V 6.22 |  | Pass |  |  |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  4
http://www.acorp.com.tw

| Parallel Port Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Model | O. S. Environment | Status | Remark/Special setting |
| Print | HP Laser Jet 5L | MS-DOS V 6.22 Win98SE V 4.10 Win2000 V2128 | Pass |  |
|  | EPSON Color 740 | Win95 V 950B Win98SE V 4.10 | Pass |  |
| USB Devices |  |  |  |  |
| Typer | Model | O. S. Environment | Status | Remark/Special setting |
| Mouse | USB Mouse | Win98SE V 4.10 | Pass |  |
| Keyboard | USB Keyboard | MS-DOS V 6.22 Win98SE V 4.10 | Pass |  |
| HUB | USB 4- Port HUB | Win98SE V 4.10 | Pass |  |
| Modem | USB Modem | Win98SE V4.10 Win95 V 950B | Pass |  |
| ZIP | IOEMGA Z100USB | Win98SE V 4.10 Win2000 2128 | Pass |  |
| Print | EPSON Color 740 | Win98SE V 4.10 | Pass |  |

2 . System BIOS Test

2.1 System CMOS Setup

| System BIOS Test |  |  |  |
| --- | --- | --- | --- |
| Test Item |  | Status | Remark/Special setting |
| CMOS Setup Utility | Load BIOS Default | Pass |  |
|  | Load Setup Default | Pass |  |
|  | Supervisor Password | Pass |  |
|  | User Password | Pass |  |
|  | Save & Exit Setup | Pass |  |
|  | Exit Without Saving | Pass |  |
| Standard CMOS Setup Test | Sign-On Message And BIOS Information | Pass |  |
|  | POST error detection message Test | Pass |  |
|  | System Date and Time | Pass | Over 2000 year (BIOS /DOS/Win95/Win98) |
|  | Diskette A: / B: | Pass | None /1.44 MB |
|  | Hard Disk Drive | Pass | HDD Auto Detect/User type /None |
|  | Video System | Pass | EGA/VGA |
|  | System Memory | Pass |  |
|  | Extended Memory | Pass |  |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  5
http://www.acorp.com.tw

| System BIOS Test |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Item |  |  | Status | Remark/Special setting |
| #### BIOS Features Setup |  | Virus Warning | Pass | Enabled / Disabled |
|  |  | CPU Internal Cache | Pass | Enabled / Disabled |
|  |  | External Cache | Pass | Enabled / Disabled |
|  |  | Quick Power On Self Test | Pass | Enabled / Disabled |
|  |  | Boot Sequence | Pass | Floppy/LS,ZIP/SCSI HDD-0,1,2,3/CD-ROM LAN/Disabled |
|  |  | Swap Floppy Drive | Pass | Enabled / Disabled |
| Chipset Features Setup |  | On Chip USB | Pass | Enabled / Disabled |
|  |  | USB Keyboard Support | Pass | Enabled / Disabled |
|  |  | On Chip Sound | Pass | Enabled / Disabled |
| PNP/PCI Configuration |  | Resources controlled By | Pass | Auto/ Manual |
|  |  | Assign IRQ For USB | Pass | Enabled / Disabled |
| Integrated Peripherals |  | On Chip IDE Channe 0 | Pass | Enabled / Disabled |
|  |  | On Chip IDE Channe 1 | Pass | Enabled / Disabled |
|  |  | IDE Prefetch Mode | Pass | Enabled / Disabled |
|  |  | IDE HDD Block Mode | Pass | Enabled / Disabled |
|  |  | On –Chip Primary PCI IDE | Pass | Enabled / Disabled |
|  |  | On –Chip Secondary PCI IDE | Pass | Enabled / Disabled |
|  |  | Onboard FDC Controller | Pass | Enabled / Disabled |
|  |  | Onboard Serial port 1 | Pass | Auto/3F8/IRQ4/ Disabled |
|  |  | Onboard Serial port 2 | Pass | Auto/2F8/IRQ3/ Disabled |
|  |  | Onboard Parallel port | Pass | 3BC/IRQ7/ Disabled |
| Power Management Setup |  | ACPI Function | Pass | Enabled / Disabled |
|  |  | Power Management | Pass | User Define / Min Saving / Max Saving |
|  |  | Soft-off by PWR-BTTN | Pass | Delay 4 Sec / Instant-Off |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  6
http://www.acorp.com.tw

2.1 Boot sequence Test

| Boot Sequence Test |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Peripheral device | O.S. Environment | Status | Features Tested |
| #### A , C | WD 28400 UDMA66 | MS-DOS V 6.22 Win98SE V 4.10 | Pass |  |
| #### C , A | IBM DTTA-351010 UDMA33 | MS –DOS V 6.22 Win98SE V 4.10 | Pass |  |
|  | Quantum 4300 SE UDMA 33 | Win95 V 950B NT WKS V 4.0 | Pass |  |
|  | Quantum CX6400AT UDMA 66 | Win98SE V 4.10 Win2000 V 2128 | Pass |  |
|  | Fujitsu MPC 3064AT UDMA 33 | NT WKS V 4.0 Win98SE V 4.10 | Pass |  |
|  | Maxtor 90845U3 DMA 66 | Win2000 V 2128 | Pass |  |
| A , CDROM , C | Fujitsu APC3064AT Teac CD-532E | NT WKS 4.0 | Pass |  |
|  | Quantum SE43A012 Pioneer Dr-A14S | WIN98SE V 4.10 | Pass |  |
| COROM ,C,A | Seagate 6531 SDR-520H | Win98SE V 4.10 | Pass |  |
|  | WD 28400 VUEGO 623A | WinNT V 4.0 Win98SE V 4.10 | Pass |  |
|  | Seagate ST38421A Creative Dxr2 | WinNT V 4.10 | Pass |  |
|  | Maxtor 82560A4 DCS 36x | WinNT V 4.10 Win98SE V 4.10 | Pass |  |
| C only | IBM DJNA-352500 UDMA 66 | MS-DOS V 6.22 WIN98SE V 4.10 | Pass |  |
| LS/ZIP , C | WD 36400 Mitsubushi MF357G -111MA | MS-DOS V 6.22 Win98SE V 4.10 | Pass |  |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  7
http://www.acorp.com.tw

3 System Utility Test

| System Utility |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | O.S. Environment | Status |  | Features Tested |  |
| CBROM Utility | 1.3 | Award | MS-DOS V 6.22 | Pass |  | Add NCR ROM |  |
| AFLASH Utility | 6.5 7.05 | Award | MS -DOS V 6.22 | Pass |  | Flash a new BIOS |  |
| MODBIN Utility | 4.50.75 | Award | MS -DOS V 6.22 Win95 V 950B | Pass |  | Utility BIOS Version |  |
| Bus Master Driver |  |  |  |  |  |  |  |
| Package Name | Primary IDE |  | Secondary IDE |  | O.S. Environment |  | Status |
|  | HDD (Mode/Speed) |  | CD-ROM (Mode/Speed) |  |  |  |  |
| Win98 Driver Ver : 4.16 | IBM DTTA-351010 UDMA 33 |  | PINNEER 36x |  | Win98SE V 4.10 |  | Pass |
|  | WD 36400 UDMA 33 |  | Teac CD-532E 32x |  | Win98 V 4.10 (Chinese ) |  | Pass |
|  | Fujitsu MPC3064AT UDMA 33 |  | Pioneer Dr-A14S 36x |  | Win98SE V 4.10 |  | Pass |
|  | Quantum CX6400AT UDMA 66 |  | Creative Dxr2 DVD 2x |  | Win98 V 4.10 |  | Pass |
|  | Maxtor 90845U3 UDMA 66 |  | VUEGO 632A 32x |  | Win98 V 4.10 |  | Pass |
| WinNT 4.0 Driver Ver : 4.16 | Fujitsu MPC3064AT UDMA 33 |  | Creative Dxr2 DVD 2x |  | Win NT V 4.0 |  | Pass |
|  | Quantum CX 64A013 DMA 66 |  | VUEGO 632A 32x |  | Win NT V 4.0 |  | Pass |
|  | IBM DCAA-34330 DMA 33 |  | Teac CD-532E 32x |  | Win NT V 4.0 |  | Pass |
|  | WD 28400 UDMA 66 |  | DCS C840P 40x |  | WinNT V4.0 |  | Pass |

4. System Utility Test

| System Utility |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Package Name | Version | Publisher | O.S. Environment | Status | Features Tested |
| Check It | V3.0 | Tochstone | MS-DOS V 6.22 | Pass |  |
| QAPLUS | V5.52 | Diagsoft | MS -DOS V 6.22 | Pass |  |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  8
http://www.acorp.com.tw

5. Fied (stress) Test

| Test Type |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 III-450 + 512 MB ( SDRAM 256 MB x 2 ) + On- Board IDE /AGP VGA/Sound + AHA-2940UW S9 + Intel 82558 LAN + |  |  |
| Test procedure | Win98SE & WinBench 99 demo Test . |  |  |
| Result | ###### Pass | Duration | 72 Hours |

| Test Type |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 III - 450 + 384 MB ( SDRAM 128 MB x 3 ) + On- Board IDE /AGP VGA /Sound + IBM DJNA-352500 + 3Com 509B-TX LAN |  |  |
| Test procedure | Win98SE & WinStone 99 demo Test . |  |  |
| Result | ###### Pass | Duration | 48 Hours |

| Test Type |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 II – 350 + 192MB ( SDRAM 64 MB x 3 ) + On- Board IDE /AGP VGA /Sound + Maxtor 90845U3 + RTL 8139B + RCVDL 56ACF Modem Card |  |  |
| Test procedure | Win98SE & Winstone 98 demo Test . |  |  |
| Result | ###### Pass | Duration | 48 Hours |

| Test Type |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 - 233 + 192 MB (SDRAM 32 MB x 3 ) + On- Board IDE /AGP VGA / Sound + Quantum CX6400AT |  |  |
| Test procedure | Win98 & 3D MARK demo Test . |  |  |
| Result | ###### Pass | Duration | 48 Hours |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  9
http://www.acorp.com.tw

PART B. OPERATING SYSTEM ENVIRONMENT TEST

1.Microsoft Windows Operating System Test

  - Software Compatibility Test

1.1.1 Operating System Compatility

| O.S Name |  | ### Version | Publisher | Status | Hardware configuration / Hard disk & Controller |  |
| --- | --- | --- | --- | --- | --- | --- |
| # Windows 95 |  | V950B | Microsoft | Pass | On -Board IDE/AGP VGA/Sound + WD 28400 + PIONEER (36x) CD-ROM + Microsoft PS/2 Mouse |  |
| Windows 95 ( Chinese ) |  | V950B | Microsoft | Pass | On -Board IDE/AGP VGA/Sound + Quantum CX6400AT+ DCS C840P (40x) + Microsoft PS/2 Mouse |  |
| Windows 95 |  | V950B | Microsoft | Pass | AHA-2940UW + Quantum TM 3200 + TEAC CD-532E (32x) CD-ROM + logitech mouse |  |
| Windows 95 ( Chinese ) |  | V950B | Microsoft | Pass | Bus Logic SCSI + Quantum TM 3200 + VUEGO 632A (32X) CD-ROM + Microsoft PS/2 Mouse |  |
| # Windows98SE |  | 4.10 | Microsoft | Pass | On -Board IDE/VGA AGP/Sound + SDR-520H (DVD-ROM) + DTK PS/2 Mouse |  |
| Windows 98 (Chinese ) |  | 4.10 | Microsoft | Pass | On -Board IDE/AGP VGA + IBM DJNA-351010 + VUEGO 632A (32X) CD-ROM + NEC serial Mouse |  |
| Windows98SE |  | 4.10 | Microsoft | Pass | AHA-2940UW + Quantum TM 3200 + SDR-520H (DVD-ROM ) + DTK PS/2 Mouse |  |
| Windows 98 (Chinese ) |  | 4.10 | Microsoft | Pass | BusLogic SCSI + Quantum TM 3200 + VUEGO 632A (32X) CD-ROM + NEC serial Mouse |  |
| Windows 98SE |  | 4.10 | Microsoft | Pass | On-Board IDE/AGP VGA/Sound + Maxtor ST90845U3 + Creative Dx2 (DVD ROM) + Microsoft PS/2 Mouse |  |
| Windows98SE |  | 4.10 | Microsoft | Pass | On-Board IDE/AGP VGA/Sound + Quantum SE43A012 + PIONEER (36x) CD-ROM + AT&T PS/2 Mouse |  |
| Windows98SE |  | 4.10 | Microsoft | Pass | On-Board IDE/AGP VGA/Sound + WD 36400 + Creative Dx2 (DVD ROM) + Microsoft PS/2 Mouse |  |
| Windows98 (Chinese ) |  | 4.10 | Microsoft | Pass | On-Board IDE/AGP VGA/Sound + Quantum CR8400AT + TEAC CD-532E (32x) CD-ROM + DTK PS/2 Mouse |  |
| Windows98SE |  | 4.10 | Microsoft | Pass | On-Board IDE/AGP VGA/Sound + Seagate ST36531A + EPO (50x) CD-ROM + Microsoft PS/2 Mouse |  |
| Windows98SE |  | 4.10 | Microsoft | Pass | On-Board IDE/VGA AGP/Sound + WD 36400+ Pioneer (36X) CD-ROM + Microsoft PS/2 Mouse |  |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  10
http://www.acorp.com.tw

    - Application Software Compatibility

| Package Name |  |  | ## Version |  | Publisher | O. S. Environment |  | Status | Features tested |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Office 95 |  |  | Professional |  | Microsoft | Win95 V950B Win98SE V 4.10 |  | Pass | Install From CD-ROM and execute Power point, Excel , Schedule Plus , Word Access application sample Files Open and Save Function . |  |  |  |
| Office 97 |  |  | Professional |  | Microsoft | Win95 V 950B Win98SE V 4.10 |  | Pass | Install From CD-ROM and execute Power point ,Excel , Schedule Plus , Word Access application sample Files Open and Save Function . |  |  |  |
| Windows 95 PLUS |  |  | V1.0 |  | Microsoft | Win95 V 950B |  | Pass | Install From CD-ROM and using Screen Saver Themes Change , mouse Pointer and Events Sound Change Function |  |  |  |
| Acrobat Reader |  |  | V 3.01 |  | Adobe | Win95 V 950B Win98SE V 4.10 |  | Pass | Install From CD-ROM Open PDF Files |  |  |  |
| PhotoShop |  |  | V4.0 |  | Adobe | Win95 V 950B Win98SE V 4.10 |  | Pass | Install From CD-ROM Open Sample and Edit JPG Files |  |  |  |
| PhotoShop |  |  | V5.0 |  | Adobe | Win95 V 950B Win98SE V 4.10 |  | Pass | Install From CD-ROM Open Sample and Edit JPG Files |  |  |  |
| PhotoImpact |  |  | V4.0 |  | Ulead | Win98 V4.10 ( Chinese ) |  | Pass | Install From CD-ROM Open Sample and Edit JPG Files |  |  |  |
| Frontpage 98 |  |  | V3.0.2 |  | Microsoft | Win98SE V4.10 |  | Pass | Install From CD-ROM Edit Html Files. |  |  |  |
| #### Winstone 98 |  |  | V 1.0 |  | Ziff -Davis | Win95 V 950B Win98SE V4.10 |  | Pass | Run Business Winstone98 |  |  |  |
| #### Winstone 99 |  |  | V 1.0 |  | Ziff -Davis | Win95 V 950B Win98SE V4.10 |  | Pass | Run Business Winstone99 |  |  |  |
| 3D Winbench 98 |  |  | V1.0 |  | Ziff -Davis | Win95 V 950B Win98SE V4.10 |  | Pass | All Test |  |  |  |
| 3D Winbench 99 |  |  | V1.0 |  | Ziff -Davis | Win95 V 950B Win98SE V4.10 |  | Pass | All Test |  |  |  |
| #### Winbench 99 |  |  | V1.0 |  | Ziff -Davis | Win95 V 950B Win98SE V4.10 |  | Pass | All Test |  |  |  |
| #### WinTach |  |  | V1.2 |  | Texas Instruments | Win95 V950B Win98SE V4.10 |  | Pass | Run Demo TEST |  |  |  |
| #### Speedy |  |  | V1.1 |  | Rights Reserved | Win95 V950B Win98SE V4.10 |  | Pass | Run Demo TEST |  |  |  |
| #### Fina Reality |  |  | V1.01 |  | VNU Euripean Labs | Win95 V950B Win98SE V4.10 |  | Pass | Run Demo TEST |  |  |  |
| #### 3DMARK |  |  | V1.0 |  | Futuremark | Win95 V950B Win98SE V4.10 |  | Pass | Run Demo TEST |  |  |  |
| #### CHECK IT 98 |  |  | V6.00.35 |  | TouchStone | Win98SE V4.10 |  | Pass | Run Function TEST |  |  |  |
| SCSI Card Controller |  |  |  |  |  |  |  |  |  |  |  |  |
| Card Type |  | Card Model |  | H .D .D |  |  | Secondary IDE |  |  | O.S. Environment | Status |  |
|  |  |  |  |  |  |  | CD-ROM (Mode/Speed) |  |  |  |  |  |
| SCSI ( PCI ) |  | AHA-2940UW |  | Quantum TM3200S |  |  | Teac CD-532E (32X) |  |  | Win95 V 950B Win98SE V 4.10 | Pass |  |
|  |  | Bus Logic |  | Quantum TM3200S |  |  | VUEGO 632A (32X) |  |  | Win95 V 950B (Chinese ) Win98SE V 4.10 | Pass |  |
|  |  | INIC-950OP |  | IBM DDRS-34560 |  |  | DCS C840P 40x |  |  | Win98SE V 4.10 | Pass |  |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  11
http://www.acorp.com.tw

  - Hardware Compatibility Test

| IDE Controller |  |  |  |  |
| --- | --- | --- | --- | --- |
| Card type | ## Primary IDE | Secondary IDE | O.S. Environment | Status |
|  | HDD (Mode/Speed) | CD-ROM (Mode/Speed) |  |  |
| On-Board IDE ( PCI ) | WD 28400 UDMA 66 | VUEGO 632A (32X) | Win95 V 950B Win98SE V 4.10 | Pass |
|  | Fujitsu MPC3064AT UDMA 33 | Pioneer Dr-A14S (36X) | Win95 V 950B Win98SE V 4.10 Novell 3.12 | Pass |
|  | Maxtor 90845U3 UDMA 66 | SDR-520H (DVD 5X) | Win98SE V 4.10 Win2000 V 2128 | Pass |
|  | Seagate 6531 UDMA 33 | Teac CD-532E (32X) | Win95 V 950B (Chinese ) Win98SE V 4.10 | Pass |
|  | IBM DJNA-352500 UDMA 66 | Creative Dxr2 (DVD 2X) | Win95 V 950B Win98 V 4.10 (Chinese) | Pass |
|  | Seagate ST38421A UDMA 66 | Teac CD-532E (32X) | Win95 V 950B (Chinese ) Win98SE V 4.10 | Pass |
|  | Maxtor 90845U3 UDMA 66 | Creative Dxr2 (DVD 2X) | Win95 V 950B (Chinese-Simplified) Win98SE V 4.10 SCO UNIX | Pass |
|  | WD 36400 UDMA 33 | DCS C840P 40x | Win95 V 950B Win98SE V 4.10 Win2000 V 2128 | Pass |
|  | Quantum CR8400AT UDMA 66 | SDR-520H (DVD 5x) | Win95 V 950B Win98SE V 4.10 | Pass |
|  | Quantum SE43A012 UDMA 33 | Pionner Dr-A14S 36x | Win98 V 4.10 WinNT 5.0 B2 | Pass |
| Graphic Adapters |  |  |  |  |
| Card Type | Card Model | Peripheral Device | O.S. Environment | Status |
| VGA | S3 VIRGE /DX 4 MB | Quantum CR8400AT UDMA 66 | Win95 V 950B Win98 V 4.10 | Pass |
|  | S3 VIRGE /GX 4MB | WD 28400 UDMA 66 | Win95 V 950B Win NT V4.0 | Pass |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  12

http://www.acorp.com.tw

| Ethernet adapters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Card Type | Card Model | Peripheral Device | O.S. Environment | Status |
| Ethernet ( ISA ) | Acorp -908 | WD 28400 UDMA 66 | Win95 V 950B Win98SE V 4.10 Novell V 3.12 | Pass |
| Ethernet ( PCI ) | Acorp -970 | Quantum CX64A013 UDMA 66 | Win95 V 950B Win98SE V 4.10 WinNT V 4.0 | Pass |
|  | 3Com 3C905B -TX | IBM DTTA-351010 UDMA 33 | Win95 V 950B Win98SE V 4.10 | Pass |
|  | Intel 82558 | Maxtor 90845U3 UDMA 66 | Win98SE V 4.10 WinNT V 4.0 | Pass |
|  | RTL8139B/100MB | Seagate ST38421A UDMA 66 | Win98SE V 4.10 WinNT V 4.0 | Pass |
|  | RTL8029B/10MB | IBM DJNA-352500 UDMA 33 | Win98SE V 4.10 WinNT V 4.0 | Pass |
|  | 56 AMR CTR-21 | Quantum CR8400AT UDMA 66 | Win98SE V 4.10 | Pass |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  13
http://www.acorp.com.tw

- Microsoft Windows NT 4.0 Server , workstation

Operating System Test

  - Software Compatibility Test

2.1.1 Operating System Compatility

| O.S Name | ### Version | Publisher | Status | Hardware configuration / Hard disk & Controller |
| --- | --- | --- | --- | --- |
| # Windows NT Server | V 4.0 | Microsoft | Pass | On -Board IDE/AGP VGA/Sound + Seagate ST36421A + PIONEER (36x) CD-ROM + Microsoft PS/2 Mouse |
| Windows NT Server ( Chinese ) | V 4.0 | Microsoft | Pass | On -Board IDE/AGP VGA/Sound + IBM DTTA-351010 + DCS C840P (40x) CD-ROM + Genius Serial Mouse |
| Windows NT Workstation | V 4.0 | Microsoft | Pass | AHA-2940AW + Quantum TM 3200 + PIONEER (36x) CD-ROM + logitech mouse |
| Windows NT Workstaion ( Chinese ) | V 4.0 | Microsoft | Pass | BusLogic SCSI + Quantum TM 3200 + Teac CD-532E (32X) CD-ROM + Microsoft PS/2 Mouse |
| # Windows NT #### Server | V 5.0 Beta2 | Microsoft | Pass | On –Board IDE/A¦GP VGA/Sound + SDR-520H (DVD-ROM ) + DTK PS/2 Mouse |
| Windows NT Workstation | V 5.0 Beta2 | Microsoft | Pass | On -Board IDE/AGP VGA/Sound + Quantum CR8400AT + VUEGO 623A (23X) CD-ROM + NEC serial Mouse |
| Windows 2000 | V 5.0 Beta3 | Microsoft | Pass | On-Board IDE/VGA AGP/Sound + WD 36400 + PIONEER (36X) CD-ROM + Microsoft PS/2 Mouse |

2.1.2 Application Software Compatibility

| Package Name |  | ## Version | Publisher | O. S. Environment | Status | Features tested |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Winstone 98 |  | V 1.0 | Ziff -Davis | NT Server V 4.0 NT. WKS V 4.0 | Pass | Install From CD-ROM an Run business Winstone 98 High-End Winstone 98 Demo |  |
| Winstone 99 |  | V 1.0 | Ziff -Davis | NT Server V 4.0 NT. WKS V 4.0 | Pass | Install From CD-ROM an Run business Winstone 98 High-End Winstone 98 Demo |  |
| Microsoft Office 97 |  | Profession | Microsoft | NT Server V 4.0 NT. WKS V 4.0 | Pass | Install From CD-ROM and execute Powerpoint ,Excel , Schedule Plus , Word Access application sample Files Open and Save Function |  |
| WinTach |  | V 1.2 | Texas Inctruments | NT Server V 4.0 NT. WKS V 4.0 | Pass | Install From Floppy and Run TEST. |  |
| Final Reality |  | V 1.01 | VNU European Labs | NT Server V 4.0 NT. WKS V 4.0 | Pass | Install From Hard Disk and Run Demo TEST. |  |
| Speedy |  | V 1.1 | Rights Reserved | NT Server V 4.0 NT. WKS V 4.0 | Pass | Install From Hard Disk and Run TEST. |  |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  14
http://www.acorp.com.tw

    - Hardware Compatibility Test

| IDE Controller |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Card type | ## Primary IDE | Secondary IDE | O.S. Environment |  | Status |
|  | HDD (Mode/Speed) | CD-ROM (Mode/Speed) |  |  |  |
| On-Board IDE ( PCI ) | Seagate ST36531A UDMA 33 | Teac CD-532E (32X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | Seagate ST36421A UDMA 33 | Pioneer Dr-A14S (36X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | IBM DTTA-351010 UDMA 33 | VUEGO 632A (32X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | WD 28400 UDMA 66 | SDR-520H (DVD 5X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | WD 36400 UDMA 33 | Creative Dxr2 (DVD 2X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | Seagate ST36421A UDMA 33 | Teac CD-532E (32X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | IBM DTTA-351010 UDMA 33 | DCS C840P 40x | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | WD 36400 (DMA 33) | VUEGO 632A (32X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | WD 28400 UDMA 66 | SDR-520H (DVD 5X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | Quantum SE43A012 UDMA 33 | Pioneer Dr-A14S (36X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
|  | Maxtor 90845U3 UDMA 66 | Creative Dxr2 (DVD 2X) | NT Server V 4.0 NT. WKS V 4.0 |  | Pass |
| IDE Controller |  |  |  |  |  |
| Card type | ## Primary IDE | Secondary IDE |  | O.S. Environment | Status |
|  | HDD (Mode/Speed) | HDD (Mode/Speed) |  |  |  |
| On-Board IDE ( PCI ) | Quantum CX6400AT UDMA 66 | Quantum CR8400AT UDMA 66 |  | DOS V 6.22 Win95 V 950 B | Pass |
|  | Fujitsu MPC3064AT UDMA 33 | Quantum SE43A012 UDMA 33 |  | Win95 V 950 B Win98 V 4.10 | Pass |
|  | IBM DTTA- 351010 UDMA 33 | WD 28400 UDMA 66 |  | Win98 V 4.10 OS2 WARP 4 | Pass |
|  | WD 28400 UDMA 66 | IBM DTTA-351010 UDMA 33 |  | DOS V 6.22 WinNT V 4.0 | Pass |
|  | Fujitsu MPC3043AT UDMA 33 | WD 36400 UDMA 33 |  | Win98 V 4.10 WinNT V 4.0 | Pass |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  15
http://www.acorp.com.tw

| SCSI Card Controller |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Card Type | Card Model | H .D .D | Secondary IDE | O.S. Environment | Status |
|  |  |  | CD-ROM (Mode/Speed) |  |  |
| SCSI ( PCI ) | AHA-2940UW | Quantum TM3200S | Teac CD-532E (32X) | NT Server V 4.0 NT. WKS V 4.0 | Pass |
|  | Bus Logic | Quantum TM3200S | DCS C840P 40x | NT Server V 4.0 NT. WKS V 4.0 | Pass |
|  | INIC-950OP | Quantum PXO4L011-4.5L | Pioneer Dr-A14S (36X) | NT Server V 4.0 NT. WKS V 4.0 | Pass |

| Ethernet adapters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Card Type | Card Model | Peripheral Device | O.S. Environment | Status |
| Ethernet ( ISA ) | Acorp -908 | WD 28400 UDMA 66 | NT Server V 4.0 NT. WKS V 4.0 | Pass |
| Ethernet ( PCI ) | Intel 558 LAN | WD 36400 UDMA 33 | NT Server V 4.0 NT. WKS V 4.0 | Pass |
|  | Acorp -970 | Seagate ST36421A DMA 33 | NT Server V 4.0 NT. WKS V 4.0 | Pass |
|  | 3Com 905B-TX | IBN DTMST-351010 UDMA 66 | NT Server V 4.0 NT. WKS V 4.0 | Pass |
|  | RTL8139B/100 MB | Quantum CX64A013 UDMA 66 | NT Server V 4.0 NT. WKS V 4.0 | Pass |
|  | RTL8029B/10 MB | Quantum CX64A013 UDMA 66 | NT Server V 4.0 NT. WKS V 4.0 | Pass |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  16
http://www.acorp.com.tw

3. Operating System Fied (stress) Test

| Test Type Windows NT 4.0 Server Operating System |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 III - 450+ 512 MB ( SDRAM 256 MB x 2 ) + On- Board IDE /AGP VGA /Sound + Quantum CR8400AT + Intel 82558 LAN |  |  |
| Test procedure | High-End Winstone 99 demo Test . |  |  |
| Result | ###### Pass | Duration | 48 Hours |

| Test Type Windows 98 Operating System |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 III – 450 + 384 MB (SDRAM 128 MB x 3 ) + On- Board IDE /AGP VGA /Sound + IBM DTTA-351010 + RTL 8139B LAN |  |  |
| Test procedure | Business Winstone 99 demo Test . |  |  |
| Result | ###### Pass | Duration | 48 Hours |

| Test Type Windows NT 4.0 workstation Operating System |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 II - 350 + 256 MB (SDRAM 128 MB x 2 ) + On- Board IDE/AGP VGA /Sound + Quantum + 3Com 905B-TX |  |  |
| Test procedure | High-End WinBench 99 demo Test . |  |  |
| Result | ###### Pass | Duration | 72 Hours |

| Test Type Windows 95 Operating System |  |  |  |
| --- | --- | --- | --- |
| Configuration | AMD K6 – 233 + 192 MB (SDRAM 64 MB x 3 ) + On- Board IDE/AGP VGA /Sound + Seagate ST38421A + RTL 8029B LAN |  |  |
| Test procedure | Business Wintone 98 demo Test . |  |  |
| Result | ###### Pass | Duration | ###### 48 Hours |

ACORP ELECTRONICS CORPORATON.                                                                                    Page  17
http : // www.acorp.com.tw

Manager :                  Supervisor :

Tester :

ACORP ELECTRONICS CORPORATON.                                                                                    Page  18
http://www.acorp.com.tw