### AOpen AX4T BIOS Release Notice

| Item Number | Description | # Date | Fixed BIOS Version | ## Solution | Comment |
| --- | --- | --- | --- | --- | --- |
| 030 | Audio Subsystem ID | Apr.30.2001 | #### R1.04 | SSID=0053 |  |
| 029 | Sub-Vendor ID default value | Apr.30.2001 | #### R1.04 | Intel default is 8086 |  |
| 028 | Fixed password lost issue (R1.03 fail but R1.02 ok). | Apr.30.2001 | #### R1.04 | The CMOS was conflict, remove it ok! |  |
| 027 | Helo LAN loop back test fail when use SIS 6326 AGP card | Apr.30.2001 | #### R1.04 | When assign IRQ, set Audio IRQ9 first. |  |
| 026 | Helo test fail when use SIS 6326AGP card. | Mar.30.2001 | #### R1.03 | Set all PCI IRQ defaults to IRQ11 |  |
| 025 | Update the “AC PWR Auto Recovery” failure function | Mar.30.2001 | #### R1.03 | Former-Sts => If pre-state is off, then turn off power If pre-state is on, then keep power on. |  |
| 024 | 1. Changed Record_PWRON_Status | Mar.14.2001 | #### R1.02 | 1.Always off => never turn on power 2.Always on => always turn on power. 3.Former-Sts => 3.1 if pre-state is off, then turn off power 3.2 if pre-state is on, then keep power on |  |
| 023 | Sub-Vendor ID | Mar. 01 2001 | R1.01 | SVID=A0A0 |  |
| 022 | Default values | Mar. 01.2001 | R1.01 | Follow AX4T101-2.doc |  |
| 021 | Showdown temperature default | Feb. 27 2001 | R1.01 | Disabled |  |
| 020 | Ultra DMA default | Feb. 27 2001 | R1.01 | Auto |  |
| 019 | Gate A20 default | Feb. 27 2001 | R1.01 | Setup default=Normal; Turbo default=Fast |  |
| 018 | Changed item message | Feb. 26 2001 | R1.00 |  |  |
| 017 | Changed default value | Feb. 26 2001 | R1.00 |  |  |
| 016 | Speed up booting time | Feb. 26 2001 | R1.00 |  |  |
| 015 | Added to RamBus clock | Feb. 26 2001 | R1.00 |  |  |
| 014 | To solve PnP enable/disable test in HCT9.502 in WIN2000 | Feb. 26 2001 | R1.00 |  |  |
| 013 | Fix the issue of [System stopped with <AOpen 48X CD-ROM + LG CD-RW> or <Hitachi DVD + LG CD-RW>] | Feb. 19, 2001 | R0.04 |  |  |
| 012 | Fix the issue of [USB keyboard resumed too slowly from ACPI S4 mode] | Feb. 19, 2001 | R0.04 |  |  |
| 011 | Implement the PC Health function | Feb. 19, 2001 | R0.04 |  |  |
| 010 | Implement PC600/800 RIMMs auto detection and setting | Feb. 14, 2001 | R0.03 |  | Needs H/W rework |
| 009 | Change the default setting | Feb. 13, 2001 | R0.02 | As AOpen’s request |  |
| 008 | Implement Doctor LED | Feb. 13, 2001 | R0.02 |  |  |
| 007 | Display Product Name, BIOS Version, Release Date and the Manufacturer during POST | Feb. 12, 2001 | R0.01 | e.g. AX4T R0.01 Feb.12.2001 AOpen Inc. |  |
| 006 | Implement AOpen specific Setup Menu | Feb. 12, 2001 | R0.01 |  |  |
| 005 | Enable AC Power Failure Recovery function | Feb. 12, 2001 | R0.01 |  |  |
| 004 | Implement ACPI S1 mode LED | Feb. 12, 2001 | R0.01 | Green + Orange |  |
| 003 | Enable PS/2 keyboard/mouse wakeup event for ACPI S1 mode | Feb. 12, 2001 | R0.01 |  |  |
| 002 | Support PC600 RIMMs | Feb. 12, 2001 | R0.01 | Implement the option in BIOS Setup |  |
| 001 | ACPI S3 mode resumed fail with nVIDIA GForce2 VGA chip (LeadTech NV11). | Feb. 12, 2001 | R0.01 |  |  |