# AGP Card Compatibility Test

Test By: Beta Chung

Date: 03/23/2001

Purpose:	Check AGP Card Compatibility and 3D Function.

| Motherboard: | MX34 (686B) v95A/95B |
| --- | --- |
| CPU: | Intel PC-PGQ Coppermine 850 / Celeron 700MHz |
| Cache: | On CPU |
| Memory: | NEC PC-133 D45128163G5-A75-9JF 128M |
| System BIOS: | Award R1.14 |
| BIOS CheckSum | AB05h |
| Hard Disk: | Quantum Fireball lct 10 15GB |
| HDD Driver: |  |
| VGA Card: | ATI for VGA Device 02h(46) 03h(56) |
| VGA Driver: | 4.12.6292 |
| Sound Card: | ON Board Codec (ad1885) |
| Sound Driver: |  |
| Modem Card: |  |
| Modem Driver: |  |
| Network Card: | 3COM 3C905-TX-M |
| Network Driver: |  |
| Floppy Drive: | Panasonic 1.44M |
| CDROM: | Aopen 32X CDROM |
| CDROM Driver: |  |
| Serial Mouse: |  |
| PS/2 Mouse: | Aopen PS/2 Mouse |
| USB Keyboard: |  |
| USB Mouse: |  |
| Power Supply: | Aopen FSP300-60BT / FSP250-60GTA |

Procedure:

- BIOS load default setup, and set AGP 4X.

- Setup Win 98 SE, and install all on-board device driver.

- Enable DMA mode.

- Install Direct X latest version.

- Boot from Win 98 SE, check PnP function.

- Test all different resolution.

- Run PCIlist to check AGP ratio.

- Run 3D WinBench 2000 / 3D WinMark @1024x768 all setting use 32bit or highest setting value.

- Run 3D Mark 2000 or 2001/ 3D Marks @1024x768 all setting use 32bit or highest setting value.

- Test TV, DVI , TwinView output if support.

- Shutdown to DOS and exit to Windows Test.

Necessary Sample Vendor :

- ATI, Matrox, 3dfx, ASUS, Leadtek, Elsa, AOpen, Canopus,

- Gigabyte/MSI, Creative, Triplex

- Prolink, Ennyah, TST, Gainward, APM

| AGP model/ Vendor | Chipset model | Driver Version | AGP Mode | 3D Mark |  |  |  | 3D WinBench 2000 | Result | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 99 MAX | 2000 v1.1 | 2001 | Over Night |  |  |  |
| ATI Rage Magnum | ATI Rage 128 GL | 4.12.6292 | 2x | 3882 | 1657 | 602 |  | 24.6 | Pass |  |
| ATI All-In-Wonder 128 AGP 16 | ATI Rage 128 GL |  | 2x | 4256 | 1763 | 780 |  | 25.4 | Pass |  |
| ATI Rage Fury MAXX | ATI Rage 128 Pro *2 | 4.12.6292 | 2x | 4290 | 1777 | 902 |  | 25.5 | Pass |  |
| ATI Rage Fury | ATI Rage 128 GL | 4.12.6292 | 2x | 3503 | 1421 | 530 |  | 21.7 | Pass |  |

Note:

- Must turn on the “V-sync” to run 3D WinBench 2000.(chip’s limitation)

- Run 3D WinBench 2000 must set “Color depth = High Color(16bit)” and “Desired Width = 16bit” ( chip’s limitation)