PCI64X.DOC

PCI64X.EXE

-----------

A utility, PCI64X.EXE, for programming the PCI-064x is included

in the distribution.  PCI64X.EXE allows you to experiment with

different Enhanced IDE timing modes, set-up the PCI-064x's timing,

and display vital information about your drives.  PCI-064x is

compatible with both PCI-bus and VL-bus.  Below is a command

description, followed by a tutorial example.

WARNING: PCI64X.EXE is for power users only!  Do not use it to alter

your drive timings unless you are thoroughly familiar with

the ATA-2 Specification.  SERIOUS DATA LOSS MAY OCCUR IF THE

PROGRAM IS IMPROPERLY USED!

Form: PCI64X D[riveMode] drv mode

where

drv = target drive (0-3)

mode= desired drive PIO mode (0-5)

Use:  Most drives revert to slower timing modes at power-up.  This

command enables setting up a drive to a particular PIO timing

mode.  After running DriveMode, the TimingMode command

must be used to set-up the proper timing on the PCI-064x chip.

To set-up both the drive and PCI-064x in a single command,

see the Mode command below.  To determine which timing modes

your particular drive supports, use the Info command.

Example:

PCI64X M 0 3 : set Drive 0 to Mode 3 operation

Form: PCI64X I[nfo] drv

where:

drv = target drive (0-3)

Use:  Displays info gathered from the IDE IDENTIFY DRIVE command in

tabular form.  This command is useful for determining which

Advanced PIO modes your drives support.

Example:

PCI64X I 2 : display info about drive 2

Form: PCI64X M[ode] drv mode

where

drv = target drive (0-3)

mode= desired drive PIO mode (0-5)

Use:  Most drives revert to slower timing modes at power-up.  This

command enables advanced PIO timing modes on both the drive and

the PCI-064x.  To determine which timing modes your particular

drive supports, use the Info command.

Example:

PCI64X M 0 3 : set Drive 0 to Mode 3 operation

Form:  PCI64X R[egister] [idx val]

idx = register index in hexadecimal (51-59h) to set to val

val = hexadecimal value to set register idx

Use:  Allows you to manually set the PCI-064x registers.  Set

register index <idx> to <val>.  If <idx> and <val> are not

specified, the current register settings are displayed.

Example:

PCI64X R 52 45 : set register 52h to 45h

Form:  PCI64X S[peed] mhz

where:

mhz = local bus speed in MHz (20,25,33,40,50)

Use: Specify system local bus speed.  The program must know the

current local bus speed in order for the Mode command

to work properly.  This information is stored in PCI64X.DAT.

Example:

PCI64X S 33 : specify 33 MHz PCI or VL bus speed.

Form: PCI64X T[imingMode] drv mode

where

drv = target drive (0-3)

mode= desired drive PIO mode (0-5)

Use:  Most drives revert to slower timing modes at power-up.  This

command enables advanced PIO timing modes on the the PCI-064x.

Unlike the Mode command, the TimingMode command DOES NOT setup

the drive accordingly.  You must run the DriveMode command before

running the TimingMode command.  To determine which timing modes

your particular drive supports, use the Info command.

WARNING:  TimingMode is for diagnostic purposes only.  Setting

the PCI-064x timing faster than what you drive can support

may result in data corruption or system lock-up.

Example:

PCI64X T 0 3 : set PCI-064x timing for Drive 0 to Mode 3 operation

Tutorial Example

----------------

Say you have a new drive from FOOBAR Electronics hooked up as

the master on Primary IDE.  First, find out which timing modes it

supports:

pci64x i 0

---------------------------------------------------------------

CMD PCI-064x IDE Configuration Utility V1.04     17 March 1994

Copyright 1993  CMD Technology, Inc.     ALL RIGHTS RESERVED

Local Bus Speed: 50 MHz

Drive 0: Foobar Electronics IDE-345

Serial: XXFOBR000004579       Firmware Rev: 92919022

Geometry:

cylinders 489, heads 11, sectors/track 35

unfmt bytes/track 57600, unfmt bytes/sector 600

Buffer size: 64 KB, Max R/W Mult Count: 32 sectors

Current Translation:

cylinders 0, heads 0, sectors/track 0, total sectors 0

Default PIO Transfer Mode: 2

Advanced PIO Transfer Mode(s): 3, 4

---------------------------------------------------------------

From the above information, we can see that the drive boots up in

Mode 2, and it also supports Mode 3 and 4.  If we want to try out

Mode 3, we must set the drive and PCI-064x to Mode 3 timings:

pci64x m 0 3

---------------------------------------------------------------

CMD PCI-064x IDE Configuration Utility V1.0     10 December 1993

Copyright 1993  CMD Technology, Inc.     ALL RIGHTS RESERVED

Local Bus Speed: 50 MHz

Drive 0: Foobar Electronics IDE-345

Setting drive to Mode 3

Setting PCI-064x timing to PIO Mode 3

---------------------------------------------------------------

Further performance improvements can be achieved by speeding up the

host-side timing.  As described in the PCI-064x PCI-to-IDE Chip

Manual, Register index 51 contains bits which control host timing.

The fastest host timing is obtained with value 23h:

pci64x r 51 23

For more information, see PIOMODES.DOC.