# CL50-LX Intel® 440LX Celeron™ PPGA Processor Baby AT System Board PROCESSOR

300A/333/366/400/433MHz Celeron™ PPGA processor

Intel® 440LX AGP set

8MB to 384MB using unbuffered DIMMs

Three 168-pin DIMM sockets

Uses SDRAM (10/12/13ns), 3.3V

ECC supported (uses x72 DRAM)

Award BIOS, Windows® 95 Plug and Play compatible

Flash EPROM for easy BIOS upgrades (2MB)

Supports DMI 2.0 function

Selectable CPU bus frequency (FSB) - 60/66/68/75/83MHz

Auto detect CPU's voltage ID

Quickly boot up Windows or other operating systems

Y2K compliant

System power management supported

CPU stopped clock control

Hardware supports SMI green mode

Microsoft®/Intel® APM 1.2 compliant

CPU and chassis fans will automatically turn off once the system enters the Suspend mode

Ultra DMA/33 supported (Synchronous DMA mode -data transfer rate up to a maximum of 33MB/sec.)

PIO Mode 3 and Mode 4 Enhanced IDE (data transfer rate up to 16.6MB/sec.)

Bus mastering reduces CPU utilization during disk transfer

ATAPI CD-ROM, LS-120 and ZIP supported

Two NS16C550A-compatible high speed UARTs

One SPP/ECP/EPP parallel port

Supports up to 2.88MB floppy drives

Intel® Socket 370

## Equipped with a switching voltage regulator that automatically detects 1.30V to 2.0V

AGP is an interface designed to support high performance 3D graphics cards

Supports up to 533MB/sec. bandwidth for 3D graphics applications

2 connectors for external USB ports

1 connector for IrDA interface

2 serial ports

1 parallel port

2 IDE connectors

1 floppy connector

1 PS/2 mouse port

1 PS/2 or AT keyboard port

1 12-pin standard AT power supply connector

3 fan connectors for CPU, chassis and AGP fans

1 dedicated AGP slot

3 dedicated PCI slot

1 dedicated 16-bit ISA slots

1 shared PCI/ISA slot

4 layers, Baby AT form factor

26.5cm (10.43") x 22cm (8.66")