P2BLX

Intel® 440LX Pentium® II Processor, Baby AT System Board

Feature

| Accelerated Graphics Port (AGP) with up to 533MB/sec. bandwidth |  |
| --- | --- |
| Supports ACPI specification (ATX power only) |  |
| Ultra synchronous DMA mode-33MB/sec. maximum |  |
| Supports Modem Wake-Up/Ring-on (ATX power only) |  |
| Supports Wake-On-LAN/Keyboard/Mouse function (ATX power only) |  |
| RTC timer to power on or wake-up the system (ATX power only) |  |

Specifications

PROCESSOR

233/266/300/333MHz Pentium® II processor

266/300/300A/333MHz Celeron™ processor

Intel® 440LX AGP set

SYSTEM MEMORY

8MB to 384MB using unbuffered SDRAM or EDO memory

three 168-pin DIMM sockets (unbuffered)

Uses x64/x72 EDO (50/60ns) or SDRAM (10/12/13ns), 3.3V

BIOS

Award BIOS, Windows® 95 Plug and Play compatible

Flash EPROM for easy BIOS upgrades

Supports DMI 2.0 function

ENERGY EFFICIENT DESIGN

Supports ACPI specification and OS Directed Power Management (ATX power supply only)

Wake-On-Events include (ATX power supply only): - RTC timer to power on the system - Modem Ring-On - Wake-On-LAN - Wake-On-keyboard - Wake-on-mouse

System power management supported

CPU stopped clock control

Hardware supports SMI green mode

Microsoft®/Intel® APM 1.2 compliant

Soft Power supported - ACPI v1.0a specification (ATX power supply only)

DAMAGE FREE INTELLIGENCE (optional)

Monitors processor temperature and overheat alarm

Monitors 1.5V/3.3V/5V/12V voltages and failure alarm

Monitors processor/system fan speed and failure alarm

Fan on/off control includes CPU fan, AGP fan, and system fan - saves energy, prevent system overheat and prolong fan life

Three types of sound for different failure alarm

Read back capability that displays accurate temperature, voltage and fan speed

PCI IDE INTERFACE

Ultra DMA/33 supported (Synchronous DMA mode - data transfer rate up to a maximum of 33MB/sec.)

PIO Mode 3 and Mode 4 Enhanced IDE (data transfer rate up to 16.6MB/sec.)

Bus mastering reduces CPU utilization during disk transfer

ATAPI CD-ROM, LS-120 and ZIP supported

INTEGRATED I/O

Two NS16C550A-compatible high speed UARTs

One SPP/ECP/EPP parallel port

Supports up to 2.88MB floppy drives

PROCESSOR SLOT

Intel® Slot 1 connector (242-pin processor slot)

Equipped with a switching voltage regulator that automatically detects 1.8V to 3.5V

ACCELERATED GRAPHICS PORT (A.G.P.)

AGP is an interface designed to support high performance 3D graphics cards

Supports up to 533MB/sec. bandwidth for 3D graphics applications

CONNECTORS

2 connectors for external USB ports

1 connector for IrDA interface

2 serial ports

1 parallel port

2 IDE connectors

1 floppy connector

1 PS/2 mouse port

1 PS/2 or At keyboard port

1 20-pin ATX power supply connector

1 12-pin standard AT power supply connector

1 Wake-on-LAN connector

1 SB-LINK connector

EXPANSION SLOTS

1 dedicated AGP slot

3 dedicated PCI slot

1 dedicated 16-bit ISA slots

1 shared PCI/ISA slot

PCB

4 layers, Baby AT form factor

26.5cm (10.43") x 22cm (8.66")