| PB60-VB VIA® BX Plus Pentium® II Processor ATX System Board |  |
| --- | --- |
| 233/66MHz, 266/66MHz, 300/66MHz, 333/66MHz, 350/100MHz, 400/100MHz, 450/100MHz or 500/100MHz Pentium® II processor |  |
| 266/66MHz Pentium® II processor |  |
| 266/300/300A/333/366MHz Celeron™ processor |  |
| VIA® BX Plus AGP set |  |
| 8MB to 384MB memory using unbuffered DIMMs |  |
| Three 168-pin DIMM sockets |  |
| Uses x64 or x72 PC SDRAM, 3.3V - PC-66 SDRAM DIMM for 66/100MHz FSB processor - PC-100 SDRAM DIMM for 100MHz FSB processors |  |
| ECC supported (uses x72 PC SDRAM DIMM) |  |
| Award BIOS, Windows® 95 Plug and Play compatible |  |
| Flash EPROM for easy BIOS upgrades |  |
| Supports DMI 2.0 function |  |
| Supports ACPI specification and OS Directed Power Management |  |
| Wake-On-Events include: - RTC timer to power-on the system - Modem ring-on - Wake-On-LAN - Wake-On-keyboard/Wake-On-Mouse |  |
| System power management supported |  |
| CPU stopped clock control |  |
| Hardware supports SMI green mode |  |
| Microsoft®/Intel® APM 1.2 compliant |  |
| Soft Power supported - ACPI v1.0a specification |  |
| Monitors processor/system temperature and overheat alarm |  |
| Monitors 1.5V/3.3V/5V/12V voltages and failure alarm |  |
| Monitors processor/system fan speed and failure alarm |  |
| Automatic processor fan control to save energy, prevent system overheat, prolong fan life and implement silent system |  |
| Three types of sound for different failure alarm |  |
| Read back capability that displays accurate temperature, voltage and fan speed. |  |
| Ultra DMA/66 supported (Synchronous, DMA mode - data transfer rate up to a 33MB/sec.) |  |
| PIO Mode 3 and Mode 4 Enhanced IDE (data transfer rate up to 16.6MB/sec.) |  |
| Bus mastering reduces CPU utilization during disk transfer |  |
| ATAPI CD-ROM, LS-120 and ZIP supported |  |
| Two NS16C550A-compatible high speed UARTs |  |
| One SPP/ECP/EPP parallel port |  |
| Supports up to 2.88MB floppy drives |  |
| Intel® Slot 1 connector (242-pin processor slot) |  |
| Equipped with a switching voltage regulator that automatically detects 1.8V to 3.5V |  |
| AGP is an interface designed to support high performance 3D graphics cards |  |
| Supports up to 533MB/sec. bandwidth for 3D graphics applications |  |
| 2 USB ports |  |
| 1 DB-9 serial ports |  |
| 1 DB-25 parallel port |  |
| 1 mini-DIN-6 PS/2 mouse port |  |
| 1 mini-DIN-6 PS/2 keyboard port |  |
| 1 connector for IrDA interface |  |
| 2 IDE connectors |  |
| 1 floppy connector |  |
| 1 20-pin ATX power supply connector |  |
| 1 3-pin WOL (Wake-ON-Lan) connector |  |
| 1 SB-LINK connector |  |
| 3 fan connectors for CPU, chassis and AGP fans |  |
| 1 dedicated AGP slot |  |
| 3 dedicated PCI slot |  |
| 2 dedicated 16-bit ISA slots |  |
| 1 shared PCI/ISA slot |  |
| 4 layers, ATX form factor |  |
| 30.48cm (12") x 19cm (7.48") |  |