82430I PCI BUS Mother Board

Jumper Setting

# CLOCK SELECT

| clock | 50MHz | 60MHz | 66MHz |
| --- | --- | --- | --- |
| JP13 | ON | ON | ON |
| JP14 | OFF | OFF | ON |
| JP15 | OFF | ON | ON |
| CPU TYPE | Pentium 75MHz M1 6X86 100MHz AMD K5 75MHz | Pentium 90MHz Pentium 120MHz Pentium 150MHz Pentium 180MHz M1 6X86 120MHz | Pentium 100MHz Pentium 133MHz Pentium 166MHz Pentium 200MHz |

If you want to use Pentium 120/133/ MHz Please reference JP25, JP29 Setting.

# BUS FREQUENCY

| JP29 | JP25 | CPU-TO-BUS CLOCK | note |
| --- | --- | --- | --- |
| OFF | OFF | 3 / 2 (X1.5 default) | For AMD K586-P75 |
| OFF | ON | 2 / 1 (X2) | For Pentium 120/133 M1 6X86 100/120 |
| ON | ON | 5 / 2 (X2.5) | For Pentium 150/166 |
| ON | OFF | 3 / 1 (X3) | For Pentium 180 / 200 |

The CPU-to-BUS frequency ratio is defined by bus frequency

# CACHE MEMORY CONFIGURATION

| L2 CACHE SIZE | 256K | 512K | PBSRAM |
| --- | --- | --- | --- |
| JP3 | 2-3 | 1-2 | 2-3 |
| JP20 | 1-2 | 2-3 | 1-2 |
| JP21 | 1-2 | 2-3 | 1-2 |
| TAG SIZE | 8K*8 16K*8 32K*8 | 16K*8 32K*8 | 8K*8 16K*8 32K*8 |

# CPU INTERNAL CACHE CONTROL

| L1 Cache | W / T (Write Through) | W / B (Write Back) |
| --- | --- | --- |
| JP23 | ON | OFF (default) |

CPU NA#  (Next Address)

| Pin Line | Disable | Enable |
| --- | --- | --- |
| JP24 | OFF | ON (default) |

VR/VRE VOLTAGE SPEC.

| JP32 | CPU Voltage |
| --- | --- |
| 1-2 3-4 | 3.3V (default) |
| 1-2 | 3.45V |
| ALL OFF | 3.6V |

Note: Standard Spec is 3.135-3.465V

VR     Spec is 3.300-3.465V

VRE    Spec is 3.450-3.600V

Please reference INTEL Pentium Processor spec.

# BIOS Option

| Type | JP16 | JP17 |
| --- | --- | --- |
| EPROM | 1-2 | 1-2 |
| 5V FLASH ROM | 1-2 | 2-3 |
| 12V FLASH ROM | 2-3 | 2-3 |

# I/O JUMPER SETTING

| ON Board I/O Chipset Function | Enable | Disable |
| --- | --- | --- |
| JP6 | 1-2 | 2-3 |

# AT BUS CLOCK SETTING

| JP12 | AT CLOCK |
| --- | --- |
| 1-2 | PCI CLOCK / 3 |
| 2-3 (default) | PCI CLOCK / 4 |

OTHER JUMPER SETTING

| JUMPER | SETTING | FUNCTION |
| --- | --- | --- |
| TB LED |  | Turbo LED |
| HDD LED |  | Hard disk detect LED |
| RESET |  | Reset switch |
| SPEAKER |  | Speaker connector |
| KEY LOCK |  | Keyboard lock |
| JP10 | 1-2 (default) | CPU & system CLK=PCIX2 |
|  | 2-3 | CPU & system CLK=PCI |
| J11 |  | External battery connector |
| JP18 | Break function | Connect to case turbo push button as triggle switch (on then off), switch that system go into green mode immedeatly by hardware |
| JP11 | 1-2 (default) | CPU clock voltage 5V |
|  | 2-3 | CPU clock voltage 3V |
| JP9 | On | For DALLAS CMOS DATA clear |
| JP30 | 2-3 | For RTC power pin |
| JP30 | On | JP30 (off) first and JP31 (on) Then clear RTC CMOS data |
| J1 |  | External Ps2 mouse connector |
| JP4,JP5 JP26,JP27 | On (default) | For P54C CPU with VRM module |
|  | Off | CPU with VRM module |
| JP1,JP2 | 1-2 (default) | SRAM 5v voltage |
|  | 2-3 | SRAM 3.3V voltage |
|  | 1-2 (default) | ASYNC SRAM |
| JP325 | 2-. | PBSRAM board |