메인 보드 점퍼 설정

1. CPU FSB(Front Side Bus) 설정: JP4

O N: 100MHz(EV6 bus : 200MHz)

OFF: 133MHz(EV6 bus : 266MHz)

*ON : 점퍼을 끼운상태, OFF: 점퍼을 뺀상태.

2. CPU 배율(Ratio) 설정(10핀): SW1

자동 배율설정시 : 1번점퍼을 OFF. (이후(2-5)점퍼 설정은 적용되지않습니다.)

수동 배율설정시 : 1번점퍼을 ON.  (이후(2-5)점퍼 설정따라 배율이 적용됩니다.)

| 배율 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5.0 | ON | ON | ON | OFF | ON |
| 5.5 | ON | OFF | ON | OFF | ON |
| 6.0 | ON | ON | OFF | OFF | ON |
| 6.5 | ON | OFF | OFF | OFF | ON |
| 7.0 | ON | ON | ON | ON | OFF |
| 7.5 | ON | OFF | ON | ON | OFF |
| 8.0 | ON | ON | OFF | ON | OFF |
| 8.5 | ON | OFF | OFF | ON | OFF |
| 9.0 | ON | ON | ON | OFF | OFF |
| 9.5 | ON | OFF | ON | OFF | OFF |
| 10.0 | ON | ON | OFF | OFF | OFF |
| 10.5 | ON | OFF | OFF | OFF | OFF |
| 11.0 | ON | ON | ON | ON | ON |
| 11.5 | ON | OFF | ON | ON | ON |
| 12.0 | ON | ON | OFF | ON | ON |
| 12.5 | ON | OFF | OFF | ON | ON |

*ON : 점퍼을 끼운상태, OFF: 점퍼을 뺀상태.

3. AMD ATHLON XP  CPU FSB & CPU Ratio.

| 모델명 | CLOCK (GHz) | CPU FSB / EV6 BUS(MHz) | CPU RATIO(x) |
| --- | --- | --- | --- |
| AMD ATHLON 1500+ | 1.33 | 133 / 266 | 10.0 |
| AMD ATHLON 1600+ | 1.40 | 133 / 266 | 10.5 |
| AMD ATHLON 1700+ | 1.47 | 133 / 266 | 11.0 |
| AMD ATHLON 1800+ | 1.53 | 133 / 266 | 11.5 |

4. CMOS RAM Clear설정: JBAT

1-2: ON        기본설정

2-3: ON        CMOS RAM Clear설정(기본설정값으로 변경)

*전원 OFF상태에서 점퍼을 2-3접점하고 다시 1-2으로 변경한다.

5. KeyBoard Power on 설정: JP1

1-2: ON        기본설정(KeyBoard Power on을 사용안함)

2-3: ON        KeyBoard Power on 사용.

*전원 OFF상태에서 점퍼을 2-3접점하면 된다.

6. 드라이버 설치.

- VIA 4 IN 1 드라이버을 설치한다.

- 오디오 드라이버을 설치한다.

- 필요에 따라 기타 기능용 프로그램을 설치한다.