Board Name:	M4P

Part #:		09-00190-xx

Size:			Baby AT (8 1/2” x 13”)			Chip Sets:		Intel 82420 PCI set

SMC 665 Super I/O

Expansion Slots:		5 16-bit ISA				Cache Options:		128K/ 256K (32K x 8 SRAM)

2 32-bit 120-pin PCI

1 shared PCI ISA				BIOS: 			Phoenix BIOS on 1MB flash BIOS

Auto detection of memory size

Processor Specifications:	486SX25; SX33, PQFP

486DX33, PGA				IDE Interface:		Resident 40-pin IDE connectors

486DX2/50; 486DX2/66, PGA

Floppy Support:		360K, 1.2MB, 1.44MB or 2.88MB

Upgrade CPU:		P24T; 238-pin ZIP socket			Communication Ports:	2 on-board 16550 serial ports

1 on-board bi-directional port

Memory:		(4) 72-pin SIMM sockets to accommodate:

Keyboard:		Standard AT-Style DIN connector			 	1MB x 36 (4MB); 2MB x 36 (8MB)

4MB x 36 (16MB); 8MB x 36 (32MB)

MAX: 128MB

Cache Size Selection

| Jumper | Configuration | Setting |
| --- | --- | --- |
| J52 | 256K L2 Cache 128 L2 Cache | Pins 2-3 Pins 1-2 |
| J53 | 256K L2 Cache 128 L2 Cache | Pins 1-2 Pins 2-3 |
| J58 | 256K L2 Cache 128 L2 Cache | Installed Not Installed |
| J61 | L2 Cache in WT mode L2 Cache in WB mode | Pins 1-2 Pins 2-3 |

On-Board I/O Connectors

| Jumper | Configuration | Setting |
| --- | --- | --- |
| J14 | PCI #3 (Slave, Shared with ISA Slot J6) |  |
| J15 | PCI #2 (Master/ Slave) |  |
| J17 | PCI #1 (Master/ Slave) |  |
| J27 | PCI IRQ 5 IRQ5 Available for ISA | Installed Not Installed |
| J28 | PCI IRQ 11 IRQ11 Available for ISA | Installed Not Installed |
| J29 | PCI IRQ 15 IRQ15 Available for ISA | Installed Not Installed |

CPU Selection

| Jumper | Configuration | Setting |
| --- | --- | --- |
| J40 | 33Mhz 25Mhz | Installed Not Installed |
| J59 | 487/P23T/486 P24T | Installed Not Installed |
| J60 | P24T in WB mode P24T in WT mode | Installed Not Installed |
| J65 | P24D All Other CPUs | Installed Not Installed |
| J66 | P24D All Other CPUs | Installed Not Installed |
| J67 | P24C (2x Clock) P24t, P24CT, 487SX, 486SX, P23S, 486DX486DX2, P24C (3x Clock), P24S, and P24S | Installed Not Installed |
| J68 | P24D All Other CPUs | Installed Not Installed |
| J69 | P24T, P24CT, P23S, P24C, P4S, P24S, and P24D 487SX, 486SX, 486DX, and 486DX2 | Installed Not Installed |
| J71 | 486SX All Other CPUs | Pins 2-3 Pins 1-2, 3-4 |
| J72 | P24D P23S, P24C, P4S, and P24S P24T, P24CT, 487SX, 486SX, 486DX and 486DX2 | Pins 1-2, 3-4 Pin 2-3 Not Installed |

Real-Time Settings

| Jumper | Function | Setting |
| --- | --- | --- |
| W1 | Real-Time Clock Installed | IN |
| W5 | To discharge Real-Time Clock For normal operation (default) | IN OUT |

Monitor Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W2 | Color Mono | IN OUT |

Remaining “W”

| Jumper | Function | Setting |
| --- | --- | --- |
| W1 | Reserved | 1-2 |
| W7 | Color Monitor Mono Monitor | Short Open |
| W16 | AT Style Keyboard PS/2 Style Keyboard | Open Short |
| W17 | Reserved | 1-2 |
| W18 | Clear RMA in RTC (may not be present) | Short |

Case and Peripheral Connections

| Jumper | Function | Setting |
| --- | --- | --- |
| J31 | Parallel Port Connector |  |
| J33 | Serial Port A |  |
| J34 | Serial Port B |  |
| J12 | Keyboard Connector |  |
| J6/J7 | Power Supply Connector |  |
| J32 | IDE Connector | Primary |
| J31 | Floppy Connector |  |
| J3 | External Battery | 1-battery positive; 2-N/C, 3-GND; 4-GND |
| J11 | Reset Switch | 1-GND; 2-GND |
| J5 | Turbo LED | 1-Signal; 2-GND |
| J4 | Turbo Switch | 1-Signal; 2-GND |
| J8 | Keylock/Power LED | 1-Power; 2-N/C; 3-GND; 4-5V DC |
| J10 | Speaker Connector | 1-Speaker; 2-N/C 3-GND; 4-5V DC |
| J13 | External Keyboard | 1-Keyb lck, 2-data 3-N/C, 4-GND, 5-<+> 5V |

Memory Configuration

| Bank 0 | Bank 1 | Bank 2 | Bank 3 | Total |
| --- | --- | --- | --- | --- |
| 4mb |  |  |  | 4mb |
| 4mb | 4mb |  |  | 8mb |
| 4mb | 4mb | 4mb |  | 12mb |
| 4mb | 4mb | 4mb | 4mb | 16mb |
| 16mb |  |  |  | 16mb |
| 4mb | 4mb | 16mb |  | 24mb |
| 16mb | 16mb |  |  | 32mb |
| 4mb | 16mb | 16mb |  | 36mb |
| 4mb | 4mb | 16mb | 16mb | 40mb |
| 16mb | 16mb | 16mb |  | 48mb |
| 16mb | 16mb | 16mb | 16mb | 64mb |