M54Si System board (Diablo).

·	Processor:

·	Single ZIF socket for Intel P54C CPUs.

·	75, 90, 100, 120, 133 MHz upgrade processors supported.

·	Standard and VRE voltages supported.

·	Expansion:

·	Two 32-bit PCI slots; Five 16-bit ISA slots; One shared PCI/ISA slot.

·	Chipset:

·	VLSI 590P PCI chipset.

·	SMC 665 Super I/O chip.

·	Memory (parity and non-parity):

·	Four 32/36-bit 72-pin SIMM sockets.

·	Supports 1M x 36 (4MB), 2M x 36 (8MB), 4M x 36 (16MB), and 8M x 36 (32MB).

·	Maximum memory is 128MB.

·	Cache:

·	16K CPU level 1 write back cache.

·	0/256/512K Asynchronous level 2 cache.

·	PCI IDE:

·	Mode 3 IDE connector using the CMD 640B controller.

·	ISA IDE:

·	Secondary ISA IDE connector using the SMC 655.

·	Floppy:

·	Supports 360K, 1.2, 1.44, 2.88MB floppy drives.

·	Serial / Parallel ports:

·	Two on-board 16550 compatible serial ports.

·	One on-board parallel port with ECP and EPP support.

·	BIOS:

·	Phoenix 4.04 BIOS on 1MB Flash chip.

·	Keyboard:

·	Standard AT style DIN keyboard connector.

·	Optional PS/2 style keyboard and mouse connectors.

·	Form factor:

·	Baby AT footprint (8.5” x 13”)

M54Si Jumper Settings

CPU speed:

| CPU Speed | 75MHz | 90MHz | 100MHz | 120MHz | 133MHz |
| --- | --- | --- | --- | --- | --- |
| Bus Speed | 50MHz | 60MHz | 66MHz | 60MHz | 66MHz |
| W3 | closed | closed | open | closed | open |
| W4 | closed | open | closed | open | closed |
| W5 | open | closed | open | closed | open |
| W38 | open | open | closed | closed | closed |

CPU speed ratio:

| Jumper | Function | Setting |
| --- | --- | --- |
| W2 | 2/3 (1.5x) Clock (75/90/100MHz) (default) ½ (2.0x) Clock (120/133MHz) | open closed |

Cache type:

| Jumper | Function | Asynchronous |
| --- | --- | --- |
| W1 | Write-back (default) Write-through | open closed |

Cache Size Selection:

|  | 256K | 512K |
| --- | --- | --- |
| W6 | 1-2 | 2-3 |
| W7 | 1-2 | 2-3 |
| W8 | 1-2 | 2-3 |

Video Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W20 | Color (default) Mono | closed open |

PS/2 Mouse Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W21 | PS/2 mouse not installed (default) PS/2 mouse installed | open closed |

M54Si Jumper Settings:

IDE IRQ Selection:

| Primary IDE Interrupt | Secondary IDE Interrupt | W24 | W29 |
| --- | --- | --- | --- |
| none | none | open | open |
| none | IRQ15 | 2-3 | open |
| IRQ14 | none | open | 1-2 |
| IRQ14 (default) | IRQ15 | 2-3 | 1-2 |

ISA IDE DMA Channel Selection:

|  | None (default) | DMA6 | DMA7 |
| --- | --- | --- | --- |
| W25 | open | 1-2 | 2-3 |
| W26 | open | 1-2 | 2-3 |

Parallel Port DMA Channel Selection

|  | None (default) | DMA1 | DMA3 |
| --- | --- | --- | --- |
| W22 | open | 1-2 | 2-3 |
| W23 | open | 1-2 | 2-3 |

BIOS Operation Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W10 | Flash BIOS write-protected (default) Flash BIOS programmable | open closed |

BIOS Reset Jumper:

| Jumper | Function | Setting |
| --- | --- | --- |
| W17 | Normal operation (default) Clear CMOS RAM settings | 1-2 2-3 |

Reserved Jumper Settings:

| Jumper | Function | Setting |
| --- | --- | --- |
| W9 | Reserved | 2-3 |
| W31 | Reserved | 1-2 |
| W32 | Reserved | 1-2 |
| W33 | Reserved | 1-2 |
| W34 | Reserved | 1-2 |
| W35 | Reserved | 2-3 |

M54Si board layout:

µ §