|  | Jumper |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System Speed (Intel Processors) | JP5 | JP6 | JP11 | JP15 | JP16 |
| 75 MHz External, 50 MHz Internal | On | On | Off | 2-3 | 2-3 |
| 90 MHz External, 60 MHz Internal | Off | On | On | 2-3 | 2-3 |
| 100 MHz External, 66 MHz Internal | On | Off | On | 2-3 | 2-3 |
| 120 MHz External, 60 MHz Internal | Off | On | On | 1-2 | 2-3 |
| 133 MHz External, 66 MHz Internal | On | Off | On | 1-2 | 2-3 |
| 150 MHz External, 60 MHz Internal | Off | On | On | 1-2 | 1-2 |
| 166 MHz External, 66 MHz Internal | On | Off | On | 1-2 | 1-2 |
| 200 MHz External, 66 MHz Internal | On | Off | On | 2-3 | 1-2 |

|  | Jumper |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System Speed (Cyrix Processors) | JP5 | JP6 | JP11 | JP15 | JP16 |  |  |  |  |  |
| 100 MHz External, 50 MHz Internal | On | On | Off | 1-2 | 2-3 |  |  |  |  |  |
| 110 MHz External, 55 MHz Internal | Off | Off | Off | 1-2 | 2-3 |  |  |  |  |  |
| 120 MHz External, 60 MHz Internal | Off | On | On | 1-2 | 2-3 |  |  |  |  |  |
| 133 MHz External, 66 MHz Internal | On | Off | On | 1-2 | 2-3 |  |  |  |  |  |

| CPU Supply Voltage | JP3 |  | Pentium CPU Type Selection | JP8 |
| --- | --- | --- | --- | --- |
| 3.3V (P54C/P54CT) | 1-2 STD |  | P55C VCORE (2.5V) | 1-2 |
| 3.4V (P54C/P54CT) | 2-3 VR |  | P54C/Other VCC (3.4V) | 2-3 |
| 3.5V (P54C) | 4-5 VRE |  | P55C VCORE (2.8V) | 4-5 |

| TAG SRAM Type | JP1 |  | Sound Interface Enable/Disable | JP18 |
| --- | --- | --- | --- | --- |
| Aster 16K x 8 | 1-2 |  | Enabled | 1-2 |
| Winbond | Off |  | Disabled | 2-3 |
| 32K x 8 | 2-3 |  |  |  |

| Multi I/O Chipset | JP4 | JP9 |  | Clear CMOS Memory | JP13 |
| --- | --- | --- | --- | --- | --- |
| Enabled | 2-3 | 1-2 |  | Normal | OFF |
| Disabled | 1-2 | 2-3 |  | Clear | On |

| CPU Internal Cache Write-Back /Write-Through Selection | JP7 |
| --- | --- |
| Write Through | On |
| Write Back | Off |

| Clock Ration (Internal:External) |  |  |  |
| --- | --- | --- | --- |
| Intel | Cyrix | JP15 | JP16 |
| 1.5:1 | 4:1 | 2-3 | 2-3 |
| 2:1 | 2:1 | 1-2 | 2-3 |
| 2.5:1 | 1:1 | 1-2 | 1-2 |
| 3:1 | 3:1 | 2-3 | 1-2 |

| Jumper | Function |
| --- | --- |
| JP10 | Reserved |
| JP12 | Color/ Monochrome Selection (On=Monochrome; OFF*=Color |
| JP14 | DRAM Refresh Rate (On=60MHz; OFF*=66MHz) |
| JP17 | Reserved |
| JP19 | Flash BIOS VCC Select (1-2=+12V; 2-3*=+5V) |
| JP21 | EEPROM Size (2-3=2MB EEPROM; 1-2*=1MB EEPROM) |
| JP24 | IRQ for PS/2 Mouse - Enable=ON* or Disable=Off |
| * = default setting |  |

| Jumper | Function |
| --- | --- |
| J1 | Keyboard connector |
| J2 | PS2 Mouse connector |
| J3 | External Battery connector |
| J4 | Secondary IDE |
| J5 | Primary IDE |
| J8 | HDD LED |
| J11 | Reset Switch Connector |
| J13 | Speaker connector |
| J15 | Turbo LED connector |
| J16 | Turbo Switch connector |
| J17 | Power LED/Keyboard connector |
| J18 | Parallel Port (printer port) connector |
| J20 | First Serial Port (COM1) connector |
| J21 | Second Serial Port (COM2) connector |
| J22 | Floppy Drive connector |