M54Hi Plus System board

·	Processor:

·	Single ZIF socket for Intel P54C CPUs.

·	Supports Intel processors from 75 Mhz – 200 MHz

·	Supports Cyrix processors 6x86 PR150+ and PR166+

(does not support 6x86L processors)

·	Standard and VRE voltages supported.

·	Expansion:

·	Three 32-bit PCI slots; Three 16-bit ISA slots; One shared PCI/ISA slot.

·	Chipset:

·	Intel Triton FX chipset.

·	SMC 665 Super I/O chip.

·	Memory (parity and non-parity):

·	Four 32/36-bit 72-pin SIMM sockets.

·	Supports 1M x 32/36 (4MB), 2M x 32/36 (8MB), 4M x 32/36 (16MB), and 8M x 32/36 (32MB).

·	Maximum memory is 128MB.

·	Supports EDO memory.

·	Cache:

·	16K CPU level 1 write back cache.

·	0/256/512K Asynchronous level 2 cache.

·	0/256K Pipelined Burst Synchronous cache.

·	PCI IDE:

·	Two IDE connectors on-board

·	Both supporting Mode 4 transfer rates.

·	Floppy:

·	Supports 360K, 1.2, 1.44, 2.88MB floppy drives.

·	Serial / Parallel ports:

·	Two on-board 16550 compatible serial ports.

·	One on-board parallel port with ECP and EPP support.

·	Sound

·	16 bit Sound Blaster™ compatible Vibra 16S/C  (Optional)

·	BIOS:

·	Phoenix 4.05 BIOS on 1MB Flash chip.

·	Keyboard:

·	Standard AT style DIN keyboard connector.

·	Optional PS/2 style keyboard and mouse connectors.

·	Form factor:

·	Baby AT footprint (8.5” x 13”)

M54Hi Plus - Jumper Settings

CPU speed (Intel Processors):

| CPU Speed | 75 MHz | 90 MHz | 100 MHz | 120 MHz | 133 MHz | 150 MHz | 166 MHz | 200 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bus Speed | 50 MHz | 60 MHz | 66 MHz | 60 MHz | 66 MHz | 60 MHz | 66 Mhz | 66 MHz |
| W4 | closed | open | closed | open | closed | open | closed | closed |
| W5 | closed | closed | open | closed | open | closed | open | open |
| W8 | 1-2 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |
| W2 | open | open | open | closed | closed | closed | closed | open |
| W10 | open | open | open | open | open | closed | closed | closed |
| W14 | open | open | closed | closed | closed | closed | closed | closed |

NOTE:   Must have –03 BIOS or higher to use Intel P200 processor

CPU speed (Cyrix Processors):

| CPU Speed | PR150+ | PR166+ |
| --- | --- | --- |
| Bus Speed | 60 MHz | 66 MHz |
| W4 | open | closed |
| W5 | closed | open |
| W8 | 2-3 | 2-3 |
| W2 | closed | closed |
| W10 | open | open |
| W14 | open | open |

NOTE:   Must have –02 BIOS or higher to use the Cyrix 6x86 processors

Bus speed:

| Function | W4 | W5 | W8 |
| --- | --- | --- | --- |
| 50 MHz | closed | closed | 1-2 |
| 60 MHz | open | closed | 2-3 |
| 66 MHz | closed | open | 2-3 |

CPU speed ratio:

| Clock Multiplier | W2 | W10 |
| --- | --- | --- |
| 1.5x clock (75 / 90 / 100 MHz) | open | open |
| 2.0x clock (120 / 133 MHz) | closed | open |
| 2.5x clock (150 / 166 MHz) | closed | closed |
| 3.0x clock (200 MHz) | open | closed |

CPU VRE setting:

| Jumper | Function | Setting |
| --- | --- | --- |
| W14 | VR 3.300-3.465 Volts (default) VRE 3.400-3.600 Volts | open closed |

You should contact Intel if you are not sure what voltage your processor should operate at.

Boot Block Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W6 | Normal (default) Reserved | 2-3 1-2 |

BIOS Reset Jumper:

| Jumper | Function | Setting |
| --- | --- | --- |
| W9 | Normal operation (default) Clear CMOS RAM settings | open closed |