M54Li System board

·	Processor:

·	Single ZIF socket for Intel P54C CPUs.

·	Supports Intel P75 – P100 processors (Intel P120 and P133 supported on Rev B boards)

·	Standard voltage supported (no VRE).

·	Expansion:

·	Three 32-bit PCI slots; Four 16-bit ISA slots (one shared)

·	Chipset:

·	SiS 85C550X chipset.

·	SMC 665 Super I/O chip.

·	Memory (parity and non-parity):

·	Four 32/36-bit 72-pin SIMM sockets.

·	Supports 1M x 32/36 (4MB), 2M x 32/36 (8MB), 4M x 32/36 (16MB), and 8M x 32/36 (32MB).

·	Maximum memory is 128MB.

·	Supports EDO memory.

·	Cache:

·	16K CPU level 1 write back cache.

·	0/256/512K Asynchronous level 2 cache.

·	PCI IDE:

·	Two IDE connectors on-board

·	Both supporting Mode 3 transfer rates.

·	Floppy:

·	Supports 360K, 1.2, 1.44, 2.88MB floppy drives.

·	Serial / Parallel ports:

·	Two on-board 16550 compatible serial ports.

·	One on-board parallel port with ECP support.

·	BIOS:

·	Phoenix 4.04 BIOS on 1MB Flash chip.

·	Keyboard:

·	Standard AT style DIN keyboard connector.

·	Optional PS/2 style keyboard and mouse connectors.

·	Form factor:

·	Baby AT footprint (8.5” x 13”)

M54Li Jumper Settings

CPU speed:

| CPU Speed | 75 MHz | 90 MHz | 100 MHz | 120 MHz | 133 MHz |
| --- | --- | --- | --- | --- | --- |
| Bus Speed | 50 MHz | 60 MHz | 66 MHz | 60 MHz | 66 MHz |
| W4 | open | open | open | closed | closed |
| W14 | closed | closed | open | closed | open |
| W15 | closed | open | closed | open | closed |
| W16 | open | closed | open | closed | open |

NOTE:  You must have a Rev B board to use the Intel 120 and 133 processors.

Cache Size Selection:

|  | 256K | # 512K |
| --- | --- | --- |
| W2 | open | closed |
| W3 | open | open |

Cache Voltage Selection:

|  | 5.0V (default) | 3.3V |
| --- | --- | --- |
| RP55 | 1-2, 3-4, 5-6, 7-8, 9-10 | open |
| RP56 | open | 1-2, 3-4, 5-6, 7-8, 9-10 |

Parallel Port DMA Channel Selection:

|  | DMA 1 | DMA 3 |
| --- | --- | --- |
| W11 | 1-2 | 2-3 |
| W12 | 1-2 | 2-3 |

BIOS Reset Jumper:

| Jumper | Function | Setting |
| --- | --- | --- |
| W18 | Normal operation (default) Clear CMOS RAM settings | open closed |