Board Name:	EISA3/VL

Part #:		09-00181-xx

Size:			Full-Size AT (13.8” x 12.02”)		Chip Sets:		Micronics EISA Chip Set

Expansion Slots:		6 EISA					Cache Options:		256K expandable to 512K

2 EISA/VL

Processor Specifications:	486SX25; SX33				BIOS: 			Phoenix BIOS on 1MB flash BIOS

486DX33

486DX2/50; 486DX2/66			IDE Interface:		Resident 40-pin IDE connectors

Memory:		(4) 72-pin SIMM sockets to accommodate:	Floppy Support:		360K, 1.2MB, 1.44MB or 2.88MB

256K x 36 (1MB); 1MB x 36 (4MB) 	Communication Ports:	2 on-board 16550 serial ports

4MB x 36 (16MB)						1 on-board bi-directional port

MAX: 64MB

Keyboard:		Standard AT-Style DIN connector

Cache Size Selection

| Jumper | 64K | 128K | 256K | 512K |
| --- | --- | --- | --- | --- |
| W8 | 1-2 | 2-3 | 2-3 | 2-3 |
| W9 | 1-2 | 1-2 | 2-3 | 2-3 |
| W10 | 1-2 | 1-2 | 1-2 | 2-3 |
| W11 | 1-2 | 2-3 | 2-3 | 2-3 |
| W12 | 1-2 | 1-2 | 2-3 | 2-3 |
| W13 | 1-2 | 1-2 | 1-2 | 2-3 |
| W14 | 3-4 | 4-5 | 1-2 | 4-5 |
| W15 | 2-3 | 1-2 | 1-2 | 1-2 |

CPU Selection

| Main CPU | CPU Socket | W2 | W5 | W6 |
| --- | --- | --- | --- | --- |
| i486SX | None | 1-2 | 1-2 | None |
| i486SX | i486DX2 | 1-2 | 2-3 | 1-2 & 3-4 |
| i486SX | i486DX | 2-3 | 1-2 | 1-2 & 3-4 |
| i486SX | i486SX | 1-2 | 2-3 | 1-2 & 3-4 |
| None | i486DX | 2-3 | 1-2 | 1-2 & 3-4 |
| None | i486DX2 | 1-2 | 1-2 | 1-2 & 3-4 |
| None | i486SX | 1-2 | 2-3 | 2-3 |
| None | i487SX | 1-2 | 2-3 | 1-2 & 3-4 |
| None | P24T | 2-3 | 2-3 | 1-2 & 3-4 |

CPU Speed Selection

| W3 | CPU Speed |
| --- | --- |
| On | i486SX-25, i486DX-25 & i486DX2-50 |
| Off | i486SX-33, i486DX-33, i486DX2-66, P24T |

Remaining “W”

| Jumper | Function | Setting |
| --- | --- | --- |
| W1 | Reserved | 1-2 |
| W7 | Color Monitor Mono Monitor | Short Open |
| W16 | AT Style Keyboard PS/2 Style Keyboard | Open Short |
| W17 | Reserved | 1-2 |
| W18 | Clear RMA in RTC (may not be present) | Short |

Case and Peripheral Connections

| Jumper | Function | Setting |
| --- | --- | --- |
| J31 | Parallel Port Connector |  |
| J33 | Serial Port A |  |
| J34 | Serial Port B |  |
| J12 | Keyboard Connector |  |
| J6/J7 | Power Supply Connector |  |
| J32 | IDE Connector | Primary |
| J31 | Floppy Connector |  |
| J3 | External Battery | 1-battery positive; 2-N/C, 3-GND; 4-GND |
| J11 | Reset Switch | 1-GND; 2-GND |
| J5 | Turbo LED | 1-Signal; 2-GND |
| J4 | Turbo Switch | 1-Signal; 2-GND |
| J8 | Keylock/Power LED | 1-Power; 2-N/C; 3-GND; 4-5V DC |
| J10 | Speaker Connector | 1-Speaker; 2-N/C 3-GND; 4-5V DC |
| J13 | External Keyboard | 1-Keyb lck, 2-data 3-N/C, 4-GND, 5-<+> 5V |

Memory Configuration

| Bank 0 | Bank 1 | Bank 2 | Bank 3 | Total |
| --- | --- | --- | --- | --- |
| 4mb |  |  |  | 4mb |
| 4mb | 4mb |  |  | 8mb |
| 4mb | 4mb | 4mb |  | 12mb |
| 4mb | 4mb | 4mb | 4mb | 16mb |
| 16mb |  |  |  | 16mb |
| 4mb | 4mb | 16mb |  | 24mb |
| 16mb | 16mb |  |  | 32mb |
| 4mb | 16mb | 16mb |  | 36mb |
| 4mb | 4mb | 16mb | 16mb | 40mb |
| 16mb | 16mb | 16mb |  | 48mb |
| 16mb | 16mb | 16mb | 16mb | 64mb |