Board Name:	JX30GC

Part #:		09-00203-xx

Size:			Baby AT (8 1/2” x 13”)			Chip Sets:		Micronics X30 ASIC Chip Set

Expansion Slots:		5 16-bit ISA				Cache Options:		128K/ 256K (32K x 8 SRAM)

2 16-bit ISA/32bit VL-Bus

Both VL Slots support bus-mastering	BIOS: 			Phoenix BIOS on 1MB flash BIOS

Auto detection of memory size

Processor Specifications:	486SX25; SX33, PQFP

486DX33, PGA				IDE Interface:		Resident 40-pin IDE connectors

486DX2/50; 486DX2/66, PGA

486DX4 100				Floppy Support:		360K, 1.2MB, 1.44MB or 2.88MB

Upgrade CPU:		P24T; 238-pin ZIP socket			Communication Ports:	2 on-board 16550 serial ports

1 on-board bi-directional port

Memory:		(4) 72-pin SIMM sockets to accommodate:

Keyboard:		Standard AT-Style DIN connector				256K x 36 (1MB); 1MB x 36 (4MB)

4MB x 36 (16MB)

MAX: 64MB

Cache Size Selection

| Cache | W10 | W11 | W22 | W23 | W24 |
| --- | --- | --- | --- | --- | --- |
| 64K | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 |
| 128K | 1-2 | 2-3 | 1-2 | 1-2 | 2-3 |
| 256K | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |

Write-Back Cache

| Cache | W16 | W17 |
| --- | --- | --- |
| 64K | 1-2 | 1-2 |
| 128K | 1-2 | 2-3 |
| 256K | 2-3 | 2-3 |

CPU Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W12 | 486SX CPU Installed Any other CPU | 1-2 2-3 |
| W13 | PQFP CPU Installed with an Overdrivetm CPU PQFP CPU Installed or a DX, DX2, DX4 | 1-2 2-3 |

Real-Time Settings

| Jumper | Function | Setting |
| --- | --- | --- |
| W1 | Real-Time Clock Installed | IN |
| W5 | To discharge Real-Time Clock For normal operation (default) | IN OUT |

Monitor Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W2 | Color Mono | IN OUT |

Remaining “W”

| Jumper | Function | Setting |
| --- | --- | --- |
| W1 | Reserved | 1-2 |
| W7 | Color Monitor Mono Monitor | Short Open |
| W16 | AT Style Keyboard PS/2 Style Keyboard | Open Short |
| W17 | Reserved | 1-2 |
| W18 | Clear RMA in RTC (may not be present) | Short |

Case and Peripheral Connections

| Jumper | Function | Setting |
| --- | --- | --- |
| J31 | Parallel Port Connector |  |
| J33 | Serial Port A |  |
| J34 | Serial Port B |  |
| J12 | Keyboard Connector |  |
| J6/J7 | Power Supply Connector |  |
| J32 | IDE Connector | Primary |
| J31 | Floppy Connector |  |
| J3 | External Battery | 1-battery positive; 2-N/C, 3-GND; 4-GND |
| J11 | Reset Switch | 1-GND; 2-GND |
| J5 | Turbo LED | 1-Signal; 2-GND |
| J4 | Turbo Switch | 1-Signal; 2-GND |
| J8 | Keylock/Power LED | 1-Power; 2-N/C; 3-GND; 4-5V DC |
| J10 | Speaker Connector | 1-Speaker; 2-N/C 3-GND; 4-5V DC |
| J13 | External Keyboard | 1-Keyb lck, 2-data 3-N/C, 4-GND, 5-<+> 5V |

Memory Configuration

| Bank 0 | Bank 1 | Bank 2 | Bank 3 | Total |
| --- | --- | --- | --- | --- |
| 4mb |  |  |  | 4mb |
| 4mb | 4mb |  |  | 8mb |
| 4mb | 4mb | 4mb |  | 12mb |
| 4mb | 4mb | 4mb | 4mb | 16mb |
| 16mb |  |  |  | 16mb |
| 4mb | 4mb | 16mb |  | 24mb |
| 16mb | 16mb |  |  | 32mb |
| 4mb | 16mb | 16mb |  | 36mb |
| 4mb | 4mb | 16mb | 16mb | 40mb |
| 16mb | 16mb | 16mb |  | 48mb |
| 16mb | 16mb | 16mb | 16mb | 64mb |