# M54Pe System board

	Processor:

	Dual ZIF sockets for Intel P54C CPUs.

	Supports Intel P75 – P100 processors (P120 and P133 processors supported on Rev B boards)

	Standard and VRE voltages supported.

	Expansion:

	Three 32-bit PCI slots; Six 32-bit EISA slots (one shared)

	Chipset:

	Intel Neptune chipset.

	Memory (parity and non-parity):

	Eight 32/36-bit 72-pin SIMM sockets.

	Supports 1M x 32/36 (4MB), 2M x 32/36 (8MB), 4M x 32/36 (16MB), 8M x 32/36 (32MB), and 16M x 32/36 (64MB).

	Maximum memory is 512 MB.

	Cache:

	16K CPU level 1 write back cache.

	0/256/512K Asynchronous level 2 cache.

	Onboard IDE:

	Primary PCI IDE supporting Mode 3 transfers

	Secondary ISA IDE supporting Mode 0 transfers

	Floppy:

	Supports 360K, 1.2, 1.44, 2.88MB floppy drives.

	Serial / Parallel ports:

	Two on-board 16550 compatible serial ports.

	One on-board parallel port with ECP and EPP support.

	BIOS:

	Phoenix 4.04 BIOS on 1MB Flash chip.

	Keyboard:

	Standard AT style DIN keyboard connector.

	Optional PS/2 style keyboard and mouse connectors.

	Form factor:

	Extended AT footprint (13.8” x 12.02”)

M54Pe Jumper Settings

CPU speed:

| CPU Speed | 75 MHz | 90 MHz | 100 MHz | 120 MHz | 133 MHz |
| --- | --- | --- | --- | --- | --- |
| Bus Speed | 50 MHz | 60 MHz | 66 MHz | 60 MHz | 66 MHz |
| W6 | 2-3 | 1-2 | 2-3 | 1-2 | 2-3 |
| W40 | closed | open | open | open | open |
| W2 | open | open | open | closed | closed |

NOTE:    You must have a Rev B board to use the P120 and P133 processors.

Cache Setting:

| Jumper | Function | Setting |
| --- | --- | --- |
| W1 | Write-through Write-back | closed open |

Cache Size Selection:

|  | 256K | 512K |
| --- | --- | --- |
| W5 | 1-2 | 2-3 |
| W10 | 1-2 | 2-3 |

Video Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W20 | Color (default) Mono | closed open |

PCI IDE:

| Jumper | Function | Setting |
| --- | --- | --- |
| W28 | Enabled Disabled | closed open |

ISA IDE:

| Jumper | Function | Setting |
| --- | --- | --- |
| W25 | Enabled – IRQ 14 Enabled – IRQ 15 Disabled | 1-2 2-3 open |

Clearing CMOS:

The only way to clear the CMOS on this board is to remove the black Benchmarq RTC (real time clock) chip on the motherboard and short it out.    This is most safely done by removing the chip and laying it down pin-side down on top of the power supply for about 15 seconds, then gently rubbing it against the metal for an additional 5 seconds to short out the pins.    Place the RTC back in its socket, and power the system on.