M54Pi Jumper Settings:

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP1 | 90MHz 75Mhz | 1-2 2-3 |
| JP36 | 75Mhz/90Mhz 100Mhz | Installed Not Installed |
| JP37 | 75Mhz 90Mhz 100Mhz | Installed Not Installed Installed |
| JP38 | 75Mhz 90Mhz 100Mhz | Not Installed Installed Not Installed |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP7 | Read Only To Perform Flash BIOS Update | Not Installed Installed |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP10 | Clear the Real-Time Clock | Installed |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP5 | 256K 512K | 2-3 1-2 |
| JP19 | 256K 512K | 1-2 2-3 |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP4 | PCI IDE Chip Enabled (default) PCI IDE Chip Disabled | Not Installed Installed |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP11 | Mono Color | Not Installed Installed |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP3 | Primary IDE IRQ 14 (default) Primary IDE IRQ 15 | 1-2 2-3 |
| JP12 | Secondary IDE IRQ 14 Secondary IDE IRQ 15 (default) | 1-2 2-3 |
| JP17 | Parallel Port IRQ 7 Parallel Port IRQ 5 | 1-2 2-3 |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP13 | DMA 6 for 2nd IDE DMA 7 | 1-2 2-3 |
| JP14 | DMA 6 DMA 7 | 1-2 2-3 |
| JP15 | DMA 1 for parallel DMA 3 | 1-2 2-3 |
| JP16 | DMA 1 DMA 3 | 1-2 2-3 |
| Jumper | Configuration | Setting |
| JP18 | Enable SMC665 (default) Disable SMC665 | Not Installed Installed |

| Jumper | Configuration | Setting |
| --- | --- | --- |
| JP2 | Reserved | Not Installed |
| JP25 & JP27 | PS/2 Mouse Connector (may not be present) |  |
| JP23 | External PC/AT Keyboard Connector (may not be present) |  |
| JP26 | PS/2 Keyboard Connector (may not be present) |  |
| JP24 | PC/AT Keyboard Connector (may not be present) |  |
| JP31 | Serial Port A | Can be disabled at the CMOS |
| JP32 | Serial Port B | Can be disabled at the CMOS |
| JP33 | Parallel |  |
| J5 | PCI IDE Connector (primary) |  |
| JP34 | Floppy IDE Connector |  |
| J29 | On-board IDE LED |  |
| J12 | Reset Switch | 1-signal; 2-ground |
| J1 | Speaker Connector | 1-power LED; 2-N/C; 3-ground; 4-(+) 5V DC |
| J13 | Power LED/ Keylock | 1-power LED; 2-N/C; 3-ground; 4-keybd LCK; 5-ground |
| J30 | ISA IDE Connector (secondary) |  |