Board Name:	MX30

Part #:		09-00165-xx

Size:			Baby AT (8 1/2” x 13”)			Chip Sets:		Micronics X30 ASIC Chip Set

Expansion Slots:		6 16-bit ISA				Cache Options:		64K/ 256K

2 16-bit ISA/32bit VL-Bus

Both VL Slots support bus-mastering	BIOS: 			Phoenix BIOS

Auto detection of memory size

Processor Specifications:	486SX25; SX33

486DX33				Keyboard:		Standard AT-Style DIN connector

486DX2/50; 486DX2/66

Upgrade CPU:		P24T; 238-pin ZIP socket

Memory:		(4) 72-pin SIMM sockets to accommodate:

256K x 36 (1MB); 1MB x 36 (4MB)

4MB x 36 (16MB)

MAX: 64MB

Cache Size Selection

|  | W8 | W10 | W11 | W12 | W13 | W18 | W19 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64k | 2-3 | 1-2 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2 |
| 128k | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 | 2-3 | 1-2 |
| 256k | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |

CPU Speed

| CPU Speed | W20 | W21 |
| --- | --- | --- |
| 25Mhz-DX/50Mhz-DX2 | 2-3 | 2-3 |
| 33Mhz-DX/66Mhz-DX2 | 1-2 | 2-3 |

The Remaining Jumpers

| Jumper | Configuration | Function |
| --- | --- | --- |
| W2 | External Keyboard Connector |  |
| W4 | Default Discharge CMOS RAM | 1-2 2-3 |
| W5 | Dallas real time clock Any other real time clock | Short Open |
| W6 | Mono Color | Short Open |
| W25 | Reset Switch | 1-signal, 2-grd |
| W26 | Turbo Switch | 1-signal, 2-grd |
| W27 | Turbo LED | 1-signal, 2-grd |
| W28 | Power LED/Keyboard | 1-power; 2-N/C; 3-grd; 4-keybd lock; 5-grd |
| J12 & 13 | Power Supply Connector |  |
| J24 | Speaker Connector | 1-Speaker; 2-N/C 3-grd; 4-5V DC |

CPU Selection

| PQFP | Overdrive Socket | W22 | W23 | W24 |
| --- | --- | --- | --- | --- |
| i486SX | None | 1-2 | 1-2 | 1-2 |
| i486SX | 486DX/DX2 | 2-3 | 2-3 | 2-3 |
| i486SX | Overdrive CPU | 2-3 | 2-3 | 1-2 |
| None | i486SX | 1-2 | 1-2 | 1-2 |
| None | 486DX/DX2 | 2-3 | 2-3 | 2-3 |

Memory Configuration

| Bank 0 | Bank 1 | Bank 2 | Bank 3 | Total |
| --- | --- | --- | --- | --- |
| 1mb |  |  |  | 1mb |
| 1mb | 1mb |  |  | 2mb |
| 4mb |  |  |  | 4mb |
| 1mb | 1mb | 4mb |  | 6mb |
| 4mb | 4mb |  |  | 8mb |
| 1mb | 1mb | 4mb | 4mb | 10mb |
| 4mb | 4mb | 4mb |  | 12mb |
| 4mb | 4mb | 4mb | 4mb | 16mb |
| 16mb |  |  |  | 16mb |
| 1mb | 1mb | 16mb |  | 18mb |
| 4mb | 16mb |  |  | 20mb |
| 4mb | 4mb | 16mb |  | 24mb |
| 16mb | 16mb |  |  | 32mb |
| 4mb | 16mb | 16mb |  | 36mb |
| 4mb | 4mb | 16mb | 16mb | 40mb |
| 16mb | 16mb | 16mb |  | 48mb |
| 16mb | 16mb | 16mb | 16mb | 64mb |

µ §