M54Hi System board (Élan).

·	Processor:

·	Single ZIF socket for Intel P54C CPUs.

·	75, 90, 100, 120, 133 MHz Intel P54  processors supported (150 and 166 Intel processors supported on Rev B boards).

·	Standard and VRE voltages supported.

·	Expansion:

·	Three 32-bit PCI slots; Three 16-bit ISA slots; One shared PCI/ISA slot.

·	Chipset:

·	Intel Triton FX chipset.

·	SMC 665 Super I/O chip.

·	Memory (parity and non-parity):

·	Four 32/36-bit 72-pin SIMM sockets.

·	Supports 1M x 32/36 (4MB), 2M x 32/36 (8MB), 4M x 32/36 (16MB), and 8M x 32/36 (32MB).

·	Maximum memory is 128MB.

·	Supports EDO memory.

·	Cache:

·	16K CPU level 1 write back cache.

·	0/256/512K Asynchronous level 2 cache.

·	0/256K Pipelined Burst Synchronous cache.

·	PCI IDE:

·	Two IDE connectors on-board

·	Both supporting Mode 4 transfer rates.

·	Floppy:

·	Supports 360K, 1.2, 1.44, 2.88MB floppy drives.

·	Serial / Parallel ports:

·	Two on-board 16550 compatible serial ports.

·	One on-board parallel port with ECP and EPP support.

·	BIOS:

·	Phoenix 4.04 BIOS on 1MB Flash chip.

·	Keyboard:

·	Standard AT style DIN keyboard connector.

·	Optional PS/2 style keyboard and mouse connectors.

·	Form factor:

·	Baby AT footprint (8.5” x 13”)

M54Hi Jumper Settings

CPU speed:

| CPU Speed | 75 MHz | 90 MHz | 100 MHz | 120 MHz | 133 MHz | 150 MHz | 166 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bus Speed | 50 MHz | 60 MHz | 66 MHz | 60 MHz | 66 MHz | 60 MHz | 66 Mhz |
| W4 | closed | open | closed | open | closed | open | closed |
| W5 | closed | closed | open | closed | open | closed | open |
| W30 | 1-2 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |
| W2 | open | open | open | closed | closed | closed | closed |
| W3 | open | open | closed | closed | open | closed | closed |
| W13 | N/A | N/A | N/A | N/A | N/A | closed | closed |

NOTE:  You must have a Rev B board to use the P150 and P160 processors.

CPU speed ratio:

| Jumper | Function | Setting |
| --- | --- | --- |
| W2 | 1.5x clock (75/90/100 MHz) (default) 2.0x clock (120/133/150/166 MHz) | open closed |

CPU VRE setting:

| Jumper | Function | Setting |
| --- | --- | --- |
| W3 | VR 3.300-3.465 Volts (default) VRE 3.400-3.600 Volts | open closed |

You should contact Intel if you are unsure of what voltage your processor needs to operate at.

Cache type:

| Jumper | Pipelined Burst (Synchronous) | Asynchronous |
| --- | --- | --- |
| W6 | 1-2 | 1-2 |
| W7 | 1-2 | 2-3 |

Cache Size Selection:

|  | No Cache | 256K | 512K |
| --- | --- | --- | --- |
| W8 | 1-2 | 2-3 | 1-2 |
| W9 | 1-2 | 1-2 | 2-3 |
| W24 | open | 1-2 | 2-3 |

Video Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W16 | Color (default) Mono | closed open |

PS/2 Mouse Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W17 | PS/2 mouse not installed (default) PS/2 mouse installed | open closed |

Parallel Port DMA Channel Selection

|  | None (default) | DMA 1 | DMA 3 |
| --- | --- | --- | --- |
| W18 | open | 1-2 | 2-3 |
| W19 | open | 1-2 | 2-3 |

BIOS Operation Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W11 | Flash BIOS write-protected (default) Flash BIOS programmable | open closed |

Boot Block Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W10 | Normal (default) Reserved | 2-3 1-2 |

BIOS Reset Jumper:

| Jumper | Function | Setting |
| --- | --- | --- |
| W12 | Normal operation (default) Clear CMOS RAM settings | open closed |