Technical Product Summary

Classic/PCI i486TM Baby-AT Motherboard

Models:

BP4S33AT

BP4D33AT

BP4D266AT

Preliminary Version 0.1

April, 1993

Order Number PRELIMINARY

Intel Corporation makes no warranty for the use of its products and assumes no responsibility for any errors which may appear in this document nor does it make a commitment to update the information contained herein.

Intel Corporation retains the right to make changes to these specifications at any time, without notice.

Contact your local sales office to obtain the latest specifications before placing your order.

The following are trademarks of Intel Corporation and may only be used to identify Intel products:

376Ô	i750â	MAPNETÔ

AboveÔ	i860Ô	MatchedÔ

ActionMediaâ	i960â	MCSâ

BITBUSÔ	Intel287Ô	Media MailÔ

Code BuilderÔ	Intel386Ô	NetPortÔ

DeskWareÔ	Intel387Ô	NetSentryÔ

Digital StudioÔ	Intel486Ô	OpenNETÔ

DVIâ	Intel487Ô	OverDriveÔ

EtherExpressÔ	Intelâ	ParagonÔ

ETOXÔ	intel inside.Ô	PentiumÔ

ExCAÔ	Intellecâ	ProSolverÔ

Exchange and GoÔ	iPSCâ	READY-LANÔ

FaxBACKÔ	iRMXâ	Reference Pointâ

FlashFileÔ	iSBCâ	RMX/80Ô

Grand ChallengeÔ	iSBXÔ	RxServerÔ

iâ	iWARPÔ	SatisFAXtionâ

ICEÔ	LANDeskÔ	SnapIn 386Ô

iLBXÔ	LANPrintâ	Storage BrokerÔ

InboardÔ	LANProtectÔ	SuperTunedÔ

i287Ô	LANSelectâ	The Computer Inside.Ô

i386Ô	LANShellâ	TokenExpressÔ

i387Ô	LANSightÔ	Visual EdgeÔ

i486Ô	LANSpaceâ	WYPIWYFâ

i487Ô	LANSpoolâ	IntelTechDirectÔ

MDS is an ordering code only and is not used as a product name or trademark. MDS is a registered trademark of Mohawk Data Sciences Corporation.

CHMOS and HMOS are patented processes of Intel Corp.

Intel Corporation and Intel's FASTPATH are not affiliated with Kinetics, a division of Excelan, Inc. or its FASTPATH trademark or products.

TRADEMARK ACKNOWLEDGMENTS

ATI is a trademark of ATI Technologies Inc.

AutoCAD is a trademark of Autodesk Inc.

BAPCo and Sysmark92 are trademarks of the Business Applications Performance Corporation.

Cadvance is a trademark of ISICAD Inc.

Centronics is a trademark of Centronics Data Computer Corp.

IBM, MDA, MCGA, EGA, VGA, Personal System/2, PS/2, PC/XT and PC/AT are trademarks of International Business Machines.

Microsoft, MS-DOS, OS/2, Xenix and Windows are trademarks of Microsoft Corporation.

Novell is the trademark of Novell, Inc.

PolySwitch is a registered trademark of Raychem Corporation.

SCO is a trademark of Santa Cruz Operations, Inc.

SPEC, SPECratio, SPECint92 and SPECfp92 are trademarks of the Standard Performance Evaluation Corporation.

UNIX is a trademark of AT&T Bell Laboratories.

Western Digital is a trademark of Western Digital Corp.

All other trademarks are the property of their respective owners.

Classic/PCI i486 Baby-AT Motherboard

Preliminary Technical Product Summary

Revision 0.1

Table of Contents

µIntroduction	4

Baby-AT Form Factor	4

Board Level Features	5

CPU	5

Performance Upgrade	5

Second Level Cache	5

System BIOS	5

PCI Auto-configuration Capability	5

Setup Utility	6

FLASH Implementation	6

Upgrade Utility	6

Flash User Area	7

Keyboard (and Mouse) Interface	7

System Memory	7

Core Chip Set	7

82424TX Cache/DRAM/Controller (CDC)	8

82423TX Data Path Unit (DPU)	8

82378IB System I/O (SIO)	8

Expansion Slots	8

SMC 37C663 Super I/O Controller	8

Dallas DS12887 Real Time Clock, CMOS RAM and Battery	8

Front Panel Connectors	8

Security	9

BIOS Password	9

Setup Enable Jumper	9

System Integration Features	10

Back panel Connections	10

Power Supply	10

Appendices	11

Appendix A - User-Installable Upgrades	11

System Memory	11

Performance Upgrade	12

Qualified DRAM SIMMs	12

Appendix B - Jumpers	13

Appendix C - BIOS Setup Options	14

Appendix D - BIOS Recovery	15

Using the Upgrade Utility	15

Recovery Mode	15

Appendix E - Memory Map	16

Appendix F - I/O Map	17

Appendix G - Board Interrupts & DMA	18

Appendix H - Connectors	19

AT Style Keyboard Port (J8L1 = Keyboard)	19

Optional PS/2 Style Keyboard, Mouse Ports (J8L2 = Keyboard, J9L1 = Mouse)	19

Turbo LED Connector (J1A1)	19

Hard Drive LED Connector (J1A2)	19

Key lock/Power LED Connector (J1B1)	19

Speaker Connector (J1C1)	19

Turbo Switch Connector (J1C1B)	19

Primary Power Connector (J9K1)	20

Auxiliary (3.3V) Power Connector (J4H1)	20

Parallel Port (J7J1)	20

Serial Ports (J7L2 = COM1, J7L1 = COM2)	20

Reset Connector (J1B2)	20

Floppy Connector (J9F2)	21

IDE Connector (J9F3)	21

ISA Connector (J6G2, J6G1, J5G1, J4G1, J1G1)	22

PCI Connector (J3H1,J2H1,J1H1)	23

Appendix I - Baby-AT Chassis Suppliers	25

Appendix J - Environmental Standards	26

Appendix K - Reliability Data	27

Appendix L - Software Driver Support	28

Appendix M - Qualified Peripherals	29

Appendix N - Customer Support	30

iPAN (Intel Product Assistance Network)	30

iPUB (Intel Product Update Bulletin)	30

iPALS (Intel Phone Action Line Support)	30

FaxBack™	30

Appendix O - Physical Dimensions	31

Board	31

Appendix P - Product Codes	32

Boards	32

Accessories	32

Documentation	32

# Introduction

The Classic/PCI i486 Baby-AT Motherboard delivers excellent, cost effective performance in a very affordable i486ä platform. A wide range of CPU offerings provides immediate performance flexibility, and a single 238-pin blue   OverDriveä  Ready ZIF (Zero Insertion Force) processor socket allows upgrades to higher performance in the future. Additionally, by incorporating a second level, high performance cache and four SIMM sites for memory expansion to 128 MB, five ISA expansion connectors and three PCI connectors, the Classic/PCI i486 Baby-AT Motherboard is ideally featured for expandable, performance sensitive desktop applications.  The Classic/PCI i486 Baby-AT Motherboard will excel in high end i486 Processor desktop PCs running existing compatible applications.

An easy upgrade path to higher CPU performance is built into the Classic/PCI i486 Baby-AT Motherboard. The processor socket accepts either an i487äSX to enhance performance with numeric intensive applications, or an OverDrive Processor, Intel's upgrade component that doubles the speed of all i486 internal processes. Support for the next generation OverDrive Processor, based on the Pentiumä  CPU, protects today's investment in the Classic/PCI i486 Baby-AT Motherboard.

The mini-Baby-AT board - with i486 CPU, cache, and integrated I/O - is the smallest PCI-based motherboard solution on the market today that is upgrade able to an OverDrive processor based on Pentium CPU technology.

## Baby-AT Form Factor

The Classic/PCI i486 Baby-AT motherboard matches the Baby-AT standards well established in the PC industry, while reducing the size from the full length (13") form factor. This Baby-AT industry standard specifies the maximum board size, board mounting locations, and location for the keyboard connector, as well as expansion slot  placement. The Classic/PCI i486 Baby-AT meets all of these requirements while adding PCI expansion possibilities. Figure 1 illustrates the Baby-AT form factor. A list of several chassis suppliers supporting the Baby-AT standard is included in Appendix I.

### µ §

Figure 1. Classic/PCI i486 Baby-AT Motherboard dimensions.

# Board Level Features

## CPU

The Classic/PCI i486 Baby-AT motherboard has a wide price/performance range to meet a variety of customer needs. Four base CPU options are available:

· 	an i486 SX running at 33 MHz;

· 	an i486 DX at 33 MHz;

· 	an i486 DX2 at 66 MHz; or

· 	a next generation i486 processor operating at 3.3V internally, and 33 MHz bus speed

The Classic/PCI i486 Baby-AT motherboard supports all of the functionality of the i486. Common features of the CPU include backward compatibility with the 8086, 80286,  and i386ä CPUs, burst mode bus cycles, and an on-chip 8 KB cache.  The cache is 4-way set associative, uses a write-through policy, and can be disabled via software.

The i486 DX CPU contains an on-chip numeric coprocessor to increase the speed of floating point operations. This coprocessor is backward code-compatible with i387ä DX and i387 SX math coprocessors and complies with ANSI/IEEE standard 754-1985. The i486 SX does not include the numeric coprocessor. The i486 DX2 incorporates clock-doubling technology developed by Intel to offer the highest CPU performance available today.

## Performance Upgrade

The Classic/PCI i486 Baby-AT motherboard incorporates a single 238-pin processor socket allowing easy upgrades to CPU performance. All Classic/PCI i486 Baby-AT motherboards can be upgraded with an OverDrive Processor - including future OverDrive Processors based on the Pentium CPU architecture.  These upgrades will provide significantly higher CPU performance and numeric capability.  In addition, systems with a i486 SX/33 CPU can improve floating point performance by installing an i487 SX/33 in place of the CPU.  When replacing an i486 SX/33 CPU with an OverDrive Processor no jumper change are required; just power up and go!

## Second Level Cache

In addition to the i486 CPU's internal cache, the Classic/PCI i486 Baby-AT motherboard was designed with a second level cache using industry-standard SRAM. The 82424TX CDC includes a direct-mapped, write-back cache controller. The motherboard includes four 32K x 8 20ns cache SRAM devices for a total of 128 KB cache memory.

## System BIOS

The Classic/PCI i486 Baby-AT Motherboard uses American Megatrends Incorporated (AMI) i486 CPU ROM BIOS, which provides ISA compatibility. The system BIOS is stored in FLASH EEPROM, providing easy upgradability of program code space from a floppy disk or a file downloaded from a BBS; BIOS upgrades will be available for download from iPAN, the electronic bulletin board service of IntelTechDirect™. In addition to the AMI BIOS, the FLASH memory also contains the PCI Auto-configuration utility, SETUP utility, Power-On Self-Tests (POST), and update recovery code.  For improved system performance, the Classic/PCI i486 Baby-AT Motherboard supports system BIOS shadowing, allowing the BIOS to execute from 32-bit on-board write-protected DRAM instead of the slower 8-bit FLASH devices. The Classic/PCI i486 Baby-AT BIOS sign-on during POST is along the bottom of the screen, and contains information which identifies revision and type of BIOS. On the lower left is a four digit code which denotes revision; first production units will display 0101, and as updates occur will roll the "minor revision number", i.e. 0102. BIOS level and board identifier code is contained on the lower right side, and will be P00.AQ0 for the Classic/PCI i486 Baby-AT motherboard. As a note, A01 denotes Alpha revision 01, and B01 denotes Beta revision 01.

Further information on BIOS functions can be found in the IBM PS/2 and Personal Computer BIOS Technical Reference published by IBM, and the ISA and EISA Hi-Flex AMIBIOS Technical Reference published by AMI and available at most technical bookstores.

### PCI Auto-configuration Capability

The PCI Auto-configuration feature provides a new level of user satisfaction. Simply plug a PCI add-in card into an empty connector and turn the system on. The BIOS automatically configures interrupts, DMA channels, I/O space, etc. This eliminates the requirement for adapter card jumper changes due to resource conflicts, and provides unrivaled ease of use in a PC.

The auto-configuration routine operates in conjunction with an ISA configuration utility. This utility enables the user to specify the ISA options used, and ties into the PCI configuration software transparently to provide seamless add-in card installation.

### Setup Utility

Classic/PCI i486 Baby-AT incorporates many commonly used system setup features into the FLASH EEPROM. The BIOS SETUP Program has been enhanced and provides several new options to take advantage of the Classic/PCI i486 Baby-AT Motherboard's new features.  New options include:

·	Auto configuration of IDE hard disks.

·	Support for four IDE disk drives (primary and secondary)

·	Serial Port 1 -- Enable/Disable

·	Serial Port 2 -- Enable/Disable

·	Parallel Port -- Enable/Disable, Bi-directional/Output only

·	Cache/Shadow Memory Option -- Provides the user the option to assign a block of addresses below the 1 MB boundary as non-shadowed, non-cached.  Primarily used for expansion card ROM which causes timing issues when shadowed and cached.

·	ISA interrupts - Allows ISA interrupts IRQ9, IRQ10, IRQ15 to be assigned to add-in ISA adapters, thereby informing the PCI configuration utility which interrupts not to use.

The setup utility is accessible only during the Power-On Self Test by pressing the <F1> key anytime after the POST memory test has begun and before boot begins. For security purposes, access to SETUP can be disabled via a jumper on the motherboard. The ROM-based setup allows the system configuration to be modified without opening the system for most basic changes. Setup options are detailed in the Appendices.

### FLASH Implementationtc  \l 3 "Flash Implementation"§

The Intel 28F001BXT 1 Mb FLASH component is organized as 128K x 8 (128 KB). The Flash device is divided into five areas, as described in Table 2.

| System Address |  | FLASH Memory Area |
| --- | --- | --- |
| F0000H | FFFFFH | 64 KB Main BIOS |
| EE000H | EFFFFH | 8 KB Boot Block (Not FLASH erasable) |
| ED000H | EDFFFH | 4 KB Parameter Block (used for PCI) |
| EC000H | ECFFFH | 4 KB Flash User Area |
| E0000H | EBFFFH | System BIOS |

Table 1. Flash Memory Organization

The FLASH device resides in system memory in two 64 KB segments starting at E0000H, and is distributed in two different organizations, depending on the mode of operation. In Normal Mode address line A16 is inverted, switching the E000H and F000H segments so that the BIOS is organized as shown in the system address column above. Recovery mode removes the inversion on address line A16, swapping the E000H and F000H segments so that the 8 KB boot block resides at FE000H where the i486 expects the bootstrap loader to exist. This mode is only necessary in the unlikely event that a BIOS upgrade procedure is interrupted, causing the BIOS area to be left in an unusable state. For information on recovering the BIOS in the event of a catastrophic failure, refer to the Appendices.

### Upgrade Utilitytc  \l 3 "Upgrade Utility"§

FLASH memory brings new opportunities for distributing BIOS upgrades.  Installing a new version of BIOS will no longer require removal of the system cover and the replacement of EPROM's.  Instead, the upgrade can be done completely from a floppy diskette.  Easy access to BIOS upgrades will be available through download able files on the iPAN bulletin board.

Security is provided in two ways.  First, the FLASH upgrade utility insures the upgrade BIOS matches the target system to prevent accidentally installing a BIOS for a different type of system.  Second, security to prevent unauthorized changes to the BIOS is provided via a write protect jumper on the motherboard.  The default setting is to allow BIOS upgrades.  A recovery jumper is provided to recover from the unlikely event of an unsuccessful BIOS upgrade.  It forces the ROM decode to access a 32 KB block of write protected code in the FLASH device that facilitates recovery.  The default value for this jumper (RV) is for "normal" mode (note:  this jumper is not changed during normal BIOS updates, it is used only if a problem is encountered).

The disk-based FLASH upgrade utility (FMUP.EXE; download able from iPAN) has three options for BIOS upgrades:

· 	The FLASH BIOS can be updated from a file on a disk;

· 	The current BIOS code can be copied from the FLASH EEPROM to a disk file as a backup in the event that an upgrade cannot be successfully completed; and

· 	The BIOS in the FLASH device can be compared with a disk file to ensure the system has the correct BIOS version.

### Flash User Area

Classic/PCI i486 Baby-AT supports a 4 KB programmable Flash User area located at ED000H-EDFFFH. A programmer may use this area to display a customized message or to execute a small program. The Classic/PCI i486 Baby-AT BIOS accesses the user area just after completing the POST (Power-On Self-Test) if the setup option is enabled. The flash user area may be updated by running the FMUP.EXE utility, which expects the update files to have a .USR extension. Sample programs and instructions are in the file CLSUSER.ZIP on the iPAN bulletin board.

## Keyboard (and Mouse) Interface

An Intel 8742 surface mount micro controller contains the Phoenix Technologies' compatible keyboard/mouse controller code.  An AT style keyboard connector is located on the back panel side of the motherboard. The 5V line on this connector is protected with a PolySwitch circuit which acts much like a fuse except that it re-establishes the connection after an over-current condition is removed. While the PolySwitch eliminates the possibility of having to replace a fuse, care should be taken to turn the system power off before installing or removing a keyboard. As a manufacturing option, customers whose chassis will allow two PS/2 style connectors, one for mouse and one for keyboard, can be supported by offering PS/2 configuration instead of AT. The 8742 micro controller code supports Power-On/Reset (POR), network, and keyboard password protection.  Network and keyboard passwords require programs contained on the utility disk that ships with the system, the POR password is set via the SETUP program.   In addition, the keyboard controller provides for the following "HOT" key sequences:

·	CTRL-ALT-DEL: System software reset. This sequence performs a software reset of the system by jumping to the beginning of the BIOS code and running the POST operation, excluding memory tests.

·	<TBD 1> and <TBD 2>: Turbo mode selection. <TBD 1> sets the system for de-turbo mode (emulation of an 8 MHz 80286 CPU using wait states) and <TBD 2> sets the system for turbo mode (its normal operation at 33 MHz). Changing the Turbo mode may be prohibited by an operating system or application software.

## System Memory

The Classic/PCI i486 Baby-AT Motherboard provides four 36-bit wide SIMM sites for memory expansion with single/dual sided SIMM modules.  The memory array is controlled by the Intel 82424TX CDC, and data buffering is provided by an Intel 82423TX DPU.  The four SIMM sites support 256K x 36, 512K x 36, 1M x 36, 2M x 36, 4M x 36 and 8M x 36 SIMM modules.  Minimum memory size is 2 MB, and maximum memory size, using four 8M x 36 SIMM modules, is 128 MB.

Memory is always interleaved in two banks; therefore the SIMM sites must be stuffed in pairs. Memory timing is designed for 70ns fast page devices, faster DRAMs will operate in the board but will provide no performance improvement.  Parity generation/checking is provided for each 8-bit byte.

SIMMs may be installed in combinations of two or four modules; each two SIMMs within an interleaved bank must be of the same memory size and type (see the Appendix for a complete list of combinations). There are no jumper settings required for the memory size configuration, the System BIOS automatically sizes memory and initializes the 82424TX DRAM controller for appropriate DRAM configuration.

## Core Chip Set

The core chip set is the Intel Saturn chip set, consisting of one 82424TX Cache/DRAM Controller (CDC), one 82423TX Data Path Unit (DPU) device, and one 82378IB System I/O (SIO) bridge chip. This document will outline the general functionality, for more detailed information refer to the data sheet for the 82420 PCISet from Intel. The Saturn chip set provides the following functions:

· 	CPU reset control

· 	CPU L1 cache control

· 	CPU burst mode control

· 	CPU interface control

· 	Integrated L2 write-back cache controller with tag comparator

·	Page-mode DRAM controller

· 	Burst memory read/write control logic

· 	Data bus conversion to PCI

· 	Parity generation/detection to memory

· 	AT-BUS direction control

· 	Chip select for keyboard controller and RTC

· 	Speaker control

· 	NMI logic

· 	Floating-point coprocessor interface

· 	Keyboard reset and gate A20 emulation logic

· 	DMA controller

· 	Interrupt controller

· 	Counters/Timers

### 82424TX Cache/DRAM/Controller (CDC)

The 82424TX provides all control signals necessary to drive the DRAM array, including multiplexed address signals.  It also controls system access to memory and generates snoop controls to maintain cache coherency.

### 82423TX Data Path Unit (DPU)

The 82423TX provides data bus buffering and dual port buffering to the memory array. Controlled by the 82424TX, the 82423TX device adds one load to the PCI bus and performs all the necessary byte and word swapping required. Memory and I/O write buffers are included in these devices.

### 82378IB System I/O (SIO)

The 82378IB integrates seven 32-bit DMA channels, five 16-bit timer/counters, two eight-channel interrupt controllers, NMI logic, refresh address generation, and PCI/ISA bus arbitration circuitry together onto the same device.

## Expansion Slots

The Classic/PCI i486 Baby-AT Motherboard contains support for up to seven populated expansion slots, offering ISA and PCI connectors.  These connectors include four ISA bus expansion slots, and two PCI expansion slots; the seventh slot uses both an ISA connector and a PCI connector side by side, and can accept either an ISA or PCI adapter board but not both together.  The expansion cards are oriented perpendicular to the motherboard.  All three PCI expansion slots accept PCI master cards, fully supporting the PCI specification.

## SMC 37C663 Super I/O Controller

Control for the integrated serial ports, parallel port, floppy drive and IDE hard drive interface is incorporated into a single component, the SMC FDC37C663. This component provides:

·	Two NS16C552 compatible UARTs (with FIFO support)

·	IBM and Centronics compatible bi-directional parallel port controller

·	Industry standard floppy controller (with 2.88 MB floppy support)

·	IDE hard disk decode and chip select

Header connectors are available near the back of the board for cabling these options.  The serial ports can be enabled as COM1 and COM2 or disabled.  The parallel port can be enabled via the SETUP program as LPT1 or disabled, and can be set as bi-directional or output only when enabled.

## Dallas DS12887 Real Time Clock, CMOS RAM and Battery

The Real Time Clock (RTC) is implemented using a Dallas DS12887 device.  The DS12887 is accurate to within 13 minutes/year and requires no external support (the battery and oscillator are integrated into the device).  The component is soldered into the board (the internal battery has an estimated lifetime of ten years).

The RTC can be set via the BIOS SETUP Program.  CMOS memory supports the standard 128-byte battery-backed RAM, fourteen bytes for clock and control registers, and 114 bytes of general purpose non-volatile CMOS RAM.  All CMOS RAM is reserved for BIOS use.  The CMOS RAM can be set to specific values or cleared to the system default values using the BIOS SETUP program. Also, the CMOS RAM values can be cleared to the system defaults by using a hardware jumper. The appendices contain a list of jumper configurations.

## Front Panel Connectors

A connector (J1C1) is provided for installing a speaker. The speaker provides error beep code information during the Power-On Self Test if the system cannot use the video interface. The Classic/PCI i486 Baby-AT product guide contains beep and error code information.

Connectors J1B2, J1B1, J1C1B, and J1A2, J1A1 supply front panel connections to Reset, Key lock, and Turbo switches, and hard disk and turbo LEDs, respectively.

## Security

Security features are incorporated into the Classic/PCI i486 Baby-AT system BIOS.

### BIOS Password

A BIOS password feature provides security during the boot process. A password can be entered using the Setup utility and must be re-entered prior to disk boot each time the system is reset. The password can be changed at the password prompt by entering <old password> / <new password> / <new password> <enter>.  The password also can be cleared by entering <old password> / <enter>. If the password is forgotten, it can be cleared by turning off the system and setting jumper PW to the right. After the system has finished the Power-On Self Test, turn the system off and reset jumper PW to the left. This allows the user access to the password feature, but with the forgotten password cleared.

### Setup Enable Jumper

A jumper on the baseboard controls access to the BIOS Setup utility. By setting jumper SE to the right, the user is prevented from accessing the Setup utility during the Power-On Self Test or at any other time.

# System Integration Features

## Back panel Connections

The back panel provides external access to the keyboard controller integrated on the Classic/PCI i486 Baby-AT motherboard. Figure 4 shows the back panel connections.

µ §

Figure 2. Classic/PCI i486 Baby-AT Back panel

## Power Supply

The Classic/PCI i486 Baby-AT board could be powered by a 200 watt switch able power supply, providing power for onboard resources, add-in cards, and peripherals.

| DC Voltage | Max. Continuous Current | Peak Current 15 Seconds | Minimum Current Load |
| --- | --- | --- | --- |
| +5V | 18.5A | 18.5 | 2.5A |
| -5V | 0.9A | 0.9A | 0A |
| +12V | 4.6A | 9.5A | 0.5A |
| -12V | 0.5A | 0.5A | 0A |

Table 2. Classic/PCI i486 Baby-AT Current Requirements

Table 2 lists the current used by system resources. This information is preliminary and is provided only as a guide for calculating approximate total system power usage with additional resources added.

| Resource | Typical Power |
| --- | --- |
| Classic/PCI i486 Baby-AT baseboard, 16 MB (8 MB in each of two banks), 128K cache | 50 Watts |
| Teac 3½" Floppy drive | 1.7 Watts |

Table 3. Current Use by System Resources (Preliminary)

# Appendices

## Appendix A - User-Installable Upgrades

### System Memory

Table A-1 shows the total system memory based on the listed combinations of SIMMs in the four SIMM sockets.

| SIMM 1,2 (Bank 0) SIMM Type (Amount) | SIMM 3,4 (Bank 1) SIMM Type (Amount) | Total System Memory |
| --- | --- | --- |
| 256K X 36 (1 MB) | Empty | 2 MB |
| 256K X 36 (1 MB) | 256K X 36 (1 MB) | 4 MB |
| 256K X 36 (1 MB) | 512K X 36 (2 MB) | 6 MB |
| 256K X 36 (1 MB) | 1M X 36 (4 MB) | 10 MB |
| 256K X 36 (1 MB) | 2M X 36 (8 MB) | 18 MB |
| 256K X 36 (1 MB) | 4M X 36 (16 MB) | 34 MB |
| 256K X 36 (1 MB) | 8M X 36 (32 MB) | 66 MB |
| 512K X 36 (2 MB) | Empty | 4 MB |
| 512K X 36 (2 MB) | 256K X 36 (1 MB) | 6 MB |
| 512K X 36 (2 MB) | 512K X 36 (2 MB) | 8 MB |
| 512K X 36 (2 MB) | 1M X 36 (4 MB) | 12 MB |
| 512K X 36 (2 MB) | 2M X 36 (8 MB) | 20 MB |
| 512K X 36 (2 MB) | 4M X 36 (16 MB) | 36 MB |
| 512K X 36 (2 MB) | 8M X 36 (32 MB) | 68 MB |
| 1M X 36 (4 MB) | Empty | 8 MB |
| 1M X 36 (4 MB) | 256K X 36 (1 MB) | 10 MB |
| 1M X 36 (4 MB) | 512K X 36 (2 MB) | 12 MB |
| 1M X 36 (4 MB) | 1M X 36 (4 MB) | 16 MB |
| 1M X 36 (4 MB) | 2M X 36 (8 MB) | 24 MB |
| 1M X 36 (4 MB) | 4M X 36 (16 MB) | 40 MB |
| 1M X 36 (4 MB) | 8M X 36 (32 MB) | 72 MB |
| 2M X 36 (8 MB) | Empty | 16 MB |
| 2M X 36 (8 MB) | 256K X 36 (1 MB) | 18 MB |
| 2M X 36 (8 MB) | 512K X 36 (2 MB) | 20 MB |
| 2M X 36 (8 MB) | 1M X 36 (4 MB) | 24 MB |
| 2M X 36 (8 MB) | 2M X 36 (8 MB) | 32 MB |
| 2M X 36 (8 MB) | 4M X 36 (16 MB) | 48 MB |
| 2M X 36 (8 MB) | 8M X 36 (32 MB) | 80 MB |
| 4M X 36 (16 MB) | Empty | 32 MB |
| 4M X 36 (16 MB) | 256K X 36 (1 MB) | 34 MB |
| 4M X 36 (16 MB) | 512K X 36 (2 MB) | 36 MB |
| 4M X 36 (16 MB) | 1M X 36 (4 MB) | 40 MB |
| 4M X 36 (16 MB) | 2M X 36 (8 MB) | 48 MB |
| 4M X 36 (16 MB) | 4M X 36 (16 MB) | 64 MB |
| 4M X 36 (16 MB) | 8M X 36 (32 MB) | 96 MB |
| 8M X 36 (32 MB) | Empty | 64 MB |
| 8M X 36 (32 MB) | 256K X 36 (1 MB) | 66 MB |
| 8M X 36 (32 MB) | 512K X 36 (2 MB) | 68 MB |
| 8M X 36 (32 MB) | 1M X 36 (4 MB) | 72 MB |
| 8M X 36 (32 MB) | 2M X 36 (8 MB) | 80 MB |
| 8M X 36 (32 MB) | 4M X 36 (4 MB) | 72 MB |
| 8M X 36 (32 MB) | 8M X 36 (8 MB) | 128 MB |

Table A-1. SIMM Memory  Stuffing Options

### Performance Upgrade

There are several CPU upgrade paths. If you have an i486 SX processor, you can upgrade it with an Intel487 SX, an i486 DX processor, an i486 DX2, or an OverDrive Processor (including Pentium processor-based OverDrive components, when available). Systems with an i486 DX can be upgraded with i486 DX2 or OverDrive components, and an i486 DX2 can be upgraded with the Pentium CPU-based OverDrive component when available. Upgrading requires removing the current CPU from the 238-pin ZIF socket on the system board. The upgrade processor is plugged into the socket and the system powered up; it's that easy. When the next generation i486 processors which require 3.3V are available, they can also be used to upgrade the board; in this case, Jumpers CPUVCC must be changed to provide the 3.3V to the processor. Intel487 and OverDrive components are available from Intel's Personal Computer Enhancement Division. For the location of the nearest Intel dealer, phone 1 (800) 538-3373.

## µ §

Just Look for the Blue ZIF Socket!

### Qualified DRAM SIMMs

| Vendor | Part Number | Size (configuration) |
| --- | --- | --- |
| Toshiba | THM361010AS-70 |  |
| Toshiba | THM361020AS-70 |  |
| Samsung | KMM5361000A-7 |  |
| Samsung | KMM5361000B-7 |  |
| Hitachi | HB56D136SBS-7A |  |
| Micron | MT9D136M-7 |  |
| MTI | Z124MBK36A-70I1 |  |
| MTI | Z124MBK36B-70I1 |  |
| MTI | Z124MBK36R-70IN |  |

Table A-2. Sampling of memory DRAM SIMM Vendors

## Appendix B - Jumpers

All the configuration jumpers on Classic/PCI i486 Baby-AT motherboard are the three pin, two position type. These sit side by side in a 2x3 jumper block. The jumper description below assumes a  particular point of reference in viewing the motherboard. Position the board  or system so that the CPU and Cache Rams are at the front nearest you, the Dram SIMMs should be to the right. All functional jumpers have a  two-letter indicator as shown in the table. Use these indicators to locate and position the jumpers for your configuration. Some jumpers can be positioned top or bottom, (bottom is towards the front of the board)  others left or right.

| INDICATOR | DESCRIPTION | DEFAULT |
| --- | --- | --- |
| CP | Cache Present Jumper. Jumper to the top position if no Cache Ram is installed in the Board. Jumpered to the bottom indicates Cache Ram is installed in the board. | Bottom |
| RV | Recovery Jumper. Jumper to top for normal operation Jumper to the bottom for BIOS RECOVERY MODE. | Top |
| FI | IDE Paddle Card Jumper. Jumper to top to indicate PCI IDE card is installed in slot J3H1. Jumpered to the bottom for normal on board IDE operation. | Bottom |
| BW | BIOS Write Jumper. Jumper to the top to disable all writes to Flash EPROM. Jumpered to the bottom allows for Flash EPROM Updates. | Bottom |
| CPUVCC | CPUVCC Jumpers. These jumpers are not user configurable. Moving them can result in damage to the CPU!! Left is for 5 volt CPU operation. Right is for 3.3 volt operation of 5 volt tolerant CPUs. All four jumpers must be positioned to the same side. | Left |
| CL | Color/Mono Jumper. Jumpered to the left is for MGA operation. Jumpered to the right is for color graphics adapters. | Right |
| CM | Clear CMOS Jumper. Jumper to the left will force CMOS to be CLEARED. Jumper to the right for normal operation. | Right |
| SE | Setup Enable Jumper. Jumper to the left to allow setup to be enabled during Boot-Up. Jumper to the right to disable setup during Boot-up. | Left |
| PW | Password Jumper. Jumper to the left for normal operation. Jumper to the right to clear the password that was previously selected. | Left |

Table B-1. Classic/PCI i486 Baby-AT Jumper Description

µ §

Figure B-1. Classic/PCI i486 Baby-AT Jumper Locations

## Appendix C - BIOS Setup Options

### TBD

## Appendix D - BIOS Recovery

The Classic/PCI i486 Baby-AT incorporates an AMI system BIOS on a FLASH component. FLASH BIOS allows easy upgrades without the need to replace an EPROM. The upgrade utility fits on a floppy diskette and provides the capability to save, verify, and update the system BIOS. The upgrade utility can be run from a hard drive or a network drive, but no memory managers can be installed during upgrades.

The latest upgrade utility and BIOS code are available in the public section of the iPAN bulletin board.

### Using the Upgrade Utility

If the utility is obtained from iPAN, UNZIP the archive and copy the files to a bootable MS-DOS 3.3, 4.01,  5.0, or 6.0 bootable diskette. Reboot the system with the upgrade diskette in the bootable floppy drive and follow the directions in the easy to use menu-driven program.

### Recovery Mode

In the unlikely event that a FLASH upgrade is interrupted catastrophically, it is possible the BIOS may be left in an unusable state. Recovering from this condition requires the following steps (be sure a power supply has been attached to the board, and a floppy drive is connected as drive A:):

1.  Move jumper RV to the bottom position.

2.  Install the bootable upgrade diskette into drive A:

3.  Reboot the system.

4.  Because of the small amount of code available in the non-erasable boot block area, no video is available to direct the procedure. The procedure can be monitored by listening to the speaker and looking at the floppy drive LED. When the system beeps and the floppy drive LED is lit, the system is copying the recovery code into the FLASH device. As soon as the drive LED goes off, the system can be turned off.

5.  Reset jumper RV to the top position.

6.  Leave the upgrade floppy in drive A: and turn the system on.

7.  Continue with the original upgrade.

## Appendix E - Memory Map

| Address Range (Decimal) | Address Range (hex) | Size | Description |
| --- | --- | --- | --- |
| 16384K-32768K | 1000000-2000000 | 16384K | Extended Memory |
| 16256K-16383K | FE0000-FFFFFF | 128K | System & Video BIOS Copy |
| 1024K-16255K | 100000-FDFFFF | 15232K | Extended Memory |
| 960K-1023K | F0000-FFFFF | 64K | AMI System BIOS |
| 952K-959K | EE000-EFFFF | 8K | FLASH Boot Block (Available as HIMEM) |
| 948K-951K | ED000-EDFFF | 4K | User FLASH Area (available as HIMEM if no user info is here) |
| 928K-947K | E8000-ECFFF | 20K | AMI Setup Program (disable via setup pre-boot only; with this option open to HIMEM) |
| 896K-927K | E0000-E7FFF | 32K | Video BIOS (when installed) |
| 800K-895K | C8000-DFFFF | 96K | Available Hi DOS Memory(open to the ISA bus) |
| 768K-799K | C000-C7FFF | 32K | Optional VGA BIOS |
| 736K-767K | B8000-BFFFF | 32K | VGA Display Memory (not available to ISA bus) |
| 704K-735K | B0000-B7FFF | 32K | VGA & Mono Display Memory (HIMEM w/ QEMM)(not available to the bus) |
| 640K-703K | A0000-AFFFF | 64K | VGA Display Memory (not available to the ISA bus) |
| 639K | 9FC00-9FFFF | 1K | Extended BIOS Data (moveable by QEMM, 386MAX) |
| 512K-638K | 80000-9FBFF | 127K | Extended conventional |
| 0K-511K | 00000-7FFFF | 512K | Conventional |

Table E-1. Classic/PCI i486 Baby-AT Memory Map

## Appendix F - I/O Map

| Address Range (hex) | Size (Decimal) | Description |
| --- | --- | --- |
| 0000 - 000F | 16 bytes | SIO - DMA 1 |
| 0020 - 0021 | 2 bytes | SIO - Interrupt Controller 1 |
| 0040 - 0043 | 4 bytes | SIO - Timer 1 |
| 0048 - 004B | 4 bytes | SIO - Timer 2 |
| 0060 | 1 byte | Keyboard Controller Data Byte |
| 0061 | 1 byte | SIO - NMI, speaker control |
| 0064 | 1 byte | Keyboard Controller, CMD & STATUS Byte |
| 0070, bit 7 | 1 bit | SIO - Enable NMI |
| 0070, bits 6:0 | 7 bits | SIO - Real Time Clock, Address |
| 0071 | 1 byte | SIO - Real Time Clock, Data |
| 0073 | 1 byte | Reserved - Board Configuration |
| 0075 | 1 byte | Reserved - Board Configuration |
| 0078 | 1 byte | SIO - BIOS Timer |
| 0080 - 008F | 16 bytes | SIO - DMA Page Register |
| 00A0 - 00A1 | 2 bytes | SIO - Interrupt Controller 2 |
| 00C0 - 00DE | 31 bytes | SIO - DMA 2 |
| 00F0 | 1 bytes | Reset Numeric Error |
| 0170 - 0177 | 8 bytes | Secondary IDE Channel |
| 01F0 - 01F7 | 8 bytes | Primary IDE Channel |
| 0278 - 027B | 4 bytes | Parallel Port 2 |
| 02F8 - 02FF | 8 bytes | On-Board Serial Port 2 |
| 0376 | 1 byte | Secondary IDE Channel Command Port |
| 0377 | 1 byte | Secondary IDE Channel Status Port |
| 0378 - 037F | 8 bytes | Parallel Port 1 |
| 03BC - 03BF | 4 bytes | Parallel Port x |
| 03E8 - 03EF | 8 bytes | Serial Port 3 |
| 03F0 - 03F5 | 6 bytes | Floppy Channel 1 |
| 03F6 | 1 bytes | Primary IDE Channel Command Port |
| 03F7 (Write) | 1 byte | Floppy Channel 1 Command |
| 03F7, bit 7 | 1 bit | Floppy Disk Change Channel 1 |
| 03F7, bits 6:0 | 7 bits | Primary IDE Channel Status Port |
| 03F8 - 03FF | 8 bytes | On-Board Serial Port 1 |
| 0CF8 | 1 byte | PCI Configuration Space Enable |
| 0CF9 | 1 byte | De-turbo Mode Enable |
| C000 - C0FF | 256 bytes | 82424TX Configuration Registers |
| C200 - C2FF | 256 bytes | 82378IB Configuration Registers |

Table F-1. Classic/PCI i486 Baby-AT I/O Address Map

## Appendix G - Board Interrupts & DMA

| Interrupt Request | System Resource |
| --- | --- |
| NMI | Parity Error |
| 0 | Reserved, Interval Timer |
| 1 | Reserved, Keyboard buffer full |
| 2 | Reserved, Cascade interrupt from slave PIC |
| 3 | Serial Port 2 |
| 4 | Serial Port 1 |
| 5 | Parallel Port 2 |
| 6 | Floppy |
| 7 | Parallel Port 1 |
| 8 | Real Time Clock |
| 9 | User available |
| 10 | User available |
| 11 | User available |
| 12 | On-board Mouse Port if enabled, else user available |
| 13 | Reserved, Math coprocessor |
| 14 | IDE if enabled |
| 15 | User available |

Table G-1. Classic/PCI i486 Baby-AT Interrupt Map

| DMA Channel | Data Width | System Resource |
| --- | --- | --- |
| 0 | 8-or 16-bits | Open |
| 1 | 8-or 16-bits | Open - Normally used for LAN |
| 2 | 8-or 16-bits | Floppy |
| 3 | 8-or 16-bits | IDE |
| 4 |  | Reserved - Cascade channel |
| 5 | 16-bits | Open |
| 6 | 16-bits | Open |
| 7 | 16-bits | Open |

Table G-2. Classic/PCI i486 Baby-AT DMA Map

## Appendix H - Connectors

### AT Style Keyboard Port (J8L1 = Keyboard)

| Pin Number | Signal Name |
| --- | --- |
| 1 | Clock |
| 2 | Data |
| 3 | No Connect |
| 4 | Ground |
| 5 | Vcc (fused) |

Table H-1. AT Style Keyboard Connector Pin Definition

### Optional PS/2 Style Keyboard, Mouse Ports (J8L2 = Keyboard, J9L1 = Mouse)

| Pin Number | Signal Name |
| --- | --- |
| 1 | Data |
| 2 | No Connect |
| 3 | Ground |
| 4 | Vcc (fused) |
| 5 | Clock |
| 6 | No Connect |

Table H-2. PS/2 Mouse and Keyboard Connector Pin Definition

### Turbo LED Connector (J1A1)

| Pin Number | Signal Name |
| --- | --- |
| 1 | PULL_UP_330 |
| 2 | LED_TURBO- |

Table H-3. Turbo LED Connector Pin Definition

### Hard Drive LED Connector (J1A2)

| Pin Number | Signal Name |
| --- | --- |
| 1 | PULL_UP_330 |
| 2 | HD ACTIVE- |
| 3 | Key |
| 4 | PULL_UP_330 |

Table H-4 Fixed Disk LED Connector Pin Definition

### Key lock/Power LED Connector (J1B1)

| Pin Number | Signal Name |
| --- | --- |
| 1 | LED_PWR |
| 2 | Key |
| 3 | Ground |
| 4 | KEY LOCK |
| 5 | Ground |

Table H-5. Key lock Connector Pin Definition

### Speaker Connector (J1C1)

| Pin Number | Signal Name |
| --- | --- |
| 1 | SPKR_DAT |
| 2 | Key |
| 3 | No Connect |
| 4 | +5V Vcc |

Table H-6. Speaker Connector Pin Definition

### Turbo Switch Connector (J1C1B)

| Pin Number | Signal Name |
| --- | --- |
| 1 | TURBO |
| 2 | Ground |
| 3 | No Connect |

Table H-7. Turbo Switch Connector Pin Definition

### Primary Power Connector (J9K1)

| Pin | Name | Function |
| --- | --- | --- |
| 1 | PWRGD | Power Good |
| 2 | +5 V | + 5 volts Vcc |
| 3 | +12 V | + 12 volts |
| 4 | -12 V | - 12 volts |
| 5 | GND | Ground |
| 6 | GND | Ground |
| 7 | GND | Ground |
| 8 | GND | Ground |
| 9 | -5 V | -5 volts |
| 10 | +5 V | + 5 volts Vcc |
| 11 | +5 V | + 5 volts Vcc |
| 12 | +5 V | + 5 volts Vcc |

Table H-8. Power Connector Pin Definition

### Auxiliary (3.3V) Power Connector (J4H1)

| Pin | Name | Function |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | GND | Ground |
| 3 | GND | Ground |
| 4 | +3.3 V | + 3.3 volts |
| 5 | +3.3V | + 3.3 volts |
| 6 | +3.3 V | + 3.3 volts |

Table H-9. 3.3V Power Connector Pin Definition

### Parallel Port (J7J1)

| Signal Name | Pin Number | Pin Number | Signal Name |
| --- | --- | --- | --- |
| Strobe- | 1 | 2 | Auto Feed- |
| Data Bit 0 | 3 | 4 | ERROR- |
| Data Bit 1 | 5 | 6 | INIT- |
| Data Bit 2 | 7 | 8 | SLCT IN- |
| Data Bit 3 | 9 | 10 | Ground |
| Data Bit 4 | 11 | 12 | Ground |
| Data Bit 5 | 13 | 14 | Ground |
| Data Bit 6 | 15 | 16 | Ground |
| Data Bit 7 | 17 | 18 | Ground |
| ACK- | 19 | 20 | Ground |
| BUSY | 21 | 22 | Ground |
| PE (Paper End) | 23 | 24 | Ground |
| SLCT | 25 | 26 | No Connect |

Table H-10. Printer Port Connector Pin Definition

### Serial Ports (J7L2 = COM1, J7L1 = COM2)

| Signal Name | Pin Number | Pin Number | Signal Name |
| --- | --- | --- | --- |
| DCD (Data Carrier Detect) | 1 | 2 | DSR (Data Set Ready) |
| SIN- (Serial Input) | 3 | 4 | RTS (Request to Send) |
| SOUT- (Serial Output) | 5 | 6 | CTS (Clear to Send) |
| DTR (Data Terminal Ready) | 7 | 8 | RI (Ring Indicator) |
| Ground | 9 | 10 | No Connect |

Table H-11. Serial Port Connectors Pin Definition

### Reset Connector (J1B2)

| Pin Number | Signal Name |
| --- | --- |
| 1 | RESET |
| 2 | Ground |

Table H-12. Reset Switch Connector Pin Definition

### Floppy Connector (J9F2)

| Signal Name | Pin Number | Pin Number | Signal Name |
| --- | --- | --- | --- |
| Ground | 1 | 2 | FDHDIN |
| Ground | 3 | 4 | Reserved |
| Key | 5 | 6 | FDEDIN |
| Ground | 7 | 8 | Index- |
| Ground | 9 | 10 | Motor Enable A- |
| Ground | 11 | 12 | Drive Select B- |
| Ground | 13 | 14 | Drive Select A- |
| Ground | 15 | 16 | Motor Enable B- |
| Ground | 17 | 18 | DIR- |
| Ground | 19 | 20 | STEP- |
| Ground | 21 | 22 | Write Data- |
| Ground | 23 | 24 | Write Gate- |
| Ground | 25 | 26 | Track 00- |
| Ground | 27 | 28 | Write Protect- |
| Ground | 29 | 30 | Read Data- |
| Ground | 31 | 32 | Side 1 Select- |
| Ground | 33 | 34 | Diskette Change- |

Table H-13. Floppy Disk Connector Pin Definition

### IDE Connector (J9F3)

| Signal Name | Pin Number | Pin Number | Signal Name |
| --- | --- | --- | --- |
| Reset IDE | 1 | 2 | Ground |
| Host Data 7 | 3 | 4 | Host Data 8 |
| Host Data 6 | 5 | 6 | Host Data 9 |
| Host Data 5 | 7 | 8 | Host Data 10 |
| Host Data 4 | 9 | 10 | Host Data 11 |
| Host Data 3 | 11 | 12 | Host Data 12 |
| Host Data 2 | 13 | 14 | Host Data 13 |
| Host Data 1 | 15 | 16 | Host Data 14 |
| Host Data 0 | 17 | 18 | Host Data 15 |
| Ground | 19 | 20 | Key |
| DRQ3 | 21 | 22 | Ground |
| I/O Write- | 23 | 24 | Ground |
| I/O Read- | 25 | 26 | Ground |
| IOCHRDY | 27 | 28 | BALE |
| DACK3- | 29 | 30 | Ground |
| IRQ14 | 31 | 32 | IOCS16- |
| Addr 1 | 33 | 34 | Ground |
| Addr 0 | 35 | 36 | Addr 2 |
| Chip Select 0- | 37 | 38 | Chip Select 1- |
| Activity | 39 | 40 | Ground |

Table H-14. IDE Connector Pin Definition

### ISA Connector (J6G2, J6G1, J5G1, J4G1, J1G1)

| Signal Name | Pin Number | Pin Number | Signal Name |
| --- | --- | --- | --- |
| GND | B1 | A1 | IOCHK- |
| RSTDRV | B2 | A2 | SD7 |
| Vcc | B3 | A3 | SD6 |
| IRQ9 | B4 | A4 | SD5 |
| -5V | B5 | A5 | SD4 |
| DRQ2 | B6 | A6 | SD3 |
| -12V | B7 | A7 | SD2 |
| 0WS- | B8 | A8 | SD1 |
| +12V | B9 | A9 | SD0 |
| GND | B10 | A10 | IOCHRDY |
| SMEMW- | B11 | A11 | AEN |
| SMEMR- | B12 | A12 | SA19 |
| IOW- | B13 | A13 | SA18 |
| IOR- | B14 | A14 | SA17 |
| DACK3- | B15 | A15 | SA16 |
| DRQ3 | B16 | A16 | SA15 |
| DACK1- | B17 | A17 | SA14 |
| DRQ1 | B18 | A18 | SA13 |
| REFRESH- | B19 | A19 | SA12 |
| SYSCLK | B20 | A20 | SA11 |
| IRQ7 | B21 | A21 | SA10 |
| IRQ6 | B22 | A22 | SA9 |
| IRQ5 | B23 | A23 | SA8 |
| IRQ4 | B24 | A24 | SA7 |
| IRQ3 | B25 | A25 | SA6 |
| DACK2- | B26 | A26 | SA5 |
| TC | B27 | A27 | SA4 |
| BALE | B28 | A28 | SA3 |
| Vcc | B29 | A29 | SA2 |
| OSC | B30 | A30 | SA1 |
| GND | B31 | A31 | SA0 |
|  | KEY | KEY |  |
| MEMCS16- | D1 | C1 | SBHE- |
| IOCS16- | D2 | C2 | LA23 |
| IRQ10 | D3 | C3 | LA22 |
| IRQ11 | D4 | C4 | LA21 |
| IRQ12 | D5 | C5 | LA20 |
| IRQ15 | D6 | C6 | LA19 |
| IRQ14 | D7 | C7 | LA18 |
| DACK0- | D8 | C8 | LA17 |
| DRQ0 | D9 | C9 | MEMR- |
| DACK5- | D10 | C10 | MEMW- |
| DRQ5 | D11 | C11 | SD8 |
| DACK6- | D12 | C12 | SD9 |
| DRQ6 | D13 | C13 | SD10 |
| DACK7- | D14 | C14 | SD11 |
| DRQ7 | D15 | C15 | SD12 |
| Vcc | D16 | C16 | SD13 |
| Master- | D17 | C17 | SD14 |
| GND | D18 | C18 | SD15 |

Table H-15. ISA Connector Pin Definition

### PCI Connector (J3H1,J2H1,J1H1)

| Signal Name | Pin Number | Pin Number | Signal Name |
| --- | --- | --- | --- |
| GND | A1 | B1 | -12V |
| +12V | A2 | B2 | TCK |
| No Connect | A3 | B3 | GND |
| STDI | A4 | B4 | STD0 |
| Vcc | A5 | B5 | Vcc |
| PCIINT3- | A6 | B6 | Vcc |
| PCIINT1- | A7 | B7 | PCIINT2- |
| Vcc | A8 | B8 | PCIINT4- |
| Reserved | A9 | B9 | No Connect |
| Vcc | A10 | B10 | Reserved |
| Reserved | A11 | B11 | No Connect |
| GND | A12 | B12 | GND |
| GND | A13 | B13 | GND |
| Reserved | A14 | B14 | Reserved |
| SPCIRST- | A15 | B15 | GND |
| Vcc | A16 | B16 | PCLKE |
| AGNT- | A17 | B17 | GND |
| GND | A18 | B18 | REQA- |
| Reserved | A19 | B19 | Vcc |
| AD30 | A20 | B20 | AD31 |
| 3.3V | A21 | B21 | AD29 |
| AD28 | A22 | B22 | GND |
| AD26 | A23 | B23 | AD27 |
| GND | A24 | B24 | AD25 |
| AD24 | A25 | B25 | 3.3V |
| AD22 (IDSEL) | A26 | B26 | CBE3- |
| 3.3V | A27 | B27 | AD23 |
| AD22 | A28 | B28 | GND |
| AD20 | A29 | B29 | AD21 |
| GND | A30 | B30 | AD19 |
| AD18 | A31 | B31 | 3.3V |
| AD16 | A32 | B32 | AD17 |
| 3.3V | A33 | B33 | CBE2- |
| FRAME- | A34 | B34 | GND |
| GND | A35 | B35 | IRDY- |
| TRDY- | A36 | B36 | 3.3V |
| GND | A37 | B37 | DEVSEL- |
| STOP- | A38 | B38 | GND |
| 3.3V | A39 | B39 | PLOCK- |
| SDONE | A40 | B40 | PERR- |
| SBO- | A41 | B41 | 3.3V |
| GND | A42 | B42 | SERR- |
| PAR | A43 | B43 | 3.3V |
| AD15 | A44 | B44 | CBE1- |
| 3.3V | A45 | B45 | AD14 |
| AD13 | A46 | B46 | GND |
| AD11 | A47 | B47 | AD12 |
| GND | A48 | B48 | AD10 |
| AD9 | A49 | B49 | GND |
| key | A50 | B50 | key |
| key | A51 | B51 | key |
| CBE0- | A52 | B52 | AD8 |
| 3.3V | A53 | B53 | AD7 |
| AD6 | A54 | B54 | 3.3V |
| AD4 | A55 | B55 | AD5 |
| GND | A56 | B56 | AD3 |
| AD2 | A57 | B57 | GND |
| AD0 | A58 | B58 | AD1 |
| Vcc | A59 | B59 | Vcc |
| SREQ64- | A60 | B60 | SACK64- |
| Vcc | A61 | B61 | Vcc |
| Vcc | A62 | B62 | Vcc |

Table H-16. PCI Connector Pin Definition

## Appendix I - Baby-AT Chassis Suppliers

Axxion Group Corporation

11 B. Leigh Fisher

El Paso, TX  79906

(915) 772-0088

Enlight Corporation, USA

345 Cloverleaf Drive, Unit 2B

Baldwin Park, CA  91706

(818) 369-4709

Olsen Metal Products

1001 Crossroads Boulevard

Seguin, TX  78155

(512) 379-2799

Suntek Corporation

xxx

xxx

(xxx) xxx-xxxx

Kepro Corporation

xxxx

xxx

(xxx) xxx-xxxx

## Appendix J - Environmental Standards

| Parameter | Condition | Specification |
| --- | --- | --- |
| Temperature |  |  |
|  | Non-Operating | -40oC to +70oC |
|  | Operating | +0oC to +55oC |
| Humidity |  |  |
|  | Non-Operating | 92% Relative Humidity max. @ 36oC |
|  | Operating | 80% Relative Humidity max. @ 36°C |
| Altitude |  |  |
|  | Non-Operating | 50,000 feet (15,240 meters) |
|  | Operating | 10,000 feet (3048 meters) |
| ESD |  |  |
|  | 1.0kV | No Errors |
|  | 2.5kV | No Errors |
|  | 5.0kV | 5% Soft Errors, 0% Hard Errors, No physical damage |
|  | 7.5kV | 10% Soft Errors, 0% Hard Errors, No physical damage |
|  | 10.0kV | 25% Soft Errors, 5% Hard Errors, No physical damage |
|  | 12.5kV | 50% Soft, 10% Hard, No physical damage |
|  | 15.0kV | 100% Soft, 25% Hard, No physical damage |
|  | 25.0kV | 100% Soft, 100% Hard, No physical damage |
| Shock | Non-Operating | 30.0G, 11ms, 1/2 sine |

Table J-1. Environmental Standards

## Appendix K - Reliability Data

This Mean-Time-Between-Failures (MTBF) data is calculated from predicted data @ 35C.

Classic/PCI i486 Baby-AT baseboard (BP5D60LP8)			xx,xxx hours

## Appendix L - Software Driver Support

### TBD

## Appendix M - Qualified Peripherals

### TBD

## Appendix N - Customer Support

The Classic/PCI i486 Baby-AT is backed by Intel's industry-leading support groups in the OEM Products and Services Division (OPSD), including IntelTechDirectä. OPSD can support many of your network integration and service needs, including worldwide integration and system repair services. IntelTechDirect provides the following 4 major services:

### iPAN (Intel Product Assistance Network)

An electronic Bulletin board with current product information, demo software and more...

- Available worldwide through direct-dial

- Modem speeds up to 14.4k baud with standard software

- FLASH BIOS upgrade files

- Modem set at no parity, 8 data bits, 1 stop bit.

### iPUB (Intel Product Update Bulletin)

- Monthly Product updates available 24 hours a day from iPAN

- Official notification of engineering changes and technical data

- Easy information retrieval using Windows Help file format

- Intel platform system, board, and BIOS revision histories

- Hardware and software compatibility notes

- Documentation updates, spare parts and order information

### iPALS (Intel Phone Action Line Support)

A direct telephone support line backed by highly qualified and well trained technical personnel.

- Toll-free access to Intel support engineers for problem resolution

- Responses within 24 hours Monday-Friday

- Expert assistance geared to the special needs of OEMs and VARs

### FaxBack™

- Product descriptions and technical data sent to any fax machine from a touch-tone phone

- Information on End-of-Life products

- Available worldwide through direct dial at 916-356-3105

For information about IntelTechDirect please contact your local Intel Sales Representative.

## Appendix O - Physical Dimensions

### Board

| Length | xxx cm | 10.5" |
| --- | --- | --- |
| Width | xxx cm | 9.1" |

Table O-1. Board Outline Dimensions

µ §					Figure O-1. Motherboard Dimensions

## Appendix P - Product Codes

### Boards

BP4S33ATC8		Classic/PCI i486SX/33 Baby-AT, 128K cache, 8M memory

BP4D33ATC8		Classic/PCI i486DX/33 Baby-AT, 128K cache, 8M memory

BP4D266ATC8		Classic/PCI i486DX2/66 Baby-AT, 128K cache, 8M memory

(All boards are bulk shipped in quantities of 10)

### Accessories

Accessory kit includes:

### Documentation

The Classic/PCI i486 Baby-AT board ships with a product guide.