Trinity ATX (S1592) Quick Setup Guide

|  | The table below will help you set the jumpers for CPU speed, bus frequency, and memory voltage,among others. * This motherboard incorporates support for 75 MHz and 83 MHz BUS speeds. However, these speeds will push PCI bandwidth beyond the Intel recommended specification for PCI 2.1 compliance. Therefore, TYAN Computer Corporation takes no responsibility for any problems, be it hardware or software, related to the use of BUS speeds beyond 66MHz. We also cannot guarantee any PCI cards' compatibility beyond the specified speed of 33MHz. Technical support and RMA / Warranty claims relating to the use of BUS speeds beyond the 66 MHz / 33 MHz limit recommended by Intel will not be honored by TYAN. USE AT YOUR OWN RISK. ** The JPW1 setting for the Cyrix 6x86L series will vary. Some of the CPUs in this series are 2.8V, and others are 3.3V. Check the face of the CPU and the CPU Voltage Settings table for the proper settings for these jumpers. |
| --- | --- |

|  | Trinity Jumper Settings by CPU Type - Updated 7-14-98 |
| --- | --- |

|  | CPU; BUS Speed & Multiplier; JP3; JP4; JP5; JPW1 1-2; JPW1 3-4; JPW1 5-6; JPW1 7-8; JP10; JP11; JP12. Intel P54C 75; 50MHz x 1.5; OFF; OFF; OFF; ON; OFF; ON; ON; 2-3; 2-3; 2-3. Intel P54C 90; 60MHz x 1.5; OFF; OFF; OFF; ON; OFF; ON; ON; 2-3; 1-2; 1-2. Intel P54C 100; 66MHz x 1.5; OFF; OFF; OFF; ON; OFF; ON; ON; 1-2; 1-2; 1-2. Intel P54C 120; 60MHz x 2; OFF; OFF; ON; ON; OFF; ON; ON; 2-3; 1-2; 1-2. Intel P54C 133; 66MHz x 2; OFF; OFF; ON; ON; OFF; ON; ON; 1-2; 1-2; 1-2. Intel P54C 150; 60MHz x 2.5; OFF; ON; ON; ON; OFF; ON; ON; 2-3; 1-2; 1-2. Intel P54C 166; 66MHz x 2.5; OFF; ON; ON; ON; OFF; ON; ON; 1-2; 1-2; 1-2. Intel P54C 200; 66MHz x 3; OFF; ON; OFF; ON; OFF; ON; ON; 1-2; 1-2; 1-2. Intel P55C 166 MMX; 66MHz x 2.5; OFF; ON; ON; OFF; OFF; OFF; ON; 1-2; 1-2; 1-2. Intel P55C 200 MMX; 66MHz x 3; OFF; ON; OFF; OFF; OFF; OFF; ON; 1-2; 1-2; 1-2. Intel P55C 233 MMX; 66MHz x 3.5; OFF; OFF; OFF; OFF; OFF; OFF; ON; 1-2; 1-2; 1-2. AMD K5 100; 66MHz X 1.5; OFF; OFF; OFF; ON; OFF; ON; ON; 1-2; 1-2; 1-2. AMD K5 120; 60MHz x 2; OFF; OFF; ON; ON; OFF; ON; ON; 2-3; 1-2; 1-2. AMD K5 133; 66MHz x 2; OFF; OFF; ON; ON; OFF; ON; ON; 1-2; 1-2; 1-2. AMD K5 166; 66MHz x 2.5; OFF; ON; ON; ON; OFF; ON; ON; 1-2; 1-2; 1-2. AMD K6 166; 66MHz x 2.5; OFF; ON; ON; ON; OFF; OFF; ON; 1-2; 1-2; 1-2. AMD K6 200; 66MHz x 3; OFF; ON; OFF; ON; OFF; OFF; ON; 1-2; 1-2; 1-2. AMD K6 233; 66MHz x 3.5; OFF; OFF; OFF; OFF; ON; OFF; OFF; 1-2; 1-2; 1-2. AMD K6 266; 66MHz x 4; ON; OFF; ON; OFF; ON; OFF; OFF; 1-2; 1-2; 1-2. IDT C6 180; 60MHz x 3; OFF; ON; OFF; ON; OFF; ON; ON; 2-3; 1-2; 1-2. IDT C6 200; 66MHz x 3.5; OFF; ON; OFF; ON; OFF; ON; ON; 1-2; 1-2; 1-2. Cyrix PR120+ 6x86; 50MHz x 2; OFF; OFF; ON; ON; OFF; ON; ON; 2-3; 2-3; 2-3. Cyrix PR150+ 6x86; 60MHz x 2; OFF; OFF; ON; ON; OFF; ON; ON; 2-3; 1-2; 1-2. Cyrix PR166+ 6x86; 66MHz x 2; OFF; OFF; ON; ON; OFF; ON; ON; 1-2; 1-2; 1-2. Cyrix PR200+ 6x86; 75MHz x 2*; OFF; OFF; ON; ON; OFF; ON; ON; 1-2; 2-3; 1-2. Cyrix PR150+ 6x86L**; 60MHz x 2; OFF; OFF; ON; OFF; OFF; OFF; ON; 2-3; 1-2; 1-2. Cyrix PR166+ 6x86L**; 66MHz x 2; OFF; OFF; ON; OFF; OFF; OFF; ON; 1-2; 1-2; 1-2. Cyrix PR200+ 6x86L**; 75MHz x 2*; OFF; OFF; ON; OFF; OFF; OFF; ON; 1-2; 2-3; 1-2. Cyrix PR166+ 6x86MX; 60MHz x 2.5; OFF; ON; ON; ON; OFF; OFF; ON; 2-3; 1-2; 1-2. Cyrix PR166+ 6x86MX; 66MHz x 2; OFF; OFF; ON; ON; OFF; OFF; ON; 1-2; 1-2; 1-2. Cyrix PR200+ 6x86MX; 66MHz x 2.5; OFF; ON; ON; ON; OFF; OFF; ON; 1-2; 1-2; 1-2. Cyrix PR200+ 6x86MX; 75MHz x 2*; OFF; OFF; ON; ON; OFF; OFF; ON; 1-2; 2-3; 1-2. Cyrix PR200+ 6x86MX; 66MHz x 3; OFF; ON; OFF; ON; OFF; OFF; ON; 1-2; 1-2; 1-2. Cyrix PR233+ 6x86MX; 75MHz x 2.5*; OFF; ON; ON; ON; OFF; OFF; ON; 1-2; 2-3; 1-2. |
| --- | --- |

© Copyright 2001, Tyan Computer Corporation. All Rights Reserved.