### Up	Veriton 9100 (S80X for Acer) BIOS Release Notice

| Item Number | ###### Description | # Date | Fixed BIOS Version | Solution | Comment |
| --- | --- | --- | --- | --- | --- |
| 046 | #### WinME+USB mouse( plug in front panel )+SCSI card (2940U2W or 29160) will cause auto reboot issue | ## Jun. 21, 2001 | ## R01-D3 | Assign LAN IRQ 11 while booting | Must load default after flash BIOS |
| 045 | #### Fixed power button must pressed twice to boot while last shutdown was 4 seconds. | ## Jun. 21, 2001 | ## R01-D3 | Don’t cover 3Com LAN card issue |  |
| 044 | #### Fixed LDCM memory halt issue | ## Jun. 21, 2001 | ## R01-D3 |  |  |
| 043 | #### Add BBS and INT15 AX=9610h BL=06h 32 bits function for preload to call PnP 6xh function | ## Jun. 08, 2001 | ## R01-D2 |  | Support preload Lan download |
| 042 | #### Main Board S/N only show 16 characters | ## Jun. 08, 2001 | ## R01-D2 | Extend to show 21 characters | Middle East request |
| 041 | #### S3 resume sometime fail | ## Jun. 08, 2001 | ## R01-D2 | Don’t issue PCI reset if resuming from STR |  |
| 040 | #### There’s an error string in BIOS Setup item : [ Frequency Control ] / Processor & Processor Speed of fail rate 20% ( 6 of 30 pieces M/B ) | ## May. 16, 2001 | ## R01-D1 | The value of Processor item must be 0, it means processor’s model name. But now we have got some boards their initial value aren’t 0s. This cause unexpected strings showing in this item. So, we always force the item value to 0. This can avoid the problem. |  |
| 039 | #### Windows Me sometime install fail | ## May. 16, 2001 | ## R01-D1 | ##### Update Intel’s reference code 1.4, 1.5, 1.6 and 1.7 for new chipset version: ##### North Bridge: A3 #### South Bridge: B4 |  |
| 038 | #### Can not install Windows XP | ## May. 16, 2001 | ## R01-D1 | ##### Match for Microsoft’s workaround request: 1 Keep release date as 02/16/2001 2 Fixed LAN & USB Controller 2 the same IRQ11 and not sharing to others |  |
| 037 | #### LAN sometime losing | ## May. 16, 2001 | ## R01-D1 | #### Add PCI reset during cold boot |  |
| 036 | #### Add new Willamette CPU micro code support for CPUID=0F0Ah | ## May. 16, 2001 | ## R01-D1 |  |  |
| 035 | #### Change ACPI_OEMID to Acer | ## May. 16, 2001 | ## R01-D1 | #### Change “IntelR” to “Acer” |  |
| 034 | #### Phase in Acer new logo | ## May. 07, 2001 | R01-D0 | ##### Modified from R01-C3 |  |
| 033 | #### Can’t install Whistler | ## Apr. 18, 2001 | R01-C4 | Support APIC function |  |
| 032 | #### Windows Me can't install in MB -2A | ## Apr. 13, 2001 | R01-C4 | After add support PC600 code, it’s OK ! | Maybe to match up H/W by this way. |
| 031 | Support PC600 | Apr. 13, 2001 | R01-C4 |  | Auto detect. |
| 030 | LDCM Memory item show error ( DMI Table Type 17 ) | ## Apr. 13, 2001 | R01-C4 | Change “DIMM” to “RIMM” |  |
| 029 | ACPI_OEMID error | Apr. 13, 2001 | R01-C4 | Change “IntelR” to “Acer” |  |
| 028 | When we have 3Com add-on card (3c905c-txm) then system will auto boot when plug in power button electric wire | Apr. 13, 2001 | R01-C4 | The AC power always turn on to off after power failure |  |
| 027 | Release the formal production BIOS | Feb. 16, 2001 | R01-C3 | Change the BIOS revision from R01-C2T7 to R01-C3 | Checksum = 87C9 |
| 026 | Fix the issue of [PnP En/Disable FDD fail] | Feb. 15, 2001 | R01-C2T7 |  |  |
| 025 | Fix the issue of [S4 USB keyboard resumed too slowly] | Feb. 15, 2001 | R01-C2T7 |  |  |
| 024 | Restore the ATAPI MODE SENSE command to avoid AOpen CD-ROM fail | Feb. 15, 2001 | R01-C2T7 |  |  |
| 023 | For S4 USB keyboard resumed too slowly test | Feb. 15, 2001 | R01-C2T6 |  | Could not enter WinME S3 |
| 022 | For S4 USB keyboard resumed too slowly test | Feb. 14, 2001 | R01-C2T5 |  | S4 OK, but S3 fail |
| 021 | For S4 USB keyboard resumed too slowly test | Feb. 14, 2001 | R01-C2T4 |  | S3 OK, but S4 fail |
| 020 | Restore the ATAPI MODE SENSE command to avoid AOpen 48X CD-ROM POST fail | Feb. 14, 2001 | R01-C2T3 |  |  |
| 019 | For S4 USB keyboard resumed too slowly test | Feb. 14, 2001 | R01-C2T3 |  |  |
| 018 | For S4 USB keyboard resumed too slowly test | Feb. 13, 2001 | R01-C2T2 |  |  |
| 017 | For WHQL HCT test | Feb. 12, 2001 | R01-C2T1 |  |  |
| 016 | When LPT IRQ = 7, Intel Chipset Driver could not be installed. | Feb. 9, 2001 | R01-C2T0 | Let IRQ 9 sharable. | Please re-install the OS |
| 015 | Sometimes ATAPI devices could not be detected by W2K Device Manager. | Feb. 9, 2001 | R01-C2T0 | Phase in Award Hitachi IDE solution. |  |
| 014 | Change the POST screen hint message for Acer’s request | Feb. 7, 2001 | R01-C2 | Change the color from green t white, remove F9 key message and add F12 key message |  |
| 013 | Intel Chipset Driver could not be installed | Feb. 6, 2001 | R01-C1T4 | Change the default LPT IRQ from 7 to 5 |  |
| 012 | Enable [Wake-Up by PCI card] by default to avoid [Wake On LAN] fail | Feb. 5, 2001 | R01-C1T3 | Change the default value |  |
| 011 | Too short time for the user to press the combination keys to enter BIOS Setup | Feb. 5, 2001 | R01-C1T3 | Change the POST screen format and display the hint message at the right time |  |
| 010 | Sometimes entered BIOS Setup by USB keyboard would hang up | Feb. 2, 2001 | R01-C1T2 | Correct the programming error for combination keys |  |
| 009 | [Hitachi DVD + LG CD-RW] stopped at [Boot from ATAPI CD-ROM] | Feb. 2, 2001 | R01-C1T1 | Skip ATAPI MODE SENSE command |  |
| 008 | Wake On LAN from S5 | Feb. 2, 2001 | R01-C1 |  |  |
| 007 | Correct LDCM information | Feb. 2, 2001 | R01-C1 |  |  |
| 006 | Remove the [Evaluation ROM] string in POST screen | Feb. 1, 2001 | R01-C0 |  |  |
| 005 | Program onboard PCI device sub-vendor/sub-system ID | Feb. 1, 2001 | R01-C0 | Program sub vendor / sub system ID as 1025/000E |  |
| 004 | USB keyboard resumed too slowly from W2K S1 mode (over 10 seconds.) | Jan. 31, 2001 | R01-B4 |  |  |
| 003 | Gate A20 could not be disabled after [PQAF Cache CPU Reset Test]. | Jan. 31, 2001 | R01-B4 | Disable Port 92h in Shutdown A and B paths |  |
| 002 | Enhanced POST 10 sec function. | Jan. 19, 2001 | R01-B3 | No summary screen, set default to disable to “FDD seek”, always enable 32bit HDD access, enable L1 cache before VGA initialize |  |
| 001 | LDCM [System Manufacturer ¦ System Name] in the Windows Registry must be displayed as [Acer ¦ Veriton 9100]. | Jan. 19, 2001 | R01-B3 |  |  |