DCL51 BIOS 1.20A Release Check List

Test Purpose : Base on DCL51 BIOS version 1.20A to check basic function no bad than old version and confirm fixed items.

Test Configuration: Please refer to the system configuration below:

# System Configuration  			Release Date: 09/02/2003

| Test System | Compal Model Name → DCL51 Customer Model Name → |  |  |
| --- | --- | --- | --- |
| CPU Type | Intel Pentium-M 1.3 G | Chipset Software utility | Intel V4.30 Build 1006 |
| L2 Cache | 1 MB | VGA Driver | Intel V6.13.10.3514 |
| Memory Size | Samsung 128 MB | Audio Driver | Realtek V5.10.0.5210 |
| Hard Disk (F/W) | Hitachi 30GB | Modem Driver | Smart Link V3.4.1.168 |
| COMBO (F/W) | Toshiba SD-C2412 | LAN Driver | Realtek V5.505.1004.2002 |
| Internal FDD | N/A | Wireless LAN Driver | Intel V1.1.6.0 |
| Panel Type | Hitachi 15.0 SXGA+ | Easy Button Driver | Dritek V1.107 |
| Battery info | Sony 4 cells | Touch Pad Driver | Alps V5.3.203.13 |
| System BIOS | 1.20A | Bluetooth Driver | WIDCOMM V1.3.1.15 |
| Keyboard BIOS | 1.20A | SD controller Driver | Winbond V2.0.2003.320 |
| BOOTSEQ Ver. | 0.02A | IR controller Driver | SMC V5.1.2462.0 |
| Operation System | Windows XP Professional + SP1 |  |  |

Test procedure, criteria, result

Please refer to the tables below for testing procedure, criteria and result.

| Flash BIOS |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | ### Check Item | Pass | Fail | Remark |
| 1. | Make sure BIOS ROM check sum match BIOS release note. (Check sum: F435) | V |  |  |
| 2. | Check commend for flash BIOS is “FLASH016 CL51120A.ROM /allb /c”, please do not add extra parameter except special condition and must remove it in next version BIOS. | V |  |  |
| 3. | #### Check system will shut down after flash BIOS | V |  |  |
| 4. | Check BIOS version during POST procedure | V |  |  |
| 5. | Check WinFlash can flash new BIOS from old BIOS and auto restart system without error message, also check BIOS version is correct | V |  |  |
| 6. | Check WinFlash cannot flash same BIOS version |  |  | N/A |

| BIOS Release Note (Solved Issues) Check List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Issue No. | ### Check Item | Pass | Fail | Remark |
| 1. |  | ### When system show OSB LOGO it will cause one line of garbage be show on the left-down screen. | V |  |  |

| System Function Check List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | ### Boot Test |  | Pass | Fail | Remark |
| Please check system can boot from designated device, no hang up, no too slowly for item 2 -12. |  |  |  |  |  |
| 1. | Check BIOS default Boot sequence is FDD-HDD-CDROM-LAN. |  | V |  |  |
| 2. | Cold Boot from USB FDD |  | V |  |  |
| 3. | Warm Boot from USB FDD (Ctrl+Alt+Del) |  | V |  |  |
| 4. | Cold Boot from HDD |  | V |  |  |
| 5. | Warm Boot from HDD |  | V |  |  |
| 6. | Cold Boot from CD ROM. |  | V |  |  |
| 7. | Warm Boot from CD-ROM. |  | V |  |  |
| 8. | Cold Boot from Network. |  | V |  |  |
| 9. | Warm Boot from Network. |  | V |  |  |
| 10. | Press “ESC” key to pop boot menu for select booting from which device during POST of cold boot/warm boot. |  | V |  |  |
| No. | ### Boot Sequence utility Test |  | Pass | Fail | Remark |
| Please check utility can change boot sequence. |  |  |  |  |  |
| 1. | Check Boot sequence utility (BOOTSEQ) work fine under DOS mode. |  | V |  |  |
| 2. | Check Boot sequence utility (WBOOTSEQ) work fine under Windows mode. |  |  |  | N/A |
| No. | ### VBIOS SSC function Test |  | Pass | Fail | Remark |
| 1. | Please get the SSC test utility from \\tpe_sa4\document\SA4 A-Test Plan\Test Tools\SSC utility and it under DOS commend. Please check test result same as below. Spread Spectrum Enabled Spread selected for spread = M(Memory) P2(PCLK) Spread Down at 1.25% |  |  |  | N/A |
| No. | ### Power On Test by Manually |  | Pass | Fail | Remark |
| Please check system can boot, no hang up and no slowly. |  |  |  |  |  |
| 1. | Press the Power Button to power system on with AC adapter only. (3 times) |  | V |  |  |
| 2. | Press the Power Button to power system on with Main Battery only. (3 times) |  | V |  |  |
| 3. | Press the Power Button to power system, then insert USB device when BIOS logo appear. (Please check USB HDD, FDD, Mouse) |  | V |  |  |
| No. | ### Operating System/Driver Installation |  | Pass | Fail | Remark |
| 1. | Install the Operating System by manually, please check system no hang up and no slowly. |  | V |  |  |
| 2. | Install all drivers without hang or any error. (No “!” or “?” mark in Device management) |  | V |  |  |
| 3. | Make sure the DMI string (COMPAL DMI v3.0) can be written into system. (Pure DOS mode) Please restart system to DOS mode and read it by utility to confirm it is OK. And please test it 5 times. |  | V |  |  |
| 4. | Restore the Operating System from the recovery CD without any error or hang up. |  |  |  | N/A |
| No. | ### MDP check |  | Pass | Fail | Remark |
| 1. Please use Recovery CD. 2. Please do disk defragment. 3. Please use Bootvis utility to Optimize system, you can get it from \\tpe_sa4\document\SA4 A-Test Plan\Test Tools\MDP utility. (1). Copy this "Bootvis.exe" to Desktop (2). Run "Bootvis"--->Trace--Optimize System (3). you must wait for few minutes (4). When finish, start to test time |  |  |  |  |  |
| 1. | Make sure the MDP cold boot time is less than 35 seconds (If double byte OS is 40 seconds) |  | V |  |  |
| 2. | Make sure the MDP warm boot time is less than 35 seconds (If double byte OS is 40 seconds) |  | V |  |  |
| 3. | Make sure the MDP standby resume time is less than 5 seconds. |  | V |  |  |
| 4. | Make sure the MDP hibernation resume time is less than 25 seconds. |  | V |  |  |
| No. | ### Hot Key Function Check |  | Pass | Fail | Remark |
| 1. | Fn + ESC | Sleep button | V |  |  |
| 2. | Fn + F5 | Power on display switch sequence : LCD only, LCD+CRT, CRT only, TV+LCD, TV only (If CRT or TV not be applied, all of CRT or all of TV items will be ignored.) | V |  |  |
| 3. | Fn + F10 | Pad keypad ON/OFF | V |  |  |
| 4. | Fn + F11 | Numeric keypad ON/OFF | V |  |  |
| 5. | Fn + End | Master Volume ON/OFF | V |  |  |
| 6. | Fn +  | Increase brightness | V |  |  |
| 7. | #### Fn +  | #### Decrease brightness | #### V |  |  |
| 8. | #### Fn + PgUp | Increase Volume | #### V |  |  |
| 9. | #### Fn + PgDn | Decrease Volume | #### V |  |  |
| 10. | Press Fn+F5 without monitor connection, the display mode should not do any change. |  | V |  |  |
| 11. | Check sticky key function for above hot keys. |  | #### V |  |  |
| 12. | Resume from Standby/Hibernate, and then check sticky key function for above hot keys. |  | #### V |  |  |
| No. | ### Dritek Easy Button Function Check |  | Pass | Fail | Remark |
| 1. | #### Easy Button function check (Two user define button, On screen display of Fn + ,, PgUp, PgDn ) with Dritek Easy Button program installed under OS. |  | V |  |  |
| 2. | #### Check Easy Button function after Cold boot. |  |  | V | #1 |
| 3. | Check Easy Button function after Restart |  |  | V | #1 |
| 4. | Check Easy Button function after resume from Standby |  |  | V | #1 |
| 5. | Check Easy Button function after resume from Hibernate |  |  | V | #1 |
| No. | ### Limited user Test |  | Pass | Fail | Remark |
| 1. Go to Control Panel -> User Accounts. 2. Click “Create a new account”. 3. Type a name for the new account and click “next”. 4. Choice “Limited” for account type. 5. Click “Create Account”. 6. Restart system and login this limited user account. |  |  |  |  |  |
| 1. | Check no any error messages appear. |  | V |  |  |
| 2. | Check all above hot keys and sticky key function work fine. |  |  | V | #1 |
| 3. | Check above easy button function under OS mode work fine. |  | V |  |  |
| No. | ### Peripheral Device Function Check (Check two times) |  | Pass | Fail | Remark |
| 1. | Check all storage devices (USB FDD, HDD, CD/DVD-ROM/CD-RW etc.) are working properly. |  | V |  |  |
| 2. | Make sure all storage devices (USB FDD, HDD, CD/DVD-ROM/CD-RW etc.) are working fine after resume from standby/Hibernate/Restart/Cold boot. |  | V |  |  |
| 3. | Check the USB keyboard and mouse are working properly. |  | V |  |  |
| 4. | Make sure the USB keyboard and mouse are working fine after resume from standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 5. | Check the USB 2.0 HDD (IBM Travelstar Serial# D9HYZM7B) is working properly. (With WinXP SP1) |  | V |  |  |
| 6. | Make sure the USB 2.0 HDD (IBM Travelstar Serial# D9HYZM7B) is working fine after resume from standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 7. | Make sure 1394 GUID can be read/written from system. (Write a 1394 GUID into system, and then read for 3 times to check.) |  | V |  |  |
| 8. | Check the 1394 NIC is working fine. |  | V |  |  |
| 9. | Make sure the 1394 NIC are working fine after resume from standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 10. | Check the 1394 device (IBM Travelstar Serial# D9HYZM7B) is working properly. |  | V |  |  |
| 11. | Make sure the 1394 Device (IBM Travelstar Serial# D9HYZM7B) is working fine after resume from Standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 12. | Check the integrated Modem is working properly. |  | V |  |  |
| 13. | Make sure the integrated Modem is working fine after resume from Standby/Hibernate /Restart/Cold boot. |  | V |  |  |
| 14. | Check the on board LAN is working properly. |  | V |  |  |
| 15. | Make sure the on board LAN card is working fine after resume from Standby/ Hibernate/Restart/Cold boot |  | V |  |  |
| 16. | Check the integrated Wireless LAN is working properly. |  | V |  |  |
| 17. | Make sure the integrated Wireless LAN card is working fine after resume from Standby/Hibernate/Restart/Cold boot |  | V |  |  |
| 18. | Check the LPT port (HP LaserJet 5000) function is working fine. (Without any error message) |  | V |  |  |
| 19. | Make sure the LPT port (HP LaserJet 5000) is working fine after resume from Standby/Hibernate/Restart/Cold boot. (Without any error message) |  | V |  |  |
| 20. | Check the FIR function by transfer files between two FIR systems. |  | V |  |  |
| 21. | Make sure the FIR transfer can work between FIR systems after resume from Standby/Hibernate/Restart/Cold boot. |  | V |  |  |
| 24. | Check the SD card (128MB) are working properly. |  | V |  |  |
| 25. | Make sure the SD card (128MB) are working fine after resume from Standby/Hibernate/Restart/Cold boot. |  | V |  |  |
| 26. | Check the integrate Bluetooth module is working properly. |  | V |  |  |
| 27. | Make sure the integrate Bluetooth module is working fine after resume from Standby/Hibernate/Restart/Cold boot. |  |  | V | #2 |
| 28. | Notices that fan have been turned on when system power on. |  | V |  |  |
| No. | ### ACPI Power Management (Standby, Hibernation/Resume) Check |  | Pass | Fail | Remark |
| 1. Please check function work, system no hang up and no error message for below items. 2. Please check mouse and keyboard are working after resume. 3. For time out and scheduled task test, please check timing no delay. |  |  |  |  |  |
| 1. | Standby by Power Button (Total 10 times, after Standby for 5 times do cold boot once.) |  | V |  |  |
| 2. | Standby by close panel for 10 seconds. (Lid switch set to “Stand By “) (Total 10 times, after Standby for 5 times do cold boot once.) |  | V |  |  |
| 3. | Standby by Start Menu \Shut Down \Standby. (Total 10 times, after Standby for 5 times do cold boot once.) |  | V |  |  |
| 4. | Standby by timeout (Total 10 times, after Standby for 5 times do cold boot once.) |  | V |  |  |
| 5. | Hibernate by Power Button (Total 10 times, after Hibernation for 5 times do cold boot once.) |  | V |  |  |
| 6. | Hibernate by close panel for 10 seconds. (Lid switch set to “Hibernate”) (Total 10 times, after Hibernate for 5 times do cold boot once.) |  | V |  |  |
| 7. | Hibernate by Start Menu \Shut Down \Shift -> Hibernate. (Total 10 times, after Hibernate for 5 times do cold boot once.) |  | V |  |  |
| 8. | Hibernate by timeout (Total 10 times, after Hibernate for 5 times do cold boot once.) |  | V |  |  |
| 9. | Screen saver, Turn off monitor, Standby, Hibernate by timeout (Total 5 times, Set Screen saver(default) for 1 min, Turn off monitor for 2 mins, Standby for 3 mins, Hibernate for 5 mins.) |  | V |  |  |
| 10. | Wake from Standby by Scheduled Tasks. Test Procedures: - Click Start/Programs/Accessories/System Tools/Scheduled Tasks. - Double Click on Add Scheduled Task icon. - Click "Next" button. - Select "Calculator" then click "Next" button. - Mark "Daily" then click "Next" button. - Set the Start Time to 3 min. advance from your current time. - Set the Start Date to current date, click "Next" button. - Accept default, click "Next" button. - Check "Open advance properties.....", click "Finish" button. - Click "Settings" tab. - Under Power Management, Check "Wake the computer to run this task." - Click "Apply" button, click "OK" button. - Enter Standby mode. - Wait for 3 minutes, system will automatically resume from standby mode. |  | V |  |  |
| 11. | Wake from Hibernate by Scheduled Tasks. Test Procedures: 1. Base on 10. and enter Hibernate mode. - Wait for 3 minutes, system will automatically resume from hibernation mode. |  | V |  |  |
| 12. | Wake on Ring (Use Hyper terminal to test. After system was waked, the screen should keep black until you touch mouse or keyboard. And system still can work on call out without disconnect and re-create a new connection.) |  | V |  |  |
| 13. | Wake on LAN (After system was waked, the screen should keep black until you touch mouse or keyboard.) |  | V |  |  |
| 14. | Wake on USB device, force system enter Standby with USB mouse then click mouse left button to wake. (Please install HCT tool to test, HCT tool will enabled wake from USB function). |  | V |  |  |
| 15. | Force system enter Standby, plug in USB device, check system will not auto resume. |  | V |  |  |
| 16. | Check all LEDs indicators are working fine and correct. |  | V |  |  |
| 17. | Check all LEDs are working properly after resume from Standby, Hibernate |  | V |  |  |
| 18. | Check Fan is turned off in Standby/Hibernation mode. |  | V |  |  |
| 19. | Force the system entering standby by close panel (Lid switch set to Standby) during shutdown the system. |  | V |  |  |
| 20. | Force the system entering Hibernate by close panel (Lid switch set to Hibernate) during shutdown the system. |  | V |  |  |
| 21. | Make sure the low/critical battery alarm tone was generated and keep auto update remaining capacity of the battery. |  | V |  |  |
| 22. | Go to Start -> All Programs -> Accessories -> System Tools -> System Information -> Components -> Problem Devices, check no any error. |  | V |  |  |
| 23. | Go to Control panel -> Administrative Tools -> Event Viewer -> System, check no any unexpected error. (Please review its detail to determine, most of network and IR error are acceptable) |  | V |  |  |
| 24. | The battery life corresponds to the Spec. |  | V |  |  |
| No. | ### PCMCIA Function Check (Please follow procedures as below) |  | Pass | Fail | Remark |
| 1. Make sure the system can auto-detect the device while hot insert the PCMCIA card. 2. Make sure the PC card driver have setup complete without any error or hang up. 3. Make sure no exclamation mark in device manager when PC card was inserted. 4. Check the PCMCIA card function is working properly. 5. Make sure the both upper socket and lower socket are working properly. 6. Make sure no exclamation mark in device manager after resume from Standby/Hibernate with PC card inserted. 7. Make sure the both upper socket and lower socket are working properly after resume from Standby/Hibernate. 8. Make sure the device does not list in device management after plug out the PC card. 9. Plug in two difference type of PC card into upper socket and lower socket, and then check item 1 to item 8. |  |  |  |  |  |
| 1. | Check the APA – 1480 SCSI Card (UltraSCSI) function |  | V |  |  |
| 2. | #### Check the APA – 1460D SCSI Card (Fast SCSI) function |  | V |  |  |
| 3. | #### Check the 3Com Megahertz 10/100 LAN+56K Modem (3CCFEM556B) function |  | V |  |  |
| 4. | #### Check the Xircom CBE2-100 Card Bus Ethernet card function |  | V |  |  |
| 5. | #### Check the IBM 512MB Microdrive CF+ Type II (DSCM-10512) function |  | V |  |  |

Comments: #1. Pressing two user define button right away after resume from S3,S4 and Cold boot/Reboot will popup www ,e-mail application (SPEC : User define button). #2. Bluetooth will abnormal after cold boot. (Kill switch set on ,Bluetooth enable only, after cold boot and wireless LAN led light) # During switching to TV out will appear ripple and slowly.

Conclusion:

Base on above test result, this version BIOS V1.20A is better than old version. We agree to release BIOS V1.20A

Department: Software Application Dept.

Engineer: Rebecca Chang