DCL51 BIOS V2.10 Release Check List

Test Purpose: Base on DCL51 BIOS V2.10 to check basic function no bad than old version and confirm fixed items.

Test Form Version: V 1.0.2

Test Configuration: Please refer to the system configuration below:

# System Configuration  		    Release Date: June/16/04’

| Test System | Compal Model Name →DCL51 SKU-10 |  |  |
| --- | --- | --- | --- |
| CPU Type | INTEL Pentium M 745 (1.80GHz) | Driver Package | V0.08 |
| L2 Cache | 2MB | Chipset Software utility | Intel V5.1.0.1007 |
| Memory Size | Samsung 256MB | VGA Driver | Intel V6.14.10.3553 |
| Hard Disk (F/W) | Hitachi 60G | Audio Driver | Realtek V5.10.0.5210 |
| CD-ROM (F/W) | Toshiba SD-C2412(Mitsumi) | FIR Driver | SMSC V5.1.2462.0 |
| Internal FDD | N/A | Modem Driver | Smart Link V3.04.01E |
| Panel Type | Hitachi 15” SXGA | LAN Driver | Realtek V5.505.1004.2002 |
| Battery info | Sony 4 Cells | Wireless LAN Driver | Intel V1.1.6.0 |
| System BIOS | V2.10 | Card Reader Driver | Winbond V2.0.2003.320 |
| Keyboard BIOS | V2.10 | Touch Pad Driver | ALPS V5.3.203.13 |
| BOOTSEQ Ver. | V0.0.02 | Bluetooth Driver | N/A |
| WBOOTSEQ Ver. | N/A | PCMCIA Controller Driver | ENE V5.1.2600.1010 |
| Operation System | Windows XP Pro+SP1 Ver.EN |  |  |

Test procedure, criteria, result

Please refer to the tables below for testing procedure, criteria and result.

| Flash BIOS |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | ### Check Item | Pass | Fail | Remark |
| 1. | Make sure BIOS ROM check sum match BIOS release note. (Check sum: F4CF ) | V |  |  |
| 2. | Check commend for flash BIOS is “FLASH016 CL51210A.ROM /allb /c”, please do not add extra parameter except special condition and must remove it in next version BIOS. | V |  |  |
| 3. | #### Check system will shutdown after flash BIOS (depend on BIOS Spec) | V |  |  |
| 4. | Check BIOS version during POST procedure | V |  |  |
| 5. | Check WinFlash can plash new BIOS from old BIOS and auto restart system without error message, also check BIOS version is correct(depend on BIOS Spec ) | V |  | PS#1 |
| 6. | Check WinFlash cannot plash same BIOS version(depend on BIOS Spec ) |  | V | #1 |

| BIOS Release Note (Solved Issues) Check List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Issue Part. | ### Check Item | Pass | Fail | Remark |
| 1. |  | ### Complet SpeedStep functions for Dothan. | V |  |  |
| 2. |  | ### Add microcode for Dothan B1 stepping. | V |  |  |
| 3. |  | ### Remove AC_OFF event of BATT_UVP from AD channel. | V |  |  |
| 4. |  | ### Modify BATT_UVP's protection from AD channel: ### Old: Turn off system if detect voltage <= 12V from AD channel ### New: Turn off system if system at discharge mode and detect ### voltage <= 11.6V from AD channel during 2 seconds. | V |  |  |
| 5. |  | ### If full condition of battery from SMBus is set but voltage from ### SMBus is <=16.2V, EC will recharge battery 1 hr. | V |  |  |

| System Function Check List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | ### Boot Test |  | Pass | Fail | Remark |
| Please check system can boot from designated device, no hang up, no too slowly for item 2 -12. |  |  |  |  |  |
| 1. | Check BIOS default Boot sequence is [ FDD-HDD-CDROM-LAN ] (depend on BIOS Spec ) |  | V |  |  |
| 2. | Cold Boot from USB FDD |  | V |  |  |
| 3. | Warm Boot from USB FDD (Ctrl+Alt+Del) |  | V |  |  |
| 4. | Cold Boot from HDD |  | V |  |  |
| 5. | Warm Boot from HDD |  | V |  |  |
| 6. | Cold Boot from CD ROM. |  | V |  |  |
| 7. | Warm Boot from CD-ROM. |  | V |  |  |
| 8. | Cold Boot from Network. |  | V |  |  |
| 9. | Warm Boot from Network. |  | V |  |  |
| 10. | Press [“ESC ”] key to pop boot menu for select booting from which device during POST of cold boot/warm boot. (depend on BIOS Spec ) |  | V |  |  |
| No. | ### Boot Sequence utility Test ### ( When first version utility release and golden BIOS release ) |  | Pass | Fail | Remark |
| Please check utility can change boot sequence, please confirm it by press ESC when system boot up. (depend on BIOS Spec ) |  |  |  |  |  |
| 1. | Check Boot sequence utility (BOOTSEQ) work fine under DOS mode. |  | V |  |  |
| 2. | Check Boot sequence utility (WBOOTSEQ) work fine under Windows mode. |  |  |  | N/A |
| No. | VBIOS SSC function Test ( When VBIOS version change ) |  | Pass | Fail | Remark |
| 1. | Please get the SSC test utility from\\tpe_sa3\Test Tools\A-Test Tools\SSC utility and it under DOS commend. Please check test result same as below. Spread Spectrum Enabled Spread selected for spread = M(Memory) P2(PCLK) Spread Down at 1.25% |  |  |  | N/A |
| No. | ### Power On Test by Manually |  | Pass | Fail | Remark |
| Please check system can boot, no hang up and no slowly. |  |  |  |  |  |
| 1. | Press the Power Button to power system on with AC adapter only. (3 times) |  | V |  |  |
| 2. | Press the Power Button to power system on with Main Battery only. (3 times) |  | V |  |  |
| 3. | Press the Power Button to power system, then insert USB device when BIOS logo appear. (Please check USB HDD, FDD, mouse) |  | V |  |  |
| 4. | Press the Power Button twice quickly to check if entering OS successfully. |  | V |  |  |
| No. | ### Operating System/Driver Installation |  | Pass | Fail | Remark |
| 1. | Install the Operating System by manually, please check system no hang up and no slowly. |  | V |  |  |
| 2. | Install all drivers without hang or any error . (No “!” or “?” mark in Device management) |  | V |  |  |
| 3. | Make sure the DMI string can be written into system. ( DMI ) Please restart system to DOS mode and read it by utility to confirm it is OK. And please test it 5 times. ( depend on BIOS team and project ) |  | V |  |  |
| 4. | Restore the Operating System from the recovery CD without any error or hang up. |  |  |  | N/A |
| No. | ### MDP check |  | Pass | Fail | Remark |
| - Please use Recovery CD if ready. - Please use stopwatch to measure. 3.Cold boot time: To measure the time period from the point of “Pressing the power button” to “Windows login screen was shown up”, The way to enable login screen is set the password to the user you use. 4.Warm boot time: To measure the time period from the point of “black screen was shown up (after the “windows is shutting down…” screen) to “Windows login screen was shown up”. 5. Resume from Standby/Hibernation: To measure the time period from the point of “Pressing the power button” to “Windows screen was shown up” |  |  |  |  |  |
| 1. | Make sure the MDP cold boot time is less than 35 seconds (If double byte OS is 45 seconds) |  | V |  |  |
| 2. | Make sure the MDP warm boot time is less than 35bseconds (If double byte OS is 45 seconds) |  | V |  |  |
| 3. | Make sure the MDP standby resume time is less than 5 seconds. |  | V |  |  |
| 4. | Make sure the MDP hibernation resume time is less than 25 seconds. |  | V |  |  |
| No. | ### Hot Key Function Check |  |  |  |  |
| 1. | Fn + F5 | Force Switching Display Mode(LCD->CRT->Simultaneous) | V |  |  |
| 2. | Fn + F11 | Numeric keypad on/off | V |  |  |
| 3. | Fn + F12 | Scroll Lock on/off | V |  |  |
| 4. | Fn + ↑ | Increase brightness (total 10 levels) | V |  |  |
| 5. | #### Fn + ↓ | #### Decrease brightness (total 10 levels) | #### V |  |  |
| 6. | #### Fn+ → | #### Increase Volume (total 16 levels) | #### V |  |  |
| 7. | #### Fn+ ← | #### Decrease Volume (total 16 levels) | #### V |  |  |
| 8. | #### Fn+ END | #### Mute on/off | #### V |  |  |
| 9. | Press Fn+F5 without monitor connection, the display mode should not do any change. |  | V |  |  |
| No. | ### [ Dritek Launch Manager ] Function Check |  | Pass | Fail | Remark |
| 1. | #### Launch Manager function check (Two user define button, On screen display of Fn + , ) with Dritek Easy Button program installed under OS. |  | V |  |  |
| 2. | #### Check Launch Manager function after Cold boot. |  | V |  |  |
| 3. | Check Launch Manager function after Restart |  | V |  |  |
| 4. | Check Launch Manager function after resume from Standby |  | V |  |  |
| 5. | Check Launch Manager after resume from Hibernate |  | V |  |  |
| No. | ### Peripheral Device Function Check (Check two times) |  | Pass | Fail | Remark |
| 1. | Check all storage devices (FDD, USB FDD, HDD, CD/DVD-ROM/CD-RW etc.) are working properly. |  | V |  |  |
| 2. | Make sure all storage devices (FDD, USB FDD, HDD, CD/DVD-ROM etc.) are working fine after resume from standby/Hibernate/Restart/Cold boot. |  | V |  |  |
| 3. | Check the USB keyboard and mouse are working properly. |  | V |  |  |
| 4. | Make sure the USB keyboard and mouse are working fine after resume from standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 5. | Check the USB 2.0 HDD (LaCie U&I Serial# 32411330) is working properly. |  | V |  |  |
| 6. | Make sure the USB 2.0 HDD (LaCie U&I Serial# 32411330) is working fine after resume from standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 7. | Make sure 1394 GUID ( [command] )can be read/written from system.(Write a 1394 GUID into system, and then read for 3 times to check.) ( depend on 1394 MAC address burning utility ) |  | V |  |  |
| 8. | Check the 1394 NIC is working fine. |  | V |  |  |
| 9. | Make sure the 1394 NIC are working fine after resume from standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 10. | Check the 1394 device (LaCie U&I Serial# 32411330) is working properly. |  | V |  |  |
| 11. | Make sure the 1394 Device (LaCie U&I Serial# 32411330 ) is working fine after resume from Standby /Hibernate/Restart/Cold boot. |  | V |  |  |
| 12. | Check the integrated Modem is working properly. |  | V |  |  |
| 13. | Make sure the integrated Modem is working fine after resume from Standby/Hibernate /Restart/Cold boot. |  | V |  |  |
| 14. | Check the on board LAN is working properly. |  | V |  |  |
| 15. | Make sure the on board LAN card is working fine after resume from Standby/ Hibernate/Restart/Cold boot |  | V |  |  |
| 16. | Check the integrated Wireless LAN is working properly. |  | V |  |  |
| 17. | Make sure the integrated Wireless LAN card is working fine after resume from Standby/Hibernate/Restart/Cold boot |  | V |  |  |
| 18. | Check the LPT port (hp deskjet 450) function is working fine. (Without any error message) |  | V |  |  |
| 19. | Make sure the LPT port (hp deskjet 450) is working fine after resume from Standby/Hibernate/Restart/Cold boot. (Without any error message) |  | V |  |  |
| 20. | Check the FIR function by transfer files between two FIR systems. |  | V |  |  |
| 21. | Make sure the FIR transfer can work between FIR systems after resume from Standby/Hibernate/Restart/Cold boot. |  | V |  |  |
| 22. | Check the Smart Media card ([ memory space ]) are working properly. |  |  |  | N/A |
| 23. | Make sure the Smart Media card ([ memory space ]) are working fine after resume from Standby/Hibernate/Restart/Cold boot. |  |  |  | N/A |
| 24. | Check the Memory Stick card ([ memory space ]) are working properly. |  |  |  | N/A |
| 25. | Make sure the Memory Stick card are working fine after resume from Standby/Hibernate/Restart/Cold boot. |  |  |  | N/A |
| 26. | Check the SD card (64MB) are working properly. |  | V |  |  |
| 27. | Make sure the SD card (64MB) are working fine after resume from Standby/Hibernate/Restart/Cold boot. |  | V |  |  |
| 28. | Check the integrate Bluetooth module is working properly. |  |  |  | N/A |
| 29. | Make sure the integrate Bluetooth module is working fine after resume from Standby/Hibernate/Restart/Cold boot. |  |  |  | N/A |
| 30. | Notices that fan have been turned on when system power on. |  | V |  |  |
| No. | ### ACPI Power Management (Standby, Hibernation/Resume) Check |  |  |  |  |
| 1. | Standby by Power Button ( Total 5 times ) |  | V |  |  |
| 2. | Standby by close panel for 10 seconds. (Lid switch set to “Stand By “) ( Total 5 times ) |  | V |  |  |
| 3. | Standby by Start Menu \Shut Down \Standby. ( Total 5 times ) |  | V |  |  |
| 4. | Standby by timeout ( Total 5 times ) |  | V |  |  |
| 5. | Hibernate by Power Button ( Total 5 times ) |  | V |  |  |
| 6. | Hibernate by close panel for 10 seconds. (Lid switch set to “Hibernate”) ( Total 5 times ) |  | V |  |  |
| 7. | Hibernate by Start Menu \Shut Down \Shift -> Hibernate. ( Total 5 times ) |  | V |  |  |
| 8. | Hibernate by timeout ( Total 5 times ) |  | V |  |  |
| 9. | Hibernate Issue 01 Check: Procedures: 1. Press power button when entering Hibernate (Correct: System will automatically resume from Hibernate and work normally without any noise) |  | V |  |  |
| 10. | Hibernate Issue 02 Check: Procedures: 1. Enter Hibernate 2. Press some keys when resuming from Hibernate (Correct: System resume from Hibernate successfully) |  | V |  |  |
| 11. | Hibernate Issue 03 Check: Procedures: 1. Enter Hibernate 2. Press some keys 3. Resume from Hibernate (Correct: System resume from Hibernate successfully) |  | V |  |  |
| 12. | Screen saver, Turn off monitor, Standby, Hibernate by timeout (Total 5 times, Set Screen saver(default) for 1 min, Turn off monitor for 2 mins, Standby for 3 mins, Hibernate for 5 mins.) |  | V |  |  |
| 13. | Wake from Standby by Scheduled Tasks. Test Procedures: - Click Start/Programs/Accessories/System Tools/Scheduled Tasks. - Double Click on Add Scheduled Task icon. - Click "Next" button. - Select "Calculator" then click "Next" button. - Mark "Daily" then click "Next" button. - Set the Start Time to 3 min. advance from your current time. - Set the Start Date to current date, click "Next" button. - Accept default, click "Next" button. - Check "Open advance properties.....", click "Finish" button. - Click "Settings" tab. - Under Power Management, Check "Wake the computer to run this task." - Click "Apply" button, click "OK" button. - Enter Standby mode. - Wait for 3 minutes, system will automatically resume from standby mode. |  | V |  | PS#2 |
| 14. | Wake from Hibernate by Scheduled Tasks. Test Procedures: 1. Base on 10. and enter Hibernate mode. - Wait for 3 minutes, system will automatically resume from hibernation mode. |  | V |  |  |
| 15. | Wake on Ring (Use Hyper terminal to test. After system was waked, the screen should keep black until you touch mouse or keyboard. And system still can work on call out without disconnect and re-create a new connection.) |  | V |  |  |
| 16. | Wake on LAN (After system was waked, the screen should keep black until you touch mouse or keyboard.) |  | V |  |  |
| 17. | Wake on USB device, force system enter Standby with USB mouse then click mouse left button to wake. ( Procedure as below: ) 1. download usb.bat and wakedevs.exe in the same folder from \\tpe_sa3\Test Tools\Others\USB device wake up utility 2. plug-in the USB device first. 3. run usb.bat directly in Windows mode. 4. Step by step. |  | V |  |  |
| 18. | Force system enter Standby, plug in USB device, check system will not auto resume. |  | V |  |  |
| 19. | Check all LEDs indicators are working fine and correct. |  | V |  |  |
| 20. | Check all LEDs are working properly after resume from Standby, Hibernate |  | V |  |  |
| 21. | Check Fan is turned off in Standby/Hibernation mode. |  | V |  |  |
| 22. | Force the system entering standby by close panel (Lid switch set to Standby) during shutdown the system. |  | V |  |  |
| 23. | Force the system entering Hibernate by close panel (Lid switch set to Hibernate) during shutdown the system. |  | V |  |  |
| 24. | Make sure the low/critical battery alarm tone was generated and keep auto update remaining capacity of the battery. |  | V |  |  |
| 25. | Go to Start -> All Programs -> Accessories -> System Tools -> System Information -> Components -> Problem Devices, check no any error. |  | V |  |  |
| 26. | Go to Control panel -> Administrative Tools -> Event Viewer -> System, check no any unexpected error. (After testing whole BIOS release checking list, please review and investigate all unexpected error records. ) |  | V |  |  |
| 27. | Check battery life with [Business Winstone 2004 v1.01 BatteryMark ]can to pass master project spec. ( depend on project ) |  | V |  | 2:13 (4 Cells) |
| No. | ### PCMCIA Function Check (Please follow procedures as below) |  | Pass | Fail | Remark |
| 1. Make sure the system can auto-detect the device while hot insert the PCMCIA card. 2. Make sure the PC card driver have setup complete without any error or hang up. 3. Make sure no exclamation mark in device manager when PC card was inserted. 4. Check the PCMCIA card function is working properly. 5. Make sure the both upper socket and lower socket are working properly. 6. Make sure no exclamation mark in device manager after resume from Standby/Hibernate with PC card inserted. 7. Make sure the both upper socket and lower socket are working properly after resume from Standby/Hibernate. 8. Make sure the device does not list in device management after plug out the PC card. 9. Plug in two difference type of PC card into upper socket and lower socket, and then check item 1 to item 8. |  |  |  |  |  |
| 1. | #### Check the IBM 1GB Microdrive CF+ Type II (DSCM-10512) function |  | V |  |  |
| 2. | #### Check the Xircom CBE2-100 Card Bus Ethernet card function |  | V |  |  |

Comments: #1.WinFlash can plash same BIOS version. PS#1. After Winflash, system will shutdown. PS#2. After system was waked, the screen will keep black until touch mouse or keyboard.

Conclusion:

Base on above test result, this version BIOS V2.10 is better than old version. We agree to release BIOS V2.10.

Department:  Software Application Dept. III

Engineer: Jacky Wang