## ERA3100/ VDA9900 BIOS Release Notice Log

# V1.03

1. Recified keyboard matrix and seperated it to two for different keyboard matrix.

2. Fixed system to hang after user presses the power button to resume from S2D. This occurs on the condition before enters to S2D if the DMA function is enabled on the 2nd PCI IDE interface and user restarts the system and then re-enter to Win98.

3. Fixed hardware cursor disappeared after resume from S2R under NT4.0.

# V1.02

1. Fixed Toshiba DVD SC-2202 4x problem with resume from S2R by skipping UDMA33 function setting on autotyping during POST.

2. Support Japanese language keyboard.

3. Support LBA function against the hard drive capacity beyond 8.45 MB.

4. Fixed external keypad issue.

# V1.01

1. For ESS CODEC RESET on M/B R03, do not support audio power management under NT4.0/DOS in order to fix the system hangs during S2R/S2D process on M/B R03.

2. Support final keyboard matrix.

3. Fix mouse cursor disappeared under NT4.0 after FnF1 hardware icon function is executed.

4. Fix LB2 beep volume is more louder than normal volume during S2D process.

# V1.00

1. Disable expanded I/O pin function on M38869 V1.42. (F#209)

2. Prevent video hardware gauge icon remains on the screen while this it is activated and user express hotkey Fn+F3.(S#195)

3. Prevent hardware cuesor disappear from Fn+F1 function key.(S#221)

# B04

1. Fixed no function on the internal numeric  pad if the external keyboard is attached even though the Numlock is set to ON in CMOS Setup. (#186)

2. Support Intel (R) Mobile Celeron (TM) CPU.(#171)

Note: Compared to B03, the CPU, Cache, and Miser module have been changed.

3. Removed GPO0 AC CODEC reset function and software CODEC reset function on BIOS. (#94)

Note: This may affects PC beep function on cold boot and/or warm reset.

4. Fixed garbled VCD image and data remains on the screen during the S2D process with DVD drive under NT4.0 (#161)

5. Disabled Video hardware cursor during the S2D process under NT4.0. (#139)

6. Set ESS GPIO[8:6] pins to I2S control pins which is connected to ZV port. (#172)

7. Fixed VCD playing issues on DVD/VCD drives after resume from S2R/S2D. (#43,#46,#49,#125)

# B03

1. Remove setup option "LCD/CRT" to fix #187.

2. Restore Pad-lock flag after resuming from S2R to fix #179

3. Enable 3D wheel mouse support for keyboard controller M38867 ver1.42 to fix #77

4. Show remaining 0% battery capacity when system without battery installed to make ERA2000's customer happy.

Note:

1. The setup option "LCD/CRT" may be restoed without notice when VGA BIOS support system BIOS setting.

2. The change of remaining 0% battery capacity for ERA series will not put into specification nor put into MRA series.

# B02

1. Change VGA BIOS to fix #138, #10, #4 and the other screen garbage relative problems.

2. Update internal keyboard matrix to fix #157, #76

3. Modify BIOS setup routine to show hard size large than 8.4G. to fix #165.

4. Keep LCD brightness value when hotkey pressed to fix #168

Note:

1. #165 means that BIOS setup report size error only