MAIN BOARD

USER'S   MANUAL                                                          (MODEL NO : SV-113)

1-1 Layout reference

2-2 Jumper Settings

| INTEL® CPU | CLOCK SPEED | RATIO | JP2 | JP3 | JP4 | JP5 | JP7 | JP8 | JP9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P54C 75 3.3V | 50MHZ | 1.5 | ON | ON | ON | OFF | ON | OFF | 1-2,5-6,7-8 |
| P54C 90 3.3V | 60MHZ | 1.5 | OFF | OFF | ON | OFF | ON | OFF | 1-2,5-6,7-8 |
| P54C 100 3.3V | 66MHZ | 1.5 | OFF | OFF | OFF | OFF | ON | OFF | 1-2,5-6,7-8 |
| P54C 120 3.3V | 60MHZ | 2 | OFF | OFF | ON | 1-2 | ON | OFF | 1-2,5-6,7-8 |
| P54C 133 3.3V | 66MHZ | 2 | OFF | OFF | OFF | 1-2 | ON | OFF | 1-2,5-6,7-8 |
| P54C 150 3.3V | 60MHZ | 2.5 | OFF | OFF | ON | 1-2, 3-4 | ON | OFF | 1-2,5-6.7-8 |
| P54C 166 3.3V | 66MHZ | 2.5 | OFF | OFF | OFF | 1-2, 3-4 | ON | OFF | 1-2,5-6,7-8 |
| P55C 166 2.8/3.3V | 66MHZ | 2.5 | OFF | OFF | OFF | 1-2, 3-4 | OFF | 1-2,3-4 | 7-8 |
| P54C 180 3.3V | 60MHZ | 3 | OFF | OFF | ON | 3-4 | ON | OFF | 1-2,5-6,7-8 |
| P54C 200 3.3V | 66MHZ | 3 | OFF | OFF | OFF | 3-4 | ON | OFF | 1-2,5-6,7-8 |
| P55C 200 2.8/3.3V | 66MHZ | 3 | OFF | OFF | OFF | 3-4 | OFF | 1-2,3-4 | 7-8 |
| P55C 233 2.8/3.3V | 66MHZ | 3.5 | OFF | OFF | OFF | OFF | OFF | 1-2,3-4 | 7-8 |

| AMD® CPU for (PRxxxAB) | CLOCK SPEED | RATIO | jp2 | JP3 | JP4 | JP5 | JP7 | jp8 | jp9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PR75 3.52V | 50MHZ | 1.5 | on | on | on | OFF | ON | OFF | 1-2,3-4,5-6,7-8 |
| PR90/PR120 3.52V | 60MHZ | 1.5 | off | OFF | ON | OFF | ON | OFF | 1-2,3-4,5-6,7-8 |
| PR100/PR133 3.52V | 66MHZ | 1.5 | OFF | OFF | OFF | OFF | ON | OFF | 1-2,3-4,5-6,7-8 |
| PR166 3.52V | 66MHZ | 2.5 | OFF | OFF | OFF | 1-2, 3-4 | ON | OFF | 1-2,3-4,5-6,7-8 |
| K6 PR166 2.9/3.3V | 66MHZ | 2.5 | OFF | OFF | OFF | 1-2, 3-4 | OFF | 1-2,3-4 | 1-2, 7-8 |
| K6 PR200 2.9/3.3V | 66MHZ | 3 | OFF | OFF | OFF | 3-4 | OFF | 1-2,3-4 | 1-2, 7-8 |
| K6 PR233 3.2/3.3V | 66MHZ | 3.5 | OFF | OFF | OFF | OFF | OFF | 1-2, 3-4 | 5-6, 7-8 |

| CYRIX® CPU | CLOCK SPEED | RATIO | jp2 | JP3 | JP4 | JP5 | JP7 | jp8 | jp9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P120+ 3.52V | 50MHZ | 2 | ON | ON | ON | 1-2 | ON | OFF | 1-2,3-4,5-6,7-8 |
| P150+ 3.52V | 60MHZ | 2 | OFF | OFF | ON | 1-2 | ON | OFF | 1-2,3-4,5-6,7-8 |
| P166+ 3.52V | 66MHZ | 2 | OFF | OFF | OFF | 1-2 | ON | OFF | 1-2,3-4,5-6,7-8 |
| P150+ 2.8/3.3V 6X86L | 60MHZ | 2 | OFF | OFF | ON | 1-2 | OFF | 1-2, 3-4 | 7-8 |
| P166+ 2.8/3.3V 6X86L | 66MHZ | 2 | OFF | OFF | OFF | 1-2 | OFF | 1-2, 3-4 | 7-8 |
| MX PR166 2.9/3.3V | 60MHZ | 2.5 | OFF | OFF | ON | 1-2, 3-4 | OFF | 1-2, 3-4 | 1-2,7-8 |
| mx Pr166 2.9/3.3V | 66MHZ | 2 | OFF | OFF | OFF | 1-2 | OFF | 1-2, 3-4 | 1-2,7-8 |
| MX PR200 2.9/3.3V | 66MHZ | 2.5 | OFF | OFF | OFF | 1-2, 3-4 | OFF | 1-2, 3-4 | 1-2,7-8 |

EXPLANATION OF JUMPER SETTING

JP1 : DIMM VOLTAGE SELECTOR

|  | 3.3V DIMM | 5V DIMM |
| --- | --- | --- |
| JP1 | 3-5, 4-6 | 1-3, 2-4 |

JP2-JP4 CPU CLOCK SELECTOR

|  | 50 MHZ | 60 MHZ | 66.6 MHZ |
| --- | --- | --- | --- |
| JP2 | ON | OFF | OFF |
| JP3 | ON | OFF | OFF |
| JP4 | ON | ON | OFF |

JP7,JP8 SINGLE /DUAL VOLTAGE CPU SELECTOR

|  | SINGLE VOLTAGE CPU | DUAL VOLTAGE CPU |
| --- | --- | --- |
| JP7 | ON | OFF |
| JP8 | OFF | 1-2, 3-4 |

CAUTION !

CPU WILL BE SERIOUSLY DAMAGED IF JP6 & JP8(1-2, 3-4)

ARE SET "ON" SYNCHRONOUSLY.

Jp5 : CPU frequency RATIO SELECTOR

|  | 1.5 X | 2.0 X | 2.5 X | 3.0 X | 3.5 X |
| --- | --- | --- | --- | --- | --- |
| JP5 | OFF | 1-2 | 1-2, 3-4 | 3-4 | OFF |

NOTE : JP5 (PIN5, PIN6) RESERVED.

JP9 : VCORE VOLTAGE SELECTOR

| jp9 | voltage output |  | jp9 | voltage output |
| --- | --- | --- | --- | --- |
| 1-2, 3-4, 5-6, 7-8 | 3.50v |  | 1-2, 7-8 | 2.90v |
| 1-2, 5-6, 7-8 | 3.30v |  | 7-8 | 2.80v |
| 5-6, 7-8 | 3.20v |  | 1-2, 5-6 | 2.50V |
| 1-2, 3-4, 7-8 | 3.10v |  | 1-2 | 2.10V |
| 3-4, 7-8 | 3.00V |  | OFF | 2.00V |

JP10 : BATTERY SELECTOR

|  | NORMAL | CLEAR CMOS |
| --- | --- | --- |
| JP10 | 1-2 (DefaulT) | 2-3 |

JP12 : FLASH ROM VOLTAGE SELECTOR

|  | 5V FLASH ROM | 12V FLASH ROM |
| --- | --- | --- |
| JP12 | 1-2 | 2-3 |

JP17 : WIN95 SOFTWARE POWER DOWN SELECTOR

|  | ENABLED | DISABLED |
| --- | --- | --- |
| JP17 | ON | OFF |

jp11 : case connector

JP18 : AT / ATX  POWER  SELECTOR

|  | AT POWER | ATX POWER |
| --- | --- | --- |
| JP18 | OFF | ON |

CN1,

CN2 :

PRIMARY / SECONDARY  IDE  CONNECTOR

PCI IDE controller connector supports up to four hard disk drivers ,CD-ROM , 120MB floppy (reserved for future BIOS ) and other devices to CN1 and CN2.

CN3 : FLOPPY DISK CONNECTOR

Standard floppy disk connector supports 360K , 720K ,1.2M , 1.44M , and 2.88M floppy disk type.

CN4 : PARALLEL PORT CONNECTOR

PARALLEL port connector supports standard printer port,enhanced parallel port (EPP), extended capabilities parallel port (ECP).

CN5,CN6 : SERIAL PORT CONNECTOR.

SERIAL port connector supports 16550a high speed communication port that send/receive 16 bytes fifos.

CN7 : USB (UNIVERSAL SERIAL BUS) CONNECTOR .

USB connector supports USB device.

| USB PIN OUT |  |
| --- | --- |
| USB1 | USB2 |
| PIN1 +5V | PIN6 +5V |
| PIN2 USBP0- | PIN7 USBP1- |
| PIN3 USBP0+ | PIN8 USBP1+ |
| PIN4 GND | PIN9 GND |
| PIN5 GND | PIN10 GND |

NOTE : USB (UNIVERSAL SERIAL BUS) AND IRCON CONNECTOR

ARE OPTIONAL.

CN8 : PS/2 MOUSE CONNECTOR.

mouse connector supports 5-pin PS/2 mouse .

PS/2 MOUSE PIN OUT

| PIN # 1 | mouse clock |
| --- | --- |
| PIN # 2 | mouse data |
| PIN # 3 | none |
| PIN # 4 | gnd |
| PIN # 5 | vcc |

CN9 :  IRCON (INFRARED) CONNECTOR

IR connector supports 5-pin IR device.

| IR CONNECTOR PIN OUT |  |  |  |  |
| --- | --- | --- | --- | --- |
| PIN 1 VCC | PIN 2 NONE | PIN 3 RX | PIN 4 GND | PIN 5 TX |

note :   IR uses same I/O port as COM2.  THERE is no any hardware

jumper setting for IR/COM2 on this main board but customer

needs to set proper BIOS setting for  "irda " , "ASKIR"  or

"DISABLED"(DEFAULT)  UNDER  "INFRA  RED  (IR)  function"  of

"INTEGRATED Peripheral" according to the following table:

|  | irDA 1.0 | ASKIR | DISABLED |
| --- | --- | --- | --- |
| (under INTEGRATED PERIPHERALS)"INFRARED (IR) FUNCTION " | USE IRDA 1.0 ON IRCON. | USE AMPLITUDE SHIFT KEYED IR ON IRCON . | USE com2 (DEFAULT) |